## Chapter- 4

PLAN IMPLEMENTATION

### Implementation Strategy

Traditionally, owing to complexities, government do not attempt to include people’s participation approach in the formulation and planning process of any project related to land development, rather attempts to implement it with public fund through land acquisition. Such policy has some built-in insurmountable problems that face strong resistance from people and result in the delay in its implementation.

People rightly feel that they have the right to know about the plan which has direct bearing on their living and property. The land development projects which are being implemented in the country regardless of their quality uproot many people from their home and community they belong. Thus a strong and desperate group of affected people together with their friends and allies try to resist the implementation process with their united strength.

In our country, people record land at considerably low price than actual to save money to be paid in the form of taxes related to exchange of ownership. This reality makes people financially looser since in case of land acquisition the price mentioned in the deed-document is considered. Compensation all over the world is a complicated process. Brokers prop up to bridge the gap between innocent people and the government machinery. Brokers eat up a lion share in the process and thereby yet another sufferer group emerges to resist the plan implementation process.

Plan Implementation through Active Community Participation Technique is the answer to offset these obstacles on the way to plan implementation.

### Landuse Management

Land Management is the process to make best use of the resources of land to achieve sustainable production of food grain and other agricultural products in the future. It covers all activities concerned with the management of land as a resource both from environmental and economic perspective. It includes farming, mineral extraction, property and estate management and physical planning of towns and countryside. It embraces such matters as:

    - Property conveyance including decisions on mortgages and investment

    - Property assessment and valuation

    - Development and management of utilities and services

    - Management of land resources such as forestry, soils, or agriculture

    - Formulation and implementation of landuse policies

    - Environmental impact assessment

    - Monitoring of all activities on land that effect the best use of that land.

(UN Economic Commission for Europe, Land Administration Guidelines, final draft-June14, 1995)

However, Consultants concentrate on land development management aspect only, as others are not directly related to Detailed Area Plan.

##### 4.2.1	Land Development Techniques

Experts worldwide optimize the benefit from land through adoption of appropriate Land Development Techniques and thereby create humane living environment. Generally following tools and techniques are used in land development schemes.

- Urban Renewal

- Urban Upgrading

- Land Readjustment

- Guided Land Development

- Land Swap

- Taxation

- Land Pooling

- Land Banking

Choice of the technique will depend on the characteristics of the site and the hope and aspirations of the community under Participatory Action Area Plan. Application of these techniques as implementation strategy for Participatory Action Plan at some smaller cohesive parts of Group-B has been proposed.

It may be mentioned that one well known technique, namely, Site and Services, has not been included here since RAJUK is already familiar with it and has implemented several such projects. Private land developers in Dhaka too are implementing their land development projects through this technique.

### Areas for Action Plan

There are several patches of land in the DND area where planned development can be achieved through use of Land Readjustment Technique. These areas are designated as areas for Action Area Plan to be prepared by RAJUK on completion of DAP. Jalkundi, Matuail, Godenail, Kutubpur, Khanpur mauzas within DND are recommended to be developed through Action Plan (Map No. 4.1). It may be mentioned here that ultimately whole DMDP area has to be developed through these techniques. As these techniques have not yet been tried, a Pilot Scheme is recommended to be carried out first to pave the way for subsequent large scale adoption.

### Public Sector Action Program

A list of Public Sector Action Program has been furnished in Chapter-II. The proposals for construction and refurbishment of roads aim to improve the connectivity of various places of the area. On implementation, better road link will be established and facilitate transportation of man and materials easily in a cost effective manner. Projects of Siddhirganj and Haripur Power Station will augment the generation and distribution network of electricity. Narayanganj City Water Supply Rehabilitation Scheme will improve the present water supply network and new areas will be covered under piped water supply system. Modernization of Institute of Marine Technology will enhance the capacity of the institute to produce larger number of personnel to handle the operation of growing number of marine vessels within and outside the country. Adamjee EPZ is already functioning and quite a large number of employments have been generated through it.

### Area Development Priorities and Phasing

##### Phasing

DMDP Structure Plan Phasing of the Plan is presented in Table 4.1.

##### Table - 4.1: Structure Plan Phasing

| Structure Plan Phase | Period |
| --- | --- |
| Short-term | 1995-2000 |
| Medium-term | 2000-2005 |
| Long-term | 2005-2015 |

Source: Dhaka Structure Plan 1995-2015 (DMDP)

The development proposals under Detailed Area Plan for Group-B should have been implemented by 2015. But there are proposals which may not be possible to be implemented within this period nor does it require so. But in order to secure right of way many proposals (especially for a number of new primary roads) have been included in the plan which are not meant for implementation within the plan period. Thus the consultants propose following three phases for implementation of various proposals made in the plan.

Phase-I	2010-2012

Phase-II	2012-2015

Phase-III	Beyond the plan period

##### Priorities

Priority list projects have been prepared on the basis of stakeholders’ consultation at different stages of project activities. Priority list is provided according to landuse zones as described below.

##### Urban Residential Zone Priority

The Group-B area will have to accommodate a projected additional population of about 1.3 million. The established areas of Narayanganj paurashava and Shyampur, Dania unions have experienced highly unplanned growth and the net population density in these areas is expected to reach as high as 300 persons per acre by 2015. Existing infrastructure in those areas cannot adequately serve such huge population. Urban amenities and service facilities are highly inadequate too. Indiscriminate development of commercial and industrial facilities has prop up especially within DND area. These incompatible uses are threatening to residential environment and have been identified as nonconforming to the zone. In the first phase such non-conforming uses must be identified and abiding conditions for their removal to suitable landuse zone imposed on a case to case basis.

Densification has to be encouraged in the urbanizing and new urban areas since density of the established unplanned urban areas are expected to reach too high a figure to house further additional people. DND triangle has been earmarked as a potential area for urban expansion by higher level plans also. Therefore DND triangle is one of the potential locations for development in the Group-B area. Other areas of urban expansion are Narayanganj West and Kadam Rasul paurashava.

Large scale land development through land acquisition is not possible for financial and social reasons. This can be adequately handled if people are involved in the process. Consultants recommend that RAJUK should immediately take up a Pilot Project in DND area as first priority. Experience from this Pilot Project can be utilized for finalizing priorities and phasing of the remaining developments. Till then new areas of DND will not come under development. Control will be imposed through regulatory and fiscal measures. Table no. 4.2 shows the residential development priority areas.

##### Table - 4.2: Planned Residential Development

| Sl. No. | Description | Priority |  |  | Phase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 1 | 2 | 3 |
| 1 | Planned Residential Area Development in Jalkundi, Bhuigar, Godenail mauzas of the DND Area through People’s Participation. |  |  |  |  |  |  |
| 2 | Residential Development in the Kashipur union of Narayanganj West: facilitate through improvement of utility provisions. |  |  |  |  |  |  |
| 3 | Residential Development in Kadam Rasul paurashava through improved drainage and utility service delivery. |  |  |  |  |  |  |

Industrial Zone Priority

Group-B area is an old industrial area of the country. Due to the decline of jute sector, industrial activities suffered greatly. Sign of recovery is visible with the growth of export oriented Garment Industries. Garment Industries have been set up haphazardly throughout the area in a chaotic manner. On the other hand dyeing industries are polluting water bodies by discharging untreated effluent. In this situation consultants recommend that these industries should be relocated in the industrial zones and therefore designates several areas as industrial zones. There are a number of textile dyeing industries in the DND area. Consultants recommend that all these dyeing industries and the re-rolling mills should be re- located in the Heavy/Noxious Industrial Zones as first priority. These facilities have been designated as non-conforming and until they are shifted, any expansion, alteration or improvement of these industries shall be prohibited.

Development of the industrial area should be systematic and efficient. To ensure this consultants recommend that Industrial development be undertaken in the form of Industrial Parks or EPZs complemented by adequate service facilities such as bank branches, container yard, truck terminal, fire station etc. instead of allowing isolated facilities. Consultants recommend that in the first phase one of such EPZs or Industrial Parks be undertaken as Pilot Project to gather deeper understanding so as to apply that knowledge for improving the subsequent projects.

##### Table - 4.3: Industrial Land Development

| Sl No. | Area | Priority | Phase |
| --- | --- | --- | --- |
| 1 | Development of additional space for General Industrial Area along BSCIC industrial zone at Panchabati. | I | I |
| 2 | Development of the Heavy Industrial Zone designated area with proper infrastructure | I | I |
| 3 | Relocation of Dyeing and Re-rolling industries from DND to Heavy Industrial Zones | I | I, II & III |

Road Network Priority

A number of new North-South and East-West primary roads have been proposed by the consultants. In the Table No. 4.4 they have listed according to priority:

##### Table - 4.4: Road Network Priority

| Sl. No. | Road Name | Priority |  |  | Phase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 1 | 2 | 3 |
| 1 | Adamjee Nagar-Sayedpur River Front Road | √ |  |  | √ | √ | √ |
| 2 | Adamjee Road | √ |  |  | √ |  |  |
| 3 | Aliganj-Godenail Road | √ |  |  | √ | √ | √ |
| 4 | Aliganj-Jalkundi Road | √ |  |  | √ | √ |  |
| 5 | Amirabad -Nabiganj Road |  | √ |  |  | √ |  |
| 6 | Amoir-Gharmora Road |  |  | √ |  |  | √ |
| 7 | Bandar Bazar-Koratpara Road |  | √ |  |  | √ |  |
| 8 | Bara Vanga-Demra Road |  |  | √ |  |  | √ |
| 9 | Bhuigar-Ati Road | √ |  |  | √ | √ |  |
| 10 | Bhuigar-Kutubail Road |  |  | √ |  |  | √ |
| 12 | Bilashnagar-Talla Road | √ |  |  | √ | √ | √ |
| 13 | Issa Khan-Riverfront Road |  | √ |  |  | √ |  |
| 14 | Chashara Railway Stataion-Gate-II Road | √ |  |  | √ |  |  |
| 15 | Dakshin Matuail-Demra Road |  |  | √ |  |  | √ |
| 16 | Dapa Idrakpur-Godenail Road |  | √ |  |  | √ | √ |
| 17 | Dapa Idrakpur-Siachar Road | √ |  |  | √ | √ | √ |
| 18 | Demra-Sharif Bag Road | √ |  |  | √ |  |  |

| Sl. No. | Road Name | Priority |  |  | Phase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 1 | 2 | 3 |
| 19 | Demra-Siddhirganj Road |  |  | √ |  |  | √ |
| 20 | Demra-Simrail Road |  |  | √ |  |  | √ |
| 21 | Deuli-Barapara Road |  |  | √ |  |  | √ |
| 22 | Deulpara-Adamjee Road | √ |  |  | √ | √ |  |
| 23 | Deulpara-North Siachar Road |  | √ |  |  | √ | √ |
| 24 | Deulpara Road |  | √ |  |  | √ |  |
| 25 | Dhaka-Chittagong Road | √ |  |  | √ | √ | √ |
| 26 | DND Spine (Narayanganj Link Road) | √ |  |  | √ | √ | √ |
| 27 | Dhakeshwari road |  | √ |  |  | √ |  |
| 28 | Dhamgar-Madanganj Road |  | √ |  |  | √ | √ |
| 29 | Dharmaganj-Nabinagar Road 1 |  |  | √ |  |  | √ |
| 30 | Dharmaganj-Nabinagar Road 2 |  |  | √ |  |  | √ |
| 31 | Dudhghata Road |  |  | √ |  |  | √ |
| 32 | East Bamail-East Sanarpar Road |  |  | √ |  |  | √ |
| 33 | East Kadamtali-Shahidnagar Road |  |  | √ |  |  | √ |
| 34 | East Lamapara Road |  |  | √ |  |  | √ |
| 35 | Ekrampur-Chota Baraikhali Road |  | √ |  |  | √ |  |
| 36 | Fatulla-Godenail Link Road | √ |  |  | √ | √ |  |
| 37 | Fatulla-Lamapara Road | √ |  |  | √ | √ |  |
| 38 | Fatulla Bazar-Dharmaganj -Shashangao Road | √ |  |  | √ | √ | √ |
| 39 | General Industry Road 1 | √ |  |  | √ | √ |  |
| 40 | Gognagar-Sayedpur Road |  |  | √ |  |  | √ |
| 41 | Gobindapur-Dakshin Matuail Road |  |  | √ |  |  | √ |
| 42 | Godenail- Road |  |  | √ |  |  | √ |
| 43 | Godenail Road |  |  | √ |  |  | √ |
| 44 | -Chashara Railway Road | √ |  |  | √ |  |  |
| 45 | Heavy Industry Road 1 | √ |  |  | √ |  |  |
| 46 | Heavy Industry Road 2 | √ |  |  | √ |  |  |
| 47 | Heavy Industry Road 3 | √ |  |  | √ |  |  |
| 48 | Ispahani Bazar Road |  |  | √ |  |  | √ |
| 49 | Jalkundi -Godenail Road | √ |  |  | √ | √ |  |
| 50 | Jalkundi -Kutubpur Road |  |  | √ |  |  | √ |
| 51 | Jalkundi -Lamapara Road |  |  | √ |  |  | √ |
| 52 | Jalkundi -Nayapara Road | √ |  |  | √ | √ | √ |
| 53 | Jalkundi Road |  |  | √ |  |  | √ |
| 54 | Jamtala -Paikpara Road |  |  | √ |  |  | √ |
| 55 | Jatrabari-Demra Road |  |  | √ |  |  | √ |
| 56 | Jurain-Chandmari Railway Road |  |  | √ |  |  | √ |

| Sl. No. | Road Name | Priority |  |  | Phase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 1 | 2 | 3 |
| 57 | Jurain-South Matuail Road |  |  | √ |  |  | √ |
| 58 | Kanchpur-Chapatali Road | √ |  |  | √ | √ | √ |
| 59 | Kanchpur-Madanganj | √ |  |  | √ | √ | √ |
| 60 | Kajla-Rasulnagar Road | √ |  |  | √ | √ |  |
| 61 | Kashipur-Gognagar Road | √ |  |  | √ |  |  |
| 62 | Kashipur-Narayanganj Road | √ |  |  | √ | √ | √ |
| 63 | Kashipur-Netaiganj-Kashipur Link Road |  | √ |  |  | √ | √ |
| 64 | Kayempur Road | √ |  |  | √ |  |  |
| 65 | Khanpur- Road |  |  | √ |  |  | √ |
| 66 | Konapara-Rayer Bag Road | √ |  |  | √ |  |  |
| 68 | Konapara-Saddam Market Road |  |  | √ |  |  | √ |
| 69 | Konapara-South matuail Road |  |  | √ |  |  | √ |
| 70 | Kuripara-Gokuldaser bag Road |  |  | √ |  |  | √ |
| 71 | Kuripara-Malibag Road |  |  | √ |  |  | √ |
| 72 | Kutubpur-Enayetnagar Road |  |  | √ |  |  | √ |
| 73 | Lakshmankhola-Langalbanda Bazar Road |  |  | √ |  |  | √ |
| 74 | Laksmankhola Road 1 | √ |  |  |  | √ | √ |
| 75 | Laksmankhola Road 2 | √ |  |  |  | √ | √ |
| 76 | Laksmankhola Road 3 | √ |  |  |  | √ | √ |
| 77 | Lal Krishnadi-Gaziarpar Road |  |  | √ |  |  | √ |
| 78 | Lalpur-Panchabati Road |  |  | √ |  |  | √ |
| 79 | Madanganj-Kabilganj Road |  |  | √ |  |  | √ |
| 80 | Madanpur-Madanganj Road | √ |  |  | √ | √ | √ |
| 81 | Masdair-Kashipur Hatkhola Road | √ |  |  | √ | √ | √ |
| 82 | Masdair-Chandmari Road |  | √ |  |  | √ |  |
| 83 | Matuail-Baniar Bag Road |  |  | √ |  |  | √ |
| 84 | Matuail Road |  |  | √ |  |  | √ |
| 85 | Mijmiji-Dhankunda Road |  |  | √ |  |  | √ |
| 86 | Mijmiji-Jalkundi Road |  |  | √ |  |  | √ |
| 87 | Mijmiji Godenail Road | √ |  |  | √ | √ | √ |
| 88 | Mijmiji Road 1 |  |  | √ |  |  | √ |
| 89 | Mijmiji Road 2 |  |  | √ |  |  | √ |
| 90 | Mohammadbag-South Matuail Road |  |  | √ |  |  | √ |
| 91 | Mozahidnagar-East Kadamtali Road |  |  | √ |  |  | √ |
| 92 | Munshikhola-Siddhirganj Road | √ |  |  | √ | √ | √ |
| 93 | Nabiganj Road |  |  | √ |  |  | √ |
| 94 | Netaiganj-sayedpur Road | √ |  |  | √ | √ |  |
| 95 | Nandalapur-South Siachar Road |  |  | √ |  |  | √ |

| Sl. No. | Road Name | Priority |  |  | Phase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 1 | 2 | 3 |
| 96 | Nandan Kanon-Kalagachia Bazar-Farajikanda Road |  |  | √ |  |  | √ |
| 97 | Nischintapur-Kadamtali Road | √ |  |  | √ | √ |  |
| 98 | North Kashipur-West Deobhog Road |  | √ |  |  | √ |  |
| 99 | North Laksmankhola Road |  | √ |  |  | √ |  |
| 100 | Old Narayanganj Road | √ |  |  | √ |  |  |
| 101 | Omarpur-Godenail Road |  |  | √ |  |  | √ |
| 102 | Paikpara-Muslimnagar Road | √ |  |  | √ | √ |  |
| 103 | Panchabati-Chashara Road | √ |  |  | √ |  |  |
| 104 | Panchabati-Dharmaganj Road | √ |  |  | √ |  |  |
| 105 | Panchabati-Kashipur Road |  | √ |  |  | √ |  |
| 106 | Panchabati-Muktarpur Road | √ |  |  | √ | √ | √ |
| 107 | Power Station Road |  | √ |  |  | √ |  |
| 108 | Rajarbag-Godenail Road |  |  | √ |  |  | √ |
| 109 | Rasulbag-South Matuail Road |  |  | √ |  |  | √ |
| 110 | Roghunathpur Mijmiji Purpopara Road |  |  | √ |  |  | √ |
| 111 | Sanarpar-Jalkundi Road |  |  | √ |  |  | √ |
| 112 | Sarulia-Adamjee Road |  |  | √ |  |  | √ |
| 113 | Sarulia Road |  |  | √ |  |  | √ |
| 114 | Sastapur Road |  |  | √ |  |  | √ |
| 115 | Shahebpara Jalkundi Road |  | √ |  |  | √ | √ |
| 116 | Shahidnagar-Bhuigar Road | √ |  |  | √ | √ |  |
| 117 | Shantinagar Road |  |  | √ |  |  | √ |
| 118 | Sharifbag-Enayetnagar Road |  |  | √ |  |  | √ |
| 119 | Shashangao-Masdair |  |  | √ |  |  | √ |
| 120 | Shyampur Road | √ |  |  | √ |  |  |
| 121 | Siachar-Godenail Road | √ |  |  | √ | √ | √ |
| 122 | Siddhirganj-Godenail Road |  | √ |  |  | √ | √ |
| 123 | Sign Board-Bhuigar |  | √ |  |  | √ | √ |
| 124 | Sonakanda-Mirkandi Road |  | √ |  |  | √ | √ |
| 125 | South Dania-Siddhirganj |  |  | √ |  |  | √ |
| 126 | South Kutubkhali-East Jurain Road | √ |  |  | √ |  |  |
| 127 | South Matuail-Bhuigar Road |  |  | √ |  |  | √ |
| 128 | South Matuail-Dagar Road |  |  | √ |  |  | √ |
| 129 | South Matuail-Deulpara Road |  |  | √ |  |  | √ |
| 130 | South Matuail-Nandalalpur Road | √ |  |  | √ |  |  |
| 131 | South Sastapur Road |  |  | √ |  |  | √ |
| 132 | Stadium Road |  |  | √ |  |  | √ |
| 133 | DND Spine Road |  |  | √ |  |  | √ |

| Sl. No. | Road Name | Priority |  |  | Phase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 1 | 2 | 3 |
| 134 | Sukurshi Road |  |  | √ |  |  | √ |
| 135 | Sayedpur Road |  | √ |  |  | √ |  |
| 136 | Talla Road |  |  | √ |  |  | √ |
| 137 | Taltala–Bhuigar Road | √ |  |  | √ | √ |  |
| 138 | Tiger para-West Deobhog Road |  | √ |  |  | √ | √ |
| 139 | Tushardhara-Deulapara Road | √ |  |  | √ |  |  |
| 140 | West Bamail-West Sanarpar Road |  | √ |  |  | √ | √ |
| 141 | West Deobhog Road |  |  | √ |  |  | √ |
| 142 | West Masdair Road |  |  | √ |  |  | √ |
| 143 | Nabiganj-Kaikertek-Sonargaon Road |  |  | √ |  |  | √ |

Open Space, Recreation and Historical Preservation Priority

The Group-B area houses quite a good number of historical sites which can be developed as sites of tourist attraction. On the other hand very few open space and recreation facilities are available in the area. Following projects (Table No. 4.5) have been selected from the Stakeholders’ wish list to enhance the recreation facilities.

##### Table - 4.5: Open Space, Recreation Site and Historical Place Preservation Priority List

| Sl. No. | Description | Priority |  |  | Phase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 1 | 2 | 3 |
| 1 | Refurbishment of Fort and develop it as a Tourist Spot. |  |  |  |  |  |  |
| 2 | Refurbishment of Sonakanda Fort and develop it as a Tourist Spot. |  |  |  |  |  |  |
| 3 | International Standard Sport Complex at Chandmari. |  |  |  |  |  |  |
| 4 | Aquatic Sports Centre at Charargope |  |  |  |  |  |  |
| 5 | Theme Park at Jimkhana |  |  |  |  |  |  |
| 6 | Development of Jimkhana Football Stadium |  |  |  |  |  |  |
| 7 | Development of Khanpur Children’s Park. |  |  |  |  |  |  |

Drainage Priority

Water logging is a serious problem in the DND area. Some areas of Narayanganj Paurashava and Kadam Rasul Paurashava also seriously suffer from water logging during rainy season. Siddhirganj paurashava road network is without roadside drain. In order to improve drainage of the area, projects listed in the Table No 4.6 have been prioritized.

##### Table - 4.6: Priority List of Drainage

| Sl. No. | Description | Priority |  |  | Phase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 1 | 2 | 3 |
| 1 | Mitigation of Water Logging in DND area |  |  |  |  |  |  |
| 2 | Drainage Improvement of Narayanganj Paurashava |  |  |  |  |  |  |
| 3 | Drainage Improvement of Kadam Rasul Paurashava |  |  |  |  |  |  |
| 4 | Construction of Roadside drains in Siddhirganj Paurashava |  |  |  |  |  |  |

Priority List of Institutions

Narayanganj area seriously lacks in standard academic institutions. The issue has been discussed in the higher level plan. The people of Narayanganj feel that setting up of following institutions (Table No. 4.7) will meet their demand to some extent.

##### Table - 4.7: Priority List of Institutions

| Sl. No. | Description | Priority |  |  | Phase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 1 | 2 | 3 |
| 1 | Medical College in the Narayanganj Old D.C. office complex |  |  |  |  |  |  |
| 2 | Public University in DND Area |  |  |  |  |  |  |

Municipal Administration Coverage of DND and Narayanganj West Area

A very large urbanized and urbanizing area of DND Triangle and Narayanganj West remains outside the jurisdiction of municipal administration. As a result, delivery and maintenance of municipal services are absent in those area causing immense sufferings to the inhabitants. In this situation the Consultants recommend for creation of two new municipalities in the DND namely Matuail Paurashava and Kutubpur Paurashava respectively and extend the jurisdiction of Narayanganj municipality in the west and south up to Dhaleshwari Flood Plain and up to Sastapur Khal of DND in the north.

##### Table - 4.8: Extension of Municipal Administration Coverage

| Sl No. | Description | Priority | Phase |
| --- | --- | --- | --- |
| 1 | Area immediate west and north up to Sastapur canal of Narayanganj Paurashava should be included within jurisdiction of Narayanganj Municipality | I | I |
| 2 | Kutubpur Paurashava of DND | I | I |
| 3 | Matuail Paurashava of DND | I | I |

Boundaries of proposed Matuail Paurashava, Kutubpur Paurashava and extended area of Narayangang Paurashava are shown in Map No. 4.2.

### Development Control

Development control function consists of four distinct complementary streams of activities: Landuse Zoning, Development Permit, Field level Vigilance and Interaction with people. Following paragraphs embody the plan proposals under these activity heads.

##### Landuse Zoning

Landuse Zoning is a form of legal power which is delegated to development authorities/municipalities through enabling legislation to ensure the welfare of the community by regulating the most appropriate use of the land. Zoning is a classification of landuses that limits what activities can or cannot take place on a land parcel by establishing a range of development options.

Zoning has been defined as an action through legislation provided to a development authority/municipality to control

      - The heights to which buildings may be erected

      - The area of lots that must be left un-built upon, and

      - The uses to which buildings and lots may be put.

##### Area Zoning

Area zoning is to specify which types of landuse are considered appropriate for different areas or 'zones' and it therefore indicates the planning control objectives of the authority or municipality for its administrative area. This is therefore the primary objective of the DAP.

##### Density Zoning

The aim of the density zoning is to provide an acceptable population density which is related to the designed facilities and amenities especially in the residential areas. This ensures a healthy community and enjoyable community life. Presently within DMDP jurisdiction there is no effective rule to control population density of different areas according to their physical and socio-economic characteristics or trend. Floor Area Ratio (FAR) adopted in the Dhaka Mahanagar Imarat Nirman Bidhimala 2008 is the only tool to control population density. It too did not consider the above factors and in many instances proved to be impractical to serve the desired end. FAR should vary from place to place due to variation of the governing factors, specially, income level of the residents. Apart from FAR, there are several standards to achieve desired population density such as number of dwelling units per acre, maximum room occupancy etc. each with its comparative effectiveness. Determining the most appropriate unit and value of standard needs fair knowledge about the likely structures, number and size of land-parcels (not the mauza-plot number since one mauza plot may contain several smaller land-parcels) in addition to socio-economic condition of the residents. Determining density at this stage therefore cannot be done. Consultants recommend that FAR should be carefully determined during the preparation of Action Plans since the needful information to do so would be available and adopted in the Dhaka Mahanagar Imarat Nirman Bidhimala.

##### Height Zoning

This zoning provides height limits for structures and objects of natural growth and standards for use of an area which encourage and promote the proper and sound development of areas. It is also applicable to height restrictions for flight safety around airports or other similar purposes.

For effective development control, in addition to landuse zoning individual facility and the structures therein is to comply with certain regulations imposed to ensure desirable end. Relation between ground cover of buildings and the land parcel that house it, minimum setback of building from the adjoining plot boundaries and the maximum floor area that can be constructed in relation to the plot size and the connecting road, among many other details, are controlled by RAJUK’s Building Construction Rules. Besides, Bangladesh National Building Code focuses on the appropriate materials, construction method, building safety and associated issues.

##### Landuse Classification

Detailed Area Plan as its Area Zoning responsibility classified the Landuse of DMDP area into following zones:

  - Urban Residential Zone

  - Commercial Zone (Business)

  - Commercial Zone (Office)

  - General Industrial Zone

  - Heavy Industrial Zone

  - Mixed Use Zone (Commercial-General Industrial)

  - Mixed Use Zone (Residential-Commercial)

  - Mixed Use Zone (Residential-Commercial-General Industrial)

  - Mixed Use Zone (Residential-General Industrial)

  - Institutional Zone

  - Administrative Zone

  - Agricultural Zone

  - Flood Flow Zone

  - Open Space

  - Rural Settlement Zone

  - Water Retention Area

  - Water Body

The following sections deal with the general definition of the use and description of associated permitted and conditionally permitted uses under each Landuse zone one by one. The uses not listed here in any of the category shall be treated as Restricted Use for the corresponding landuse category and shall not be permitted only except unanimously decided otherwise by the Nagar Unnayan Committee. In such situations the use shall get permission in the category of New Use.

##### Urban Residential Zone

Urban Residential Zone is intended primarily for housing development but may also include a range of other uses particularly those that have the potential to foster the development of new residential communities such as schools, small shops, doctors’ chambers, open space like play fields and so on. Limited commercial facilities within this zone shall be planned and developed as an integral retail or business unit which, comply with the planned mixed-use concept of the Urban Area Plan.

##### Purpose

The prime objective of this zone is to provide the space for living as well as to meet the daily needs of the residents. The main purposes of the zone are:

- To provide for residential development, associated services and to protect and improve residential amenities.

- To improve the quality of existing residential areas and strengthen the provision of local community services and amenities.

- To control unscheduled development on individual lots or tracts, promote economical and efficient landuse.

- To provide for a variety of housing types and densities to meet current and future residential needs.

- To provide an appropriate mix of house sizes, types and tenures in order to meet household needs; to promote balanced communities; and to promote higher densities in the development centre to facilitate day and evening activity and ensure a ‘living’ centre.

The urban areas that are already densely and haphazardly built are termed here as Spontaneous Residential Zone. These zones primarily support living with some civic facilities. The main purpose of this landuse zone is to provide enough space for residence. There will have some supporting uses, too, for the residents.

The areas that have some scope to develop as a planned area are declared as Planned Residential Zone. In Planned Residential Zone, there will have a variety of supporting uses such as small retail, parking, post box etc with primarily residential use. Planned residential area in the neighbourhood form should be followed for new urban development.

The neighbourhood is a walkable community in human scale, with a frequently interconnected street network which weave a mix of housing types, and small local scale retail and service uses that serve the local population. The services designed here are in walking distances considering the day to day needs. Service standards for neighbourhood level have been mentioned in the previous chapter. For purposes of administering the DAP Permitted Landuses in the neighborhood, the design specification, standards and guidelines about the area and number must be maintained.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.9: Landuse Permitted in Urban Residential Zone

| All Types of Residential House | Grocery Store |
| --- | --- |
| Apartment Housing | High School |
| Artisan's Shop | Housing Projects |
| Assisted Living or Elderly Home | Household Appliance and Furniture Repair Service (No Outside Storage) |
| ATM Booth | Landscape and Horticultural Services |

| Barber Shop | Memorial Structure (Ancillary) |
| --- | --- |
| Bill Payment Booth | Monument (Neighborhood Scale) |
| Boarding and Rooming House | Mosque, Place of Worship |
| Book Stall | Newspaper Stand |
| CBO Office | Nursery School |
| Child Daycare/Pre-school | Orphanage |
| Children’s Park (Must Have Parking) | Photocopying and Duplicating Services (No Outside Storage) |
| Cleaning/Laundry Shop | Pipelines and Utility Lines |
| Communication Service Facilities | Playing Field |
| Communication Tower Within Permitted Height | Primary School |
| Community Center | Private Garages (Ancillary Use) |
| Condominium or Apartment | Project Identification Signs |
| Confectionary Shop | Property Management Signs |
| Cottage | Public Transport Facility |
| Cultural Exhibits and Libraries | Retail Shops/Facilities |
| Cyber Cafe | Satellite Dish Antenna |
| Daycare Center (Commercial or Nonprofit) | Shelter (Passers By) |
| Departmental Stores | Shoe Repair or Shoeshine Shop (Small) |
| Doctor/Dentist Chamber | Special Dwelling |
| Dormitory | Specialized School: Dance, Art, Music, Physically Challenged & Others |
| Drug Store or Pharmacy | Stationery Store |
| Dwelling | Temporary Pandle for Permitted Function |
| Eidgah | Temporary Tent |
| Employee Housing (Guards/Drivers)/Ancillary Use | Transmission Lines |
| Fast Food Establishment /Food Kiosk | Urban-Nature Reserve |
| Fitness Centre | Uses in Neighborhood Center* (Where Neighborhood Center exists) |
| Flowers, Nursery Stock and Florist Supplies | Water Pump/Reservoir |
| Gaming Clubs | Woodlot |
| General Store |  |
| *Permit of Neighborhood Center Facilities in absence of formal neighborhood should be subject to Landuse Permit Committee |  |

Landuse Conditionally Permitted

The following uses may be permitted or disallowed in the zone after review and approval by the authority/committee following appropriate procedure while the application meets the criteria mentioned in the requirement.

##### Table - 4.10: Landuse Conditionally Permitted in Urban Residential Zone

| Addiction Treatment Center | Graveyard/Cemetery |
| --- | --- |
| Amusement and Recreation (Indoors) | Guest House |
| Art Gallery, Art Studio/Workshop | Market (Katcha Bazaar) Place |

| Automobile Driving Academy | Neighborhood Co-Operative Office |
| --- | --- |
| Beauty and Body Service | Optical Goods Sales |
| Billiard Parlor/Pool Hall | Outdoor Cafe |
| Building Maintenance/Cleaning Services, No Outside Storage | Overhead Water Storage Tanks |
| Bus Passenger Shelter | Paints and Varnishes Store |
| Coffee Shop/Tea Stall | Parking Lot |
| Community Hall | Patio Homes |
| Correctional Institution | Photofinishing Laboratory |
| Courier Service | Plantation (Except Narcotic Plant) |
| Crematorium | Police Station |
| Electrical Sub Station | Post Office |
| Emergency Shelter | Postal Facilities |
| Energy Installation | Row House |
| Fire Brigade Station | Slaughter House |
| Flood Management Structure | Sports and Recreation Club |
| Fruit Markets | Static Transformer Stations |
| Funeral Services | Telephone Sub Station |
| Furniture & Variety Stores | Temporary Rescue Shed |
| Garages | Tennis Club |
| Garden Center or Retail Nursery | Tourist Home or Resort |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Commercial Zone (Business)

Commercial areas generally contain one or more of a wide variety of uses that includes business uses, offices, hotel, public houses, and limited forms of retailing. Uses that are particularly appropriate to these areas include wholesale, business service uses, the sale of motor vehicles and limited forms of retailing that involve the sale of bulky goods unsuitable to be carried away by pedestrians. This would allow uses that are compatible with, or reinforce, the commercial function of the area. As a whole, Commercial Zone is intended to provide all the facilities & services with the commercial landuse that are designed for basically trade and business.

Commercial zones are classified into two types: office and business predominated zones. Commercial Business Zone is a business predominated commercial zone that mainly supports business.This zone is intended to support large commercial developments as retail distribution, retail warehouse, retail storage and retail logistics facilities.

The Commercial Business Zone is intended to provide areas for logistical and retail warehousing activities, including storage, distribution and packaging of goods and products as well as activities which require a large site and space.

##### Purpose

- The intent of this zone is to provide commercial nodes in convenient and strategic locations of the area to meet urban needs and to encourage clustering commercial development as opposed to strip commercial development and commercial sprawl.

- The main objective for designating this zone is to promote trade and commerce compactly associated with all other facilities.

- Commercial business zone is mainly intended for supporting the business and associated works. There are several functions that are permitted in this zone.

##### Landuse Permitted

Commercial Zone (Business) is mainly intended for supporting the Business related activities. There are several functions that are permitted in the zone as mentioned in the following Table 4.11.

##### Table - 4.11: Landuse Permitted in Commercial Zone (Business)

| Accounting, Auditing or Bookkeeping Services | Grocery Store |
| --- | --- |
| Agri-Business | Guest House |
| Agricultural Sales and Services | Hotel or Motel |
| Ambulance Service | Inter-City Bus Terminal |
| Antique Store | Jewelry and Silverware Sales |
| Appliance Store | Market (Bazar) Place |
| ATM Booth | Mosque, Place of Worship |
| Auction Market | Motorcycle Sales Outlet |
| Auditorium, Meeting Halls, and Conference Facilities, Convention Hall | Multi-Storey Car Park |
| Auto Leasing or Rental Office | Newspaper Stand |
| Auto Paint Shop | Outdoor Recreation, Commercial Outdoor Recreation |
| Auto Parts and Accessory Sales (Indoors) | Parking Lot (Commercial) |
| Auto Repair Shop (With Garage) | Pet Store |
| Automobile Sales | Photocopying and Duplicating Services |
| Automobile Wash | Photofinishing Laboratory & Studio |
| Bank & Financial Institution | Pipelines and Utility Lines |
| Barber Shop | Post Office |
| Beauty and Body Service | Preserved Fruits and Vegetables Facility/Cold Storage |
| Bicycle Shop | Printing, Publishing and Distributing House |
| Billboards, Advertisements & Advertising Structure | Professional Office |
| Billiard Parlor/Pool Hall | Project Identification Signs |
| Book or Stationery Store | Property Management Signs |
| Building Material Sales or Storage (Indoors) | Public Transport Facility |
| Bulk Mail and Packaging | Refrigerator or Large Appliance Repair |
| Bus Passenger Shelter | Resort |
| Cinema Hall | Restaurant |
| Commercial Office | Retail Shops/Facilities |
| Communication Service Facilities | Satellite Dish Antenna |
| Communication Tower Within Permitted Height | Shelter (Passers By) |
| Computer Maintenance and Repair Shop | Shopping Mall/ Plaza |
| Computer Sales & Service Shops | Slaughter House |
| Confectionery Shop | Social Forestry |
| Conference Center | Software Development Firm |

| Construction Company Offices | Sporting Goods and Toys Sales Centers |
| --- | --- |
| Courier Service | Super Store |
| Cyber Cafe | Taxi Stand |
| Day-care Center (Commercial or Nonprofit) | Telephone Exchanges |
| Department Stores, Furniture & Variety Stores | Television, Radio or Electronics Repair (No Outside Storage) |
| Doctor/Dentist Chamber | Theater (Indoor) |
| Drug Store or Pharmacy | Transmission Lines |
| Electrical and Electronic Equipment & Instrument Sales | Utility Lines |
| Fast Food Establishment/Food Kiosk | Vehicle Sales & Service, Leasing or Rental |
| Freight Handling, Storage & Distribution | Veterinarian Clinics, Animal Hospitals |
| Freight Transport Facility | Warehousing |
| Freight Yard | Water Pump/Reservoir |
| Fruit and Vegetable Markets | Wood Products |
| General Store | Woodlot |

Landuse Conditionally Permitted

Functions permitted with some condition in the zone are:

##### Table - 4.12: Landuse Conditionally Permitted in Commercial Zone (Business)

| Amusement and Recreation (Indoors) | Fire/Rescue Station |
| --- | --- |
| Bicycle Assembly, Parts and Accessories | Grain & Feed Mills |
| Broadcast Studio/Recording Studio (No Audience) | Household Appliance and Furniture Repair Service |
| Coffee Shop/Tea Stall | Incineration Facility |
| Concert Hall, Stage Shows | Indoor Amusement Centers, Game Arcades |
| Construction, Survey, Soil Testing Firms | Indoor Theatre |
| Container Yard | Junk/Salvage Yard |
| Trade Shows | Lithographic or Print Shop |
| Craft Workshop | Motor Vehicle Fuelling Station/Gas Station |
| Plantation (Except Narcotic Plant) | Musical Instrument Sales or Repair Shop |
| Energy Installation | Optical Goods Sales |
| Re-fuelling Station | Painting and Wallpaper Sales |
| Firm Equipment Sales & Service | Paints and Varnishes Store |
| Agricultural Chemicals, Pesticides or Fertilizers Shop | Parking Lot |
| Fitness Centre | Patio Homes |
| Flowers, Nursery Stock and Florist Supplies | Private Garages |
| Forest Products Sales | Retail Shops Ancillary To Studio/Workshop |
| Fuel Dealers | Stone/Cut Stone Products Sales |
| Garages | Salvage Processing Activities |
| Garden Center or Retail Nursery | Truck/Covered Van Stand |
| Police Box/Barrack |  |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Commercial Zone (Office)

The Commercial Zone (Office) is intended to provide all the facilities and services with the commercial landuse. Commercial Office Zone is an office pre-dominated commercial zone that mainly supports the official works. It supports other types of commercial activities, too. Proposals for these areas involve large-scale buildings and require a high degree of accessibility and parking space for car users and delivery vehicles.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.13: Landuse Permitted in Commercial Zone (Office)

| Accounting, Auditing or Bookkeeping Services | Freight Handling, Storage & Distribution |
| --- | --- |
| Ambulance Service | Freight Transport Facility |
| Antique Store | General Store |
| Art Gallery, Art Studio/Workshop | Government Office |
| ATM Booth | Health Office |
| Auditorium, Meeting Halls, and Conference Facilities, Convention Center | Hotel |
| Auto Leasing or Rental Office | Indoor Amusement Centers, Game Arcades |
| Automobile Driving Academy | Mosque, Place of Worship |
| Automobile Wash | Multi-Storey Car Park |
| Bank & Financial Institution | Newspaper Stand |
| Billboards, Advertisements & Advertising Structure | Parking Lot (Commercial) |
| Boarding and Rooming House | Pathological Lab |
| Book Stall | Photocopying and Duplicating Services |
| Bulk Mail and Packaging | Photofinishing Laboratory & Studio |
| Bus Passenger Shelter | Post Office |
| Catering Service | Professional Office |
| Chinese Restaurant | Project Identification Signs |
| Cinema Hall | Project Office |
| Clinic | Property Management Signs |
| Coffee Shop/Tea Stall | Public Transport Facility |
| Commercial Office | Real Estate Office |
| Communication Service Facilities | Restaurant |
| Communication Tower Within Permitted Height | Retail Shops/Facilities |
| Computer Maintenance and Repair | Sales Office of Industries |
| Computer Sales & Services | Satellite Dish Antenna |
| Confectionery Shop | Software Development Firm |
| Conference Center | Stationery Store |
| Construction Company | Taxi Stand |
| Construction, Survey, Soil Testing Firms | Telephone Exchanges |

| Courier Service | Toys and Hobby Goods Processing and Supplies |
| --- | --- |
| Cyber Cafe | Training Centre |
| Day-care Center (Commercial or Nonprofit) | Utility Lines |
| Dental Laboratory | Vehicle Sales & Service, Leasing or Rental |
| Department Stores, Furniture & Variety Stores | Veterinarian Hospitals |
| Diagnostic Center | Water Pump/Reservoir |
| Doctor/Dentist Chamber | Woodlot |
| Food Court |  |

Landuse Conditionally Permitted

The following uses may be permitted or disallowed in the zone after review and approval by the authority/committee following appropriate procedure.

##### Table - 4.14: Landuse Conditionally Permitted in Commercial Zone (Office)

| Amusement and Recreation (Indoors) | Optical Goods Sales |
| --- | --- |
| Beauty and Body Service | Outdoor Cafe |
| Broadcast Studio/Recording Studio (No Audience) | Painting & Wallpaper Sales |
| Civic Administration | Paints and Varnishes Store |
| Concert Hall, Stage Shows | Parking Lot (Commercial) |
| Counseling Services | Patio Homes |
| Dental Clinic/Lab | Plantation (Except Narcotic Plant) |
| Diagnostic Center/Pathological Lab | Police Box/Barrack |
| Drug Store or Pharmacy | Printing, Publishing and Distributing |
| Fast Food Establishment/Food Kiosk | Psychiatric Hospital |
| Fire/Rescue Station | Radio/Television or T&T Station With Transmitter Tower |
| First Aid & Ambulance Service | Retail Shops Ancillary to Studio/Workshop |
| Fitness Centre | Super Store |
| Flowers, Nursery Stock and Florist Supplies | Theater (Indoor) |
| Fuelling Station | Transmission Lines |
| Garage | Vulcanizing Services |
| Lithographic or Print Shop | Workers’ Dormitory |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### General Industrial Zone

Promoting the development of industrial areas as the primary locations for uses including manufacturing, repairs, warehousing, distribution, open-storage, waste materials treatment and recovery, and transport operating centers has been earmarked. The development of inappropriate mixes of uses, such as office based industry and retailing will not normally be encouraged here.

It is an objective to ensure that, normally, new buildings in primarily industrial areas are single storey with high ceilings to provide versatility and adoptability to accommodate a broad range of types of plant, machinery and delivery vehicles throughout their design life.

It is an objective to ensure that the likely demand for car parking and delivery vehicle facilities are fully met within the site, to apply the highest standards of design, hard and soft landscaping and to ensure that pedestrian access is provided and that provision is made for public transport.

##### Purpose

The intent of this zone is to accommodate certain industrial uses which, based on their operational characteristics, is incompatible with residential, social and commercial environs. As a result, the establishment of such zones shall be restricted to areas geographically buffered from such environs, and the operations of such uses monitored by performance standards to ensure environmental compatibility.

- The purpose and intent of industrial zone is to provide areas in appropriate locations where various light and heavy industrial operations can be conducted without creating hazards to surrounding landuses. Industrial zones are cohesive, planned tracts, with all elements sharing the same or compatible architectural and landscaping themes within a parcel.

- Areas intended to accommodate a variety of industrial establishments which: 1) employ high environmental quality standards; 2) may function as an integral part of an overall development area; and, 3) have minimal impacts on adjacent uses.

- The purpose and intent of General industrial zone is to provide areas in appropriate locations for certain types of light manufacturing, processing or fabrication, warehousing establishments of non-objectionable products or adjacent uses because they are characterized by a minimum of material, processes or machinery likely to cause undesirable effects upon nearby or adjacent residential or commercial property.

General Industrial Zone is to accommodate Green, Orange A, Orange B types of industries of Department of Environment (DoE) prepared categories and Urban Area Plan designated light industries and general industries along with some other new types. Here the industries will not be noxious and heavy. DoE list of industries according to categories is attached as Annexure-IV.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.15: Landuse Permitted in General Industrial Zone

| Aluminium products | Musical instruments |
| --- | --- |
| Artificial Fibre Production | Motor vehicles repairing works |
| Assembling and manufacturing of clocks and watches | Newspaper Stand |
| Assembling and manufacturing of electrical and Electronic home products etc. | Packaging Industries |
| Assembling of motor vehicles | Perfumes, cosmetics |
| Assembling of telephones | Pharmaceutical Industry |
| ATM Booth | Photocopying and Duplicating Services |
| Automatic rice mill. | Photographic Film Factory |
| Bakery | Pipelines and Utility Lines |
| Bank & Financial Institution | Plantation (Except Narcotic Plant) |
| Bamboo and cane goods | Police Box/Barrack |
| Book-binding | Power Loom |
| Bicycle Assembly, Parts and Accessories | Printing and writing ink manufacturing Industry |

| Blacksmith | Printing Press |
| --- | --- |
| Bus Passenger Shelter | Printing, Publishing and Distributing |
| Carpet and mat production | Processing : fish, meat and food |
| Cinema Hall | Processing and bottling of drinking water and carbonated drinks |
| Clinic and Pathological lab | Production of artificial leather goods |
| Chocolate and lozenge Factory | Production of Comb, hair band, hair clip etc. |
| Cinema Hall | Production of gold ornaments. |
| Clinic and Pathological lab | Production of Pin, board pin, U Pin etc. |
| Cold Storage | Production of powder milk/condensed milk/dairy. |
| Communication Tower Within Permitted Height | Production of shoes and leather goods |
| Confectionery Shop | Production of spectacles frames. |
| Dry-cleaning | Production of utensils and souvenirs of brass and bronze. |
| Edible Oil | Public Transport Facility |
| Electric cable | Repairing of refrigerators |
| Engineering Works | Restaurant |
| Fabric Washing Plant | Retail Shops/Facilities |
| Factory for production of chocolate and lozenge. | Rope and coir mat production |
| Fast Food Establishment/Food Kiosk | Salt Industry |
| Fire/Rescue Station | Salt production |
| Flour (large) Mill | Salvage Processing |
| Freight Transport Facility | Salvage Yards |
| Furniture Manufacture of wood/iron, aluminium, etc. | Satellite Dish Antenna |
| Galvanizing | Sawmill, Chipping and Pallet Mill |
| Garments and sweater Factory | Shelter (Passers By) |
| Glass factory | Shoes and leather goods production |
| Glue (excluding animal glue) | Soap |
| Gold ornaments Production | Social Forestry |
| Grinding/husking wheat, rice, turmeric, chilly, pulses - machine above 0 Horse Power | Sodium silicate Factory |
| Grocery Store | Spinning mill |
| Hotel, multi-storied commercial building. | Sports goods Production |
| Household Appliance and Furniture Repair Service | Starch and glucose factory |
| Ice-cream | Stone grinding, cutting and polishing |
| Jute mill | Tea processing |
| Lime | Television, Radio or Electronics Repair (No Outside Storage) |
| Lithographic or Print Shop | Tyre re-treading |
| Manufacturing of Artificial flower | Transmission Lines |
| Machine Sheds | Truck Stop & Washing or Freight Terminal |
| Manufacture of Agricultural tools, equipments and small machinery. | Utensils made of clay and china clay/sanitary wares (ceramics) |

| Manufacture of Industrial tools, equipment and machinery. | Utility Lines |
| --- | --- |
| Manufacturing of wooden vessel | Water Pump/Reservoir |
| Match Factory | Water Purification Plant |
| Medical and surgical instrument production | Weaving and handloom. |
| Meat and Poultry (Packing & Processing) | Wood Products |
| Metal utensils/spoons etc | Wood treatment |
| Mosque, Place of Worship | Wood/iron/aluminium Furniture production |
| Motorcycle Sales Outlet | Woodlot |

Landuse Conditionally Permitted

The following uses may be permitted or denied in the zone after review and approval by the authority/committee following appropriate procedure.

##### Table - 4.16: Landuse Conditionally Permitted in General Industrial Zone

| Amusement and Recreation (Indoors) | Musical instruments |
| --- | --- |
| Appliance Store | Outdoor Fruit and Vegetable Markets |
| Carpet and mat production | Outside Bulk Storage |
| Cinema Hall | Overhead Water Storage Tanks |
| Clinic and Pathological lab | Painting and Wallpaper Sales |
| Cork items Production | Paints and Varnishes |
| Cyber Cafe | Parking Lot |
| Daycare Center (Commercial or Nonprofit) | Parking Lot (Commercial) |
| Doctor/Dentist Chamber | Pen and ball-pen Factory |
| Electrical and Electronic Equipment and Instruments Sales | Photographic Lab (except ultra violet and infra red) |
| Employee Housing | Plantation (Except Narcotic Plant) |
| Energy Installation | Plastic & rubber goods (excluding PVC) |
| Fast Food Establishment/Food Kiosk | Private Garages |
| Garages | Production of artificial leather goods |
| Galvanizing | Re-packing of milk powder (excluding production) |
| Glass factory | Retail Shops Ancillary to Studio/Workshop |
| Glue (excluding animal glue) | Rope and coir mat production |
| Grinding/husking wheat, rice, turmeric, chilly, pulses - machine above 0 Horse Power | Salt Industry |
| Gold ornaments Production | Salvage Processing |
| Grain & Feed Mills | Satellite Dish Antenna |
| Incineration Facility | Sawmill, Chipping and Pallet Mill |
| Laundry | Sodium silicate Factory |
| Lithographic or Print Shop | Sports goods (excluding plastic made items) |
| Manufacturing of Artificial flower | Super Store |
| Match Factory | Tea packing (excluding processing) |
| Medical and surgical instrument production | Tire re-treading |

| Motor Vehicle Fuelling Station/Gas Station | Washing Plant |
| --- | --- |
| Motorcycle Sales Outlet |  |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Heavy Industrial Zone

The purpose and intent of Heavy Industrial Zone is to provide areas in appropriate locations where various heavy industrial operations can be conducted without creating hazards to surrounding landuses.

Areas are intended to accommodate manufacturing and fabricating establishments that are generally characterized as producing noise, vibration, heavy freight traffic, fumes, and other forms of pollution which may be objectionable to adjacent uses is characterized as Heavy Industrial Zone.

Heavy industries are those industries listed as red industries in the DoE Industry Type list and industries listed in the Urban Area Plan as noxious industries including mining/quarrying.

##### Purpose

- To confine heavy, noisy and vibrating industries as they harm human living in the surrounding areas.

- To ensure that industrial development achieves high environmental standards of cleaner production, waste disposal, noise and air quality.

- To encourage the design and construction of energy efficient, functional and flexible industrial and commercial facilities.

- To restrict heavy and noxious industrial developments along major approach routes and major roads.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.17: Landuse Permitted in Heavy Industrial Zone

| Acids and their salts (organic or inorganic) Industry | Murat of Potash Manufacturing |
| --- | --- |
| Animal Glue | Newspaper Stand |
| Artificial Rubber Industry | Non-iron Basic Metals Industry |
| Asbestos Factory | Ordnance Factory |
| ATM Booth | Other Chemicals Industry |
| Bank & Financial Institution | Outside Bulk Storage |
| Basic Industrial Chemicals Industry | Paper and Pulp Mill |
| Battery Industry | Pesticides, Fungicides and Herbicides Industry |
| Bicycle Assembly, Parts and Accessories | Phosphorus and its Compounds/Derivatives Industry |
| Bitumen Industry | Photo Films, |
| Board Mills | Photo papers and Photo Chemicals Industry |
| Bus Passenger Shelter | Pipelines and Utility Lines |
| Carbon rod Industry | Plastic Raw Materials (PVC, PP/Iron, Polyesterin etc.) Factory |
| Cement Industry | Police Box/Barrack |
| Chemical dyes, Polish, Varnish, Enamel Paint Factory | Power plant |

| Chemical Industry | Public Transport Facility |
| --- | --- |
| Chlorine, Fluorine, Bromine, Iodine and their Compounds/Derivatives Industry | Raw materials of Medicines and Basic Drugs Industry |
| Communication Tower Within Permitted Height | Refractory |
| Detergent Factory | Refrigerator/Air-conditioner/Air-cooler manufacturing |
| Distillery | Re-rolling Mills |
| Effluent Treatment Plant | Rodenticide Industry |
| Electroplating | Satellite Dish Antenna |
| Explosive Industry | Saw Mills |
| Fabric Dyeing and Chemical Processing Industry | Scrap industry |
| Fibre-glass Factory | Sewage Treatment Plant |
| Fire/Rescue Station | Shelter (Passers By) |
| Flood Control Structures | Ship Manufacturing |
| Formaldehyde Industry | Social Forestry |
| Fuel Oil Refinery | Sodium Silicate Factory |
| Grocery Store | Sugar Mill |
| Heavy Engineering Workshop | Tannery |
| Heavy Repairing of Motor Vehicles | Tire and Tubes |
| Hospital | Tobacco Processing, Cigarette/Bidi Factory |
| Incineration Facility | Transmission Lines |
| Industrial Estate | Truck Stop & Washing |
| Industrial Gas (Oxygen, Nitrogen & Carbon-dioxide) Industry | TSP Fertilizer Factory |
| Iron and Steel Mill | Urea Fertilizer Factory |
| Jute Mill | Various Products made from Petroleum and Coal Industry |
| Life Saving Drugs Industry | Waste Incinerator |
| Lumber and building supply | Water Pump/Reservoir |
| Match Factory | Water Treatment Plant |
| Metallic Boat Manufacturing | Wooden Boat Manufacturing |
| Mosque, Place of Worship | Woodlot |

Landuse Conditionally Permitted

The following uses may be permitted or denied in the zone after review and approval by the authority/committee following appropriate procedure.

##### Table - 4.18: Landuse Conditionally Permitted in Heavy Industrial Zone

| Amusement and Recreation (Indoors) | Machine Sheds |
| --- | --- |
| Cyber Cafe | Motor Vehicle Fuelling Station/Gas Station |
| Daycare Center (Commercial or Nonprofit) | Nitrogen Compounds (Cyanide, Cyanamid etc.) Industry |
| Doctor/Dentist Chamber | Parking Lot (General) |
| Employee Housing | Parking Lot (Commercial) |
| Energy Installation | Plantation (Except Narcotic Plant) |

| Freight Transport Facility | Private Garages |
| --- | --- |
| Garages | Retail Shops Ancillary To Studio/Workshop |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Mixed Use Zone (Commercial-General Industrial)

There are areas where the mixture of uses is such that they can’t be segregated, the areas are declared as mixed use zone. An appropriate mix of landuses will be maintained in these types of zones. Four types of mixed use zones have been proposed here. Each of them has unique characteristics.

In Industrial cum Commercial area exclusive residential landuse will not be permitted except in the form of quarters for the employees within the Industrial complex.

##### Purpose

The purpose of the Commercial-Industrial zone is to provide areas in the city primarily for medium to high density mixed- use developments, with commercial, office and industrial uses that are sensitive to the adjacent residential areas; to support an urban growth centre where there will be a bulk of trade and commerce. The Commercial-Industrial zone is intended to achieve cohabitation of uses, while ensuring that negative impacts on residents are minimized. The spaces created in the Commercial-Industrial zone are intended to encourage a diminished need for motorized travel and shall possess characteristics of the both area of commercial and industrial. The Mixed Use Zone allows a wide variety of uses including, warehouse, shop, office and accommodation with condition.

- To preserve and promote viable industries that can coexist with more commercially oriented uses,

- To meet the need for a mix of lower rent bulky goods retailing, specialized industrial, commercial and service activities alongside general industry.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.19: Landuse Permitted in Mixed Use Zone (Commercial-General Industrial)

| Accounting, Auditing or Bookkeeping Services | General Store |
| --- | --- |
| Agricultural Chemicals, Pesticides or Fertilizers Shop | Grocery Store |
| Agricultural Sales and Services | Home Furnishings |
| Agro-Based Industry (Rice Mill, Saw Mill, Cold Storage) | Mosque, Place of Worship |
| Antique Store | Motorcycle Sales Outlet |
| Appliance Store | Multi-Storey Car Park |
| Art Gallery, Art Studio/Workshop | Musical Instrument Sales or Repair |
| Artisan’s Shop | Newspaper Stand |
| ATM Booth | Photocopying and Duplicating Services |
| Auto Leasing or Rental Office | Pipelines and Utility Lines |
| Auto Paint Shop | Preserved Fruits and Vegetables Facility/Cold Storage |
| Auto Parts and Accessory Sales (Indoors) | Project Identification Signs |
| Auto Repair Shop (With Garage) | Property Management Signs |
| Automobile Sales | Public Transport Facility |
| Automobile Wash | Public Utility Stations & Substations |

| Bakery or Confectionery Retail | Real Estate Office |
| --- | --- |
| Barber Shop | Refrigerator or Large Appliance Repair |
| Bicycle Shop | Research organization (Agriculture/Fisheries) |
| Billboards, Advertisements & Advertising Structure | Restaurant |
| Billiard Parlor/Pool Hall | Retail Shops/Facilities |
| Blacksmith | Rickshaw/Auto Rickshaw Stand |
| Boarding and Rooming House | Satellite Dish Antenna |
| Book or Stationery Store or Newsstand | Shelter (Passers By) |
| Broadcast Studio/Recording Studio (No Audience) | Social Forestry |
| Building Material Sales or Storage (Indoors) | Sporting Goods and Toys Sales |
| Bulk Fuel Sales Depot | Stone/Cut Stone Products Sales |
| Bulk Mail and Packaging | Storage & Warehousing |
| Bus Passenger Shelter | Television, Radio or Electronics Repair (No Outside Storage) |
| Cinema Hall | Toys and Hobby Goods Processing and Supplies |
| Commercial Recreational Buildings | Training Centre |
| Communication Service Facilities | Transmission Lines |
| Communication Tower Within Permitted Height | Truck Stop & Washing or Freight Terminal |
| Confectionery Shop | Utility Lines |
| Construction, Survey, Soil Testing Firms | Vehicle Sales & Service, Leasing or Rental |
| Courier Service | Warehousing |
| Doctor/Dentist Chamber | Water Pump/Reservoir |
| Firm Equipment Sales & Service | Wood Products |
| Fuel and Ice Dealers | Woodlot |
| Funeral Services |  |

Landuse Conditionally Permitted

The following uses may be permitted or disallowed in the zone after review and approval by the authority/committee.

##### Table - 4.20: Landuse Conditionally Permitted in Mixed Use Zone (Commercial-General Industrial)

| Assembling and manufacturing of clocks and watches | Freight Handling, Storage & Distribution |
| --- | --- |
| Assembling and manufacturing of electrical and Electronic home products etc. | Freight Transport Facility |
| Assembling and manufacturing of electrical and Electronic home products etc. | Freight Transport Facility |
| Assembling and manufacturing of toys (except plastic items) | Gallery/Museum |
| Assembling of telephones | Garages |
| Auction Market | Government Office |
| Bank & Financial Institution | Grain & Feed Mills |
| Beauty and Body Service | Grocery Store |
| Bicycle Assembly, Parts and Accessories | Hotel or Motel |
| Blacksmith | Household Appliance and Furniture Repair Service |
| Bus Passenger Shelter | Household Appliance and Furniture Repair Service |

| Coffee Shop/Tea Stall | Machine Sheds |
| --- | --- |
| Communication Tower Within Permitted Height | Meat and Poultry (Packing & Processing) |
| Computer Maintenance and Repair | Meat and Poultry (Packing & Processing) |
| Computer Sales & Services | Optical Goods Sales |
| Concert Hall, Stage Shows | Outdoor Fruit and Vegetable Markets |
| Confectionery Shop | Outside Bulk Storage |
| Conference Center | Painting and Wallpaper Sales |
| Construction Company | Paints and Varnishes |
| Cottage | Plantation (Except Narcotic Plant) |
| Counseling Services | Police Box/Barrack |
| Craft Workshop | Printing, Publishing and Distributing |
| Cyber Cafe | Project Office |
| Department Stores, Furniture & Variety Stores | Retail Shops Ancillary to Studio/Workshop |
| Drug Store or Pharmacy | Telephone Exchanges |
| Energy Installation | Television, Radio or Electronics Repair (No Outside Storage) |
| Fabric Store | Theater (Indoor) |
| Fast Food Establishment/Food Kiosk | Trade Shows |
| Fire Rescue Station | Transmission Lines |
| Forest Products Sales | Truck Stop & Washing or Freight Terminal |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Mixed use zone (Residential–Commercial)

Mixed-use (Residential-Commercial) development shall mean a development consisting of one or more lots developed as a cohesive project and designed with a blend of various compatible uses such as commercial and residential. These landuse areas will contain residential and limited commercial activities only such as small retail, general store, food kiosk etc.

##### Purpose

Commercial uses within the Mixed Use zone should be restricted to those uses that have a minimal amenity impact on adjoining uses and surrounding residential use. The main purposes of this type of landuse classification are:

- To provide for a mixed use development centre in conjunction with the development plan for the overall development, and to provide for high-density residential development, and to identify, reinforce, strengthen and promote urban design concepts and linkages with the existing town centre activity areas so that the pressure on vacant land are minimized.

- The aim is to develop a zone scale retail development in a mixed use centre by densification of appropriate commercial and residential developments ensuring a mix of commercial, recreational, community, educational, civic, cultural, leisure, residential uses, urban streets and urban open spaces, while delivering a quality urban environment which will enhance the quality of life of resident, visitor and workers alike.

- Provide for a range of commercial and service activities at a scale that will protect the planned hierarchy of commercial centres and the city’s preferred locations for office development.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.21: Landuse Permitted in Mixed Use Zone (Residential-Commercial)

| Accounting, Auditing or Bookkeeping Services | Employee Housing |
| --- | --- |
| Addiction Treatment Center | Fabric Store |
| Agricultural Sales and Services | Fast Food Establishment/Food Kiosk |
| Antique Store | Funeral Services |
| Apartments | General Store |
| Appliance Store | Grocery Store |
| Art Gallery, Art Studio/Workshop | Guest House |
| Artisan’s Shop | Hospital |
| Assisted Living or Elderly Home | Housing Projects |
| ATM Booth | Individual Housing |
| Auditorium, Meeting Halls, and Conference Facilities, Convention | Jewelry and Silverware Sales |
| Auto Leasing or Rental Office | Landscape and Horticultural Services |
| Automobile Driving Academy | Mosque, Place of Worship |
| Automobile Wash | Newspaper Stand |
| Bakery or Confectionery Retail | Nursery School |
| Bank & Financial Institution | Photocopying and Duplicating Services |
| Barber Shop | Pipelines and Utility Lines |
| Bicycle Shop | Primary School |
| Billboards, Advertisements & Advertising Structure | Project Identification Signs |
| Billiard Parlor/Pool Hall | Property Management Signs |
| Blacksmith | Public Transport Facility |
| Boarding and Rooming House | Resort |
| Book or Stationery Store or News Stand | Rickshaw/Auto Rickshaw Stand |
| Bus Passenger Shelter | Satellite Dish Antenna |
| Child Daycare/Pre-school | Shelter (Passers By) |
| Children’s Park | Shoe Repair or Shoeshine Shop (Small) |
| Cleaning/Laundry Shop | Slaughter House |
| Commercial Recreational Buildings | Social Forestry |
| Communication Service Facilities | Social organization |
| Communication Tower Within Permitted Height | Software Development |
| Community Center | Special Dwelling |
| Condominium or Apartment | Toys and Hobby Goods Processing and Supplies |
| Confectionery Shop | Training Centre |
| Correctional Institution | Transmission Lines |
| Courier Service | Utility Lines |
| Cyber Cafe | Vehicle Sales & Service, Leasing or Rental |
| Daycare Center (Commercial or Nonprofit) | Warehousing |

| Doctor/Dentist Chamber | Water Pump/Reservoir |
| --- | --- |
| Dormitory | Woodlot |

Landuse Conditionally Permitted

The following uses may be permitted or disallowed in the zone after review and approval by the authority/committee.

##### Table-4.22: Landuse Conditionally Permitted in Mixed Use Zone (Residential-Commercial)

| Agricultural Chemicals, Pesticides or Fertilizers Shop | Graveyard/Cemetery |
| --- | --- |
| Amusement and Recreation (Indoors) | Health Office, Dental Laboratory, Clinic or Lab |
| Beauty and Body Service | Hotel or Motel |
| Broadcast Studio/Recording Studio (No Audience) | Household Appliance and Furniture Repair Service |
| Building Maintenance/Cleaning Services, No Outside Storage | Indoor Amusement Centers, Game Arcades |
| Building Material Sales or Storage (Indoors) | Indoor Theatre |
| Coffee Shop/Tea Stall | Lithographic or Print Shop |
| Commercial Office | Market (Bazaar) |
| Computer Maintenance and Repair | Musical Instrument Sales or Repair |
| Computer Sales & Services | Optical Goods Sales |
| Concert Hall, Stage Shows | Outdoor Café |
| Conference Center | Outdoor Fruit and Vegetable Markets |
| Construction Company | Painting and Wallpaper Sales |
| Construction, Survey, Soil Testing Firms | Paints and Varnishes |
| Cottage | Patio Homes |
| Counseling Services | Photofinishing Laboratory & Studio |
| Craft Workshop | Plantation |
| Crematorium | Poultry |
| Cultural Exhibits and Libraries | Printing, Publishing and Distributing |
| Department Stores, Furniture & Variety Stores | Project Office |
| Drug Store or Pharmacy | Psychiatric Hospital |
| Energy Installation | Radio/Television or T&T Station With Transmitter Tower |
| Fitness Centre | Refrigerator or Large Appliance Repair |
| Flowers, Nursery Stock and Florist Supplies | Restaurant |
| Freight Handling, Storage & Distribution | Retail Shops/Facilities |
| Freight Transport Facility | Retail Shops Ancillary To Studio/Workshop |
| Gaming Clubs | Sporting Goods and Toys Sales |
| Garages | Sports and Recreation Club, Firing Range: Indoor |
| Garden Center or Retail Nursery | Telephone Exchanges |
| Government Office | Television, Radio or Electronics Repair (No Outside Storage) |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Mixed Use Zone (Residential-Commercial-General Industrial)

The Residential-Commercial-General Industrial zone is intended to achieve cohabitation of uses, while ensuring that negative impacts on residents are minimized. The spaces created in this zone are intended to encourage a diminished need for motorized travel and shall possess characteristics of all of the three - Residential, Commercial and Industrial.

##### Purpose

The main purpose of the zone is to support the diversification and expansion of the area’s industrial base and employment growth as well as to facilitate investment in a wide range of industrial and related activities, with efficient land utilization and provision of infrastructure. To make provision for small-scale services that support surrounding industrial activities, or which meet the needs of the local workforce is a primary aim of this type of landuse. Provide convenient access for the area and regional residents to industrial goods, services and employment opportunities, too, is an important purpose of this zone.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table-4.23: Landuse Permitted in Mixed Use Zone (Residential-Commercial-General Industrial)

| Accounting, Auditing or Bookkeeping Services | Fire/Rescue Station |
| --- | --- |
| Antique Store | General Store |
| Appliance Store | Grocery Store |
| ATM Booth | Guest House |
| Auditorium, Meeting Halls, and Conference Facilities, Convention | Hospital |
| Auto Leasing or Rental Office | Landscape and Horticultural Services |
| Auto Paint Shop | Mosque, Place of Worship |
| Auto Parts and Accessory Sales (Indoors) | Newspaper Stand |
| Auto Repair Shop (With Garage) | Nursery School |
| Automobile Driving Academy | Photocopying and Duplicating Services |
| Automobile Sales | Pipelines and Utility Lines |
| Automobile Wash | Police Box/Barrack |
| Bakery or Confectionery Retail | Primary School |
| Bank & Financial Institution | Project Identification Signs |
| Barber Shop | Property Management Signs |
| Bicycle Assembly, Parts and Accessories | Public Transport Facility |
| Bicycle Shop | Rickshaw/Auto Rickshaw Stand |
| Billboards, Advertisements & Advertising Structure | Satellite Dish Antenna |
| Billiard Parlor/Pool Hall | Service Garage |
| Blacksmith | Service Stations, Full Service (With Minor Repair) |
| Boarding and Rooming House | Sewage Treatment Plant |
| Book or Stationery Store or News Stand | Shelter (Passers By) |
| Bulk Mail and Packaging | Shoe Repair or Shoeshine Shop (Small) |
| Bus Passenger Shelter | Social Forestry |
| Children’s Park | Social organization |
| Cleaning/Laundry Shop | Solid Waste Recycling Plant |

| Commercial Recreational Buildings | Special Dwelling |
| --- | --- |
| Communication Service Facilities | Transmission Lines |
| Communication Tower Within Permitted Height | Utility Lines |
| Confectionery Shop | Water Pump/Reservoir |
| Construction, Survey, Soil Testing Firms | Woodlot |
| Courier Service |  |

Landuse Conditionally Permitted

The following uses may be permitted or denied in the zone after review and approval by the authority/committee.

##### Table-4.24: Landuse Conditionally Permitted in Mixed Use Zone (Residential-Commercial-General Industrial)

| Agricultural Chemicals, Pesticides or Fertilizers Shop | Gaming Clubs |
| --- | --- |
| Amusement and Recreation (Indoors) | Garages |
| Auction Market | Garden Center or Retail Nursery |
| Beauty and Body Service | Government Office |
| Building Maintenance/Cleaning Services, No Outside Storage | Grain & Feed Mills |
| Building Material Sales or Storage (Indoors) | Health Office, Dental Laboratory, Clinic or Lab |
| Bulk Fuel Sales Depot | Hotel or Motel |
| Coffee Shop/Tea Stall | Household Appliance and Furniture Repair Service |
| Computer Maintenance and Repair | Incineration Facility |
| Computer Sales & Services | Indoor Theatre |
| Concert Hall, Stage Shows | Jewellery and Silverware Sales |
| Condominium or Apartment | Lithographic or Print Shop |
| Conference Center | Market (Bazaar) |
| Construction Company | Musical Instrument Sales or Repair |
| Correctional Institution | Outdoor Fruit and Vegetable Markets |
| Counseling Services | Painting and Wallpaper Sales |
| Craft Workshop | Paints and Varnishes |
| Cyber Cafe | Pet Store |
| Daycare Center (Commercial or Nonprofit) | Photofinishing Laboratory & Studio |
| Department Stores, Furniture & Variety Stores | Plantation |
| Drug Store or Pharmacy | Project Office |
| Employee Housing | Psychiatric Hospital |
| Energy Installation | Refrigerator or Large Appliance Repair |
| Fabric Store | Restaurant |
| Fast Food Establishment/Food Kiosk | Retail Shops/Facilities |
| Firm Equipment Sales & Service | Retail Shops Ancillary To Studio/Workshop |
| Fitness Centre | Sporting Goods and Toys Sales |
| Flowers, Nursery Stock and Florist Supplies | Sports and Recreation Club, Firing Range: Indoor |
| Forest Products Sales | Telephone Exchanges |
| Freight Transport Facility | Television, Radio or Electronics Repair (No Outside Storage) |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Mixed Use Zone (Residential-General Industrial)

The areas with the mix of residence and industry will make up the Residential-General Industrial zone that will create an industrial urban environment to strengthen the industrial base of the area as well as the entire country.

##### Purpose

The main purpose of the zone is to provide an industrial area supported by employees’ housing, additional civic amenities, utilities and services. This zone will be composed of some light industries that are declared as green categories of industries by the Department of Environment (DoE) with some additional facilities like appliance store; bakery or confectionery shop; bank & financial institution; barber shop; book or stationery store or newspaper stand; caretaker dwelling; child day-care or pre-school; cleaning or laundry shop; communication service facilities, cyber cafe; addiction treatment centre etc.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.25: Landuse Permitted in Mixed Use Zone (Residential-General Industrial)

| Addiction Treatment Center | Fire/Rescue Station |
| --- | --- |
| Appliance Store | General Store |
| Art Gallery, Art Studio/Workshop | Grocery Store |
| Assisted Living or Elderly Home | Newspaper Stand |
| ATM Booth | Nursery School |
| Auditorium, Meeting Halls, and Conference Facilities, Convention | Photocopying and Duplicating Services |
| Auto Leasing or Rental Office | Pipelines and Utility Lines |
| Bank & Financial Institution | Police Box/Barrack |
| Barber Shop | Project Identification Signs |
| Bicycle Assembly, Parts and Accessories | Property Management Signs |
| Bicycle Shop | Public Transport Facility |
| Billiard Parlor/Pool Hall | Rickshaw/Auto Rickshaw Stand |
| Blacksmith | Satellite Dish Antenna |
| Book or Stationery Store or News Stand | Service Garage |
| Broadcast Studio/Recording Studio (No Audience) | Service Stations, Full Service (With Minor Repair) |
| Bulk Fuel Sales Depot | Shelter (Passers By) |
| Bus Passenger Shelter | Shoe Repair or Shoeshine Shop (Small) |
| Child Daycare/Preschool | Social Forestry |
| Children’s Park | Social organization |
| Cleaning/Laundry Shop | Special Dwelling |
| Commercial Recreational Buildings | Training Centre |
| Communication Service Facilities | Transmission Lines |
| Communication Tower Within Permitted Height | Utility Lines |
| Condominium or Apartment | Water Pump/Reservoir |
| Confectionery Shop | Woodlot |
| Cyber Cafe |  |

Landuse Conditionally Permitted

The following uses may be permitted or denied in the zone after review and approval by the authority/committee.

##### Table - 4.26: Landuse Conditionally Permitted in Mixed Use Zone (Residential-General Industrial)

| Amusement and Recreation (Indoors) | Hotel or Motel |
| --- | --- |
| Beauty and Body Service | Household Appliance and Furniture Repair Service |
| Building Maintenance/Cleaning Services, No Outside Storage | Incineration Facility |
| Building Material Sales or Storage (Indoors) | Lithographic or Print Shop |
| Coffee Shop/Tea Stall | Market (Bazar) |
| Computer Maintenance and Repair | Musical Instrument Sales or Repair |
| Computer Sales & Services | Outdoor Fruit and Vegetable Markets |
| Concert Hall, Stage Shows | Overhead Water Storage Tanks |
| Conference Center | Painting and Wallpaper Sales |
| Daycare Center (Commercial or Nonprofit) | Plantation |
| Department Stores, Furniture & Variety Stores | Psychiatric Hospital |
| Drug Store or Pharmacy | Refrigerator or Large Appliance Repair |
| Employee Housing | Restaurant |
| Energy Installation | Retail Shops/Facilities |
| Fast Food Establishment/Food Kiosk | Retail Shops Ancillary To Studio/Workshop |
| Farm Equipment Sales & Service | Sporting Goods and Toys Sales |
| Flowers, Nursery Stock and Florist Supplies | Sports and Recreation Club, Firing Range: Indoor |
| Forest Products Sales | Telephone Exchanges |
| Freight Transport Facility | Television, Radio or Electronics Repair (No Outside Storage) |
| Garages |  |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Institutional Zone

Institutional Zones are designed to provide some institutional boundaries. This will help improving the socio-economic condition of the area by enhancing educational status.

##### Purpose

The purpose of the zone is mainly to provide locations for institutional uses such as libraries, public and private schools of special needs, colleges or universities, research centres, cultural academies etc. Housing facilities for the students, teachers, employees and their families within the compound are also allowed in the zone.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.27: Landuse Permitted in Institutional Zone

| Addiction Treatment Center | Newspaper Stand |
| --- | --- |
| Art Gallery, Art Studio/Workshop | Nursery School |
| ATM Booth | Outdoor Religious Events |

| Automobile Driving Academy | Photocopying and Duplicating Services |
| --- | --- |
| Billboards, Advertisements & Advertising Structure | Post Office |
| Bus Passenger Shelter | Primary School |
| Child Daycare/Preschool | Professional Office |
| College, University, Technical Institute | Project Identification Signs |
| Communication Service Facilities | Property Management Signs |
| Communication Tower Within Permitted Height | Public Transport Facility |
| Confectionery Shop | Residential use ancillary to Institutions |
| Conference Center | School (Retarded) |
| Correctional Institution | Scientific Research Establishment |
| Cultural Exhibits and Libraries | Shelter (Passers by) |
| Cyber Cafe | Social Forestry |
| Dormitory | Specialized School: Dance, Art, Music & Others |
| Freight Transport Facility | Training Centre |
| General Store | Transmission Lines |
| Grocery Store | Utility Lines |
| High School | Veterinary School/College and Hospital |
| Hospital | Vocational, Business, Secretarial School |
| Lithographic or Print Shop | Water Pump/Reservoir |
| Mosque, Place of Worship | Woodlot |
| Multi-Storey Car Park |  |

Landuse Conditionally Permitted

The following uses may be permitted or denied in the zone after review and approval by the authority/committee.

##### Table - 4.28: Landuse Conditionally Permitted in Institutional Zone

| Auditorium, Meeting Halls, and Conference Facilities, Convention | Gallery/Museum |
| --- | --- |
| Bank & Financial Institution | Garages |
| Barber Shop | Indoor Theatre |
| Boarding and Rooming House | orphanage |
| Coffee Shop/Tea Stall | Outdoor Cafe |
| Counseling Services | Parking Lot |
| Courier Service | Pipelines and Utility Lines |
| Daycare Center (Commercial or Nonprofit) | Plantation |
| Doctor/Dentist Chamber | Postal Facilities |
| Drug Store or Pharmacy | Psychiatric Hospital |
| Fast Food Establishment/Food Kiosk | Stationery Store or Newsstand |
| Flowers, Nursery Stock and Florist Supplies |  |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Administrative Zone

Administrative zone will allow agglomeration of administrative structures to make a convenient communication among them. The zone houses important government machinery such as the National Assembly Complex, Nagar Bhaban, Secretariat, High Court Complex, offices of the various Public Sector Agencies etc.

##### Purpose

The main purpose of the zone is to agglomerate the administrative structures within a certain boundary for convenience of people.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.29: Landuse Permitted in Administrative Zone

| Accounting, Auditing or Bookkeeping Services | Newspaper Stand |
| --- | --- |
| ATM Booth | Outdoor Religious Events |
| Bus Passenger Shelter | Photocopying and Duplicating Services |
| Civil Administration | Post Office |
| Communication Service Facilities | Professional Office |
| Communication Tower within Permitted Height | Project Office |
| Confectionery Shop | Public Transport Facility |
| Construction, Survey, Soil Testing Firms | Satellite Dish Antenna |
| Cultural Exhibits and Libraries | Scientific Research Establishment |
| Cyber Cafe | Shelter (Passers By) |
| Emergency Shelter | Social Forestry |
| Freight Transport Facility | Training Centre |
| General Store | Transmission Lines |
| Government Office, Guest House | Utility Lines, Water Pump/Reservoir |
| Grocery Store | Woodlot |
| Multi-Storey Car Park |  |

Landuse Conditionally Permitted

The following uses may be permitted or denied in the zone after review and approval by the authority/committee.

##### Table - 4.30: Landuse Conditionally Permitted in Administrative Zone

| Amusement and Recreation (Indoors) | Flowers, Nursery Stock and Florist Supplies |
| --- | --- |
| Auditorium, Meeting Halls, and Conference Facilities, Convention | Freight Handling, Storage & Distribution |
| Bank & Financial Institution | Freight Yard |
| Boarding and Rooming House | Gallery/Museum |
| Book or Stationery Store or Newsstand | Garages |
| Coffee Shop/Tea Stall | Lithographic or Print Shop |
| Conference Center | Mosque, Place of Worship |
| Courier Service | Outdoor Café |
| Daycare Center (Commercial or Nonprofit) | Parking Lot |
| Detention Facilities | Parking Lot (Commercial) |
| Doctor/Dentist Chamber | Pipelines and Utility Lines |
| Energy Installation | Plantation (Except Narcotic Plant) |

| Fast Food Establishment/Food Kiosk | Police Box/Barrack |
| --- | --- |
| Fire/Rescue Station | Postal Facilities |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Agricultural Zone

Agricultural Zone is the zone of ‘food production’ where predominantly agricultural production will be encouraged. All types of agricultural activity such as crop production; aquaculture etc. will be dominant and will be permitted to ensure food security. The Agricultural zone is intended to prevent scattered indiscriminate conversion of crop land into settlements.

##### Purpose

- To safeguard food security by protecting agricultural land from indiscriminate conversion in the name of development.

- To promote farming activity and farming operation in the land especially suitable for the purpose.

- To conserve agricultural lands within urban areas and encourage the long term viability of agricultural operations and the supporting agricultural industry.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.31: Landuse Permitted in Agricultural Zone

| Animal Shelter | Irrigation Facilities (Irrigation Canal, Culvert, Flood Wall etc) |
| --- | --- |
| Aquaculture | Shallow Tube Well |
| Aquatic Recreation Facility (Without Structure) | Social Forestry |
| Arboriculture | Static Transformer Stations |
| Cash Crop Cultivation | Temporary Structure (Agricultural) |
| Dairy Farming | Transmission Lines |
| Deep Tube Well | Tree Plantation (Except Narcotic Plant) |
| Duckery | Utility Lines |
| Food Grain Cultivation | Vegetable Cultivation |
| Horticulture | Woodlot |

Landuse Conditionally Permitted

The following uses may be permitted or disallowed in the zone after review and approval by the authority/committee following appropriate procedure subject to compliance of the condition mentioned in the requirement by the applicant.

##### Table - 4.32: Landuse Conditionally Permitted in Agricultural Zone

| Communication Tower Within Permitted Height | Graveyard/Cemetery |
| --- | --- |
| Crematorium |  |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Flood Flow Zone

The area over which the flow of floodwater during monsoon passes through is termed here as flood flow zone.

##### Purpose

The intent of the Flood Flow Zone is to protect adverse hydrological impact, human life and health, minimize property damage, encourage appropriate construction practices, and minimize public and private losses due to overflow of water.

Additionally, the zone is intended to maintain the sound use and development of flood-prone areas and to restrict potential homestead land buyers by notifying that property is in a flood area. The provisions of the zone is intended to minimize damage to public facilities and utilities such as water and gas mains, electric, telephone, and sewer lines, street and bridges located in the floodplain, and to minimize expenditures of public money for costly flood control projects and rescue and relief efforts associated with flooding.

##### Landuse Permitted

In the zone, uses permitted by the Authority are:

##### Table - 4.33: Landuse Permitted in Flood Flow Zone

| Aquatic Recreation Facility (Without Structure) | Plantation (Except Narcotic Plant) |
| --- | --- |
| Dry Season Play Field | Static Transformer Stations |
| Marina/Boating Facility | Transmission Lines |
| Pipelines and Utility Lines | Utility Lines |

Landuse Conditionally Permitted

The following uses may be permitted or disallowed in the zone after review and approval by the authority/committee.

##### Table - 4.34: Landuse Conditionally Permitted in Flood Flow Zone

| Communication Tower Within Permitted Height | Poultry |
| --- | --- |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Open Space

The primary uses in the zone include playgrounds, parks, and other areas for outdoor activities, sports centers, sports pitches, outdoor recreation and landscaped areas. High standards of accessibility are essential in the zone. Accessibility standard is related to the use of that particular open space. For example, local amenity areas and playgrounds may require emphasis on access for pedestrians and cyclists. Sports Centers serving a wider catchments area will require accessibility by public transport and car users.

##### Purpose

To provide for the active and passive recreational needs of the city and the protection of its bountiful natural resources as well as to protect the natural and aesthetic qualities of the area for the general welfare of the community. The purpose of the zone is:

- To provide for, protect and improve the provision, attractiveness, accessibility and amenity value of public open space and amenity areas.

- To provide for, protect and improve the provision, attractiveness and accessibility of public open space intended for use of recreational or amenity purposes. Only development that is incidental to, or contributes to the enjoyment of open space, amenity or recreational facilities will be permitted within this zone.

- To preserve and provide for open space and recreational amenities. The zoning objective seeks to provide recreational and amenity resources for urban and rural populations subject to strict development controls. Only community facilities and other recreational uses will be considered and encouraged by the Planning Authority.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.35: Landuse Permitted in Open Space

| Botanical Garden & Arborium | Playing Field |
| --- | --- |
| Bus Passenger Shelter | Social Forestry |
| Carnivals and Fairs | Special Function Tent |
| Circus | Tennis Club/Basket Ball Court |
| Landscape and Horticultural Services | Transmission Lines |
| Memorial Structure | Urban-Nature Reserve |
| Open Theater | Utility Lines |
| Park and Recreation Facilities (General) | Woodlot |
| Pipelines and Utility Lines | Zoo |
| Plantation (Except Narcotic Plant) |  |

Landuse Conditionally Permitted

The following uses may be permitted or denied in the zone after review and approval by the authority/committee.

##### Table - 4.36: Landuse Conditionally Permitted in Open Space

| Communication Tower Within Permitted Height | Motorized Recreation |
| --- | --- |
| Fitness Centre | Trade Shows |
| Flowers, Nursery Stock and Florist Supplies |  |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Overlay Zone

The uses that are not compatible to the surrounding landuse but to honour the aspiration of the people or for any other unavoidable reason shall continue to maintain its present status are called overlay sites and the sites compose the Overlay Zone.

##### Purpose

There are some very important purposes of overlay zones. These are not zones; actually, these are sites only. The locally, regionally or nationally important things or uses that don’t conform to surrounding landuses will remain unchanged that are called Overlay Zone. No other use except the use of overlay site is permitted in the zone.

There is no scope for permitting or conditionally permitting the functions or uses as the zone itself is an overlay. The present and proposed use of the zone will continue until the next zoning regulation is imposed on those specific parcels of land.

There are a variety of overlay zones within the area. Some of the important types of overlay and their purpose are listed below:

###### Historic Preservation Overlay Site

The places that are historically important as fort or historic building will not be demolished and these will remain as Historic Preservation Overlay Site. Recognizing the area’s unique character and promoting the conservation and preservation of the area’s historic resources and properties, to maintain, preserve, protect and enhance the architectural

excellence, cultural significance, economic vitality, visual quality and historic importance of the area overlay zone is delineated.

##### Purpose

The purpose of the zone is to provide for protection against destruction or encroachment upon historic areas, buildings, monuments or other features or buildings and structures of recognized architectural significance which contribute or will contribute to the cultural, social, economic, political, artistic or architectural heritage. The main purposes of the zone are:

To protect the area’s valuable cultural and historic resources from degradation or destruction and promote the adoptive re- use of older buildings and structures, and enhance residents’ experience of the area’s environmental resources through the protection of scenic landscapes and roads, to maintain non-residential uses in historic structures in the Historic Preservation Overlay Zone.

The Zone is designed to protect designated landmarks and other historic or architectural features and their surroundings within a reasonable distance from destruction, damage, defacement and obviously incongruous development or uses of land. It is also designed to ensure that buildings, structures or signs shall be erected, reconstructed, altered or restored so as to be architecturally compatible with the historic landmark buildings or structures within the Zone.

It is the intent and purpose of the Historic Preservation Overlay Site is to encourage and promote the educational and cultural welfare of the citizens by preserving and protecting historic structures, sites, monuments, streets, areas etc. which serve as visible reminders of the history and cultural heritage of the community. Furthermore, the purpose is, by stabilizing and improving property values in historic areas, and to encourage construction and development that will be harmonious with existing historic structures and areas.

###### Environmental Protection Overlay Site

To preserve, protect and manage the area’s natural resources including streams and other water bodies, wetlands, woodlands, and wildlife habitats, development in some areas will be precluded. This will be treated as Environmental Protection Overlay Site.

##### Purpose

The purpose of the zone is to protect from misuse and to ensure, for future generations the areas of environmentally sensitive, wetlands, marshes, rivers, creeks, and other natural resources critical to the ecosystems they support, and however, the above is not intended to discourage quality development.

Due to the fragile nature of these resources, development standards for the zone generally are more rigid than elsewhere in the area, requiring closer attention to the environment, and mitigation of land disturbing activity which would negatively impact such resources.

To promote the use of flexible and creative planning, open space management, land stewardship, and engineering practices to facilitate protection and enhancement of the natural, cultural agricultural, scenic, historic and recreational resources of the area.

To encourage an attractive community that represents the natural environment provides for landuses located in proper relationship to each other, and to land, energy, water and air resources, and to further development at an orderly pace.

###### Graveyard Overlay Site

The existing graveyards have been treated as Graveyard Overlay Site where there will have restrictions to build it other way.

###### Sports and Recreation Overlay Site

Some open spaces, water bodies etc. are demarcated as Sports and Recreation Overlay Site to safeguard the future need of the people. Here no use will be permitted except sports and recreation.

###### Special Use Overlay Site

There are some areas that will be used for special use. These are termed as Special Use Overlay Site. Special temporary events like Fairs, Hats etc. will be permitted in the zone.

##### Rural Settlement Zone

Rural Settlement Zones are the island like isolated clusters of rural homesteads that preserve country’s traditional agrarian community beside agricultural zones within DMDP jurisdiction from indiscriminate and insane conversion into urban use so as to safeguard food security; preserve appropriate opening in between mounds that are essential to allow uninterrupted flow of annual flood water and safeguard the complex hydrological system.

The intent of this classification is to preserve, sustain, and protect rural areas and resources, particularly forest and agricultural land from suburban encroachment and maintain a balanced rural-urban environment.

The retention of open lands, woodlands, plantations and farmlands, which make up a large part of the area, are essential for clean air, water, wildlife, many natural cycles, and a balanced environment, among other things. Even more essential from an economic perspective are the agricultural lands and farming operations in the area.

##### Purpose

The purpose of the zone is to protect the natural environment; promote appropriate compact development and to promote creative developments that provide a range of housing styles and prices that suit the needs of local people.

- Protect the natural environment,

- Making development economically viable by concentrating the activities,

- To provide for development in an orderly manner that preserves the rural character of the area by guiding higher density residential and commercial development according to compatible and supportive landuses, available infrastructure, and ancillary facilities and services.

##### Rural Homestead

Rural homestead zone is the zone that contains the traditional rural character of the area. In this zone housing that are compatible to this character, mainly farmhouse, single or multifamily housing are the focus of the area. The buildings should be maximum two-storeyed so as to maintain the rural nature.

##### Purpose

As stated earlier, Isolated Rural Homestead will preserve the rural and traditional character of the area. Isolated Rural Homestead zone will be composed of dwelling, small shops, some special uses and pure landscaping elements. Cluster uses that are compatible with and complementary to existing facilities, particularly with regard to food processing and warehousing and the markets, including some small scale food retailing are permitted here.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.37: Landuse Permitted in Rural Settlement Zone

| Agricultural Dwellings | NGO/CBO Facilities |
| --- | --- |
| Animal Husbandry | Nursery School |
| Animal Shelter | Orphanage |
| Child Daycare/Preschool | Outdoor Religious Events (Eidgah) |
| Communication Tower Within Permitted Height | Plantation (Except Narcotic Plant) |
| Cottage | Playing Field |
| Crematorium | Primary School |

| Dairy Firming | Satellite Dish Antenna |
| --- | --- |
| General Store | Social Forestry |
| Graveyard/Cemetery | Special Dwelling (e.g. Dorm For Physically Challenged etc.) |
| Grocery Store | Specialized School: Dance, Art, Music, Physically Challenged & Others |
| Handloom (Cottage Industry) | Static Electrical Sub Stations |
| Housing For Seasonal Farm Labor | Temporary Shed/Tent |
| Memorial Structure | Transmission Lines |
| Mosque, Place of Worship | Utility Lines |
| Newspaper Stand | Woodlot |

Landuse Conditionally Permitted

The following uses may be permitted or disallowed in the zone after review and approval by the authority/committee following appropriate procedure while the application meets the criteria mentioned in the requirement.

##### Table - 4.38: Landuse Conditionally Permitted in Rural Settlement Zone

| Artisan’s workshop (Potter, Blacksmith, Goldsmith Etc.) | Garden Center or Retail Nursery |
| --- | --- |
| Emergency Shelter | Research organization (Agriculture/Fisheries) |
| Energy Installation | Sports and Recreation Club, Firing Range: Indoor |
| Fish Hatchery |  |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Growth Center

According to the location theorists, “Such settlements which are nuclei or central to their influence area are called growth centres”. In the context Bangladesh, Growth Centres are rural market places that have been identified by the Planning Commission on the basis of socio-economic and administrative criteria for making development investment. There are about 8,000 rural market places throughout Bangladesh that are popularly known as 'hat' or 'bazaar'. Realizing their importance in the rural sector, government felt the need to upgrade them with some infrastructure facilities in order to enhance their efficiency. However, due to fund constraint it was not possible to include all the 'hats' or 'bazaars' for development investment. Thus, initially 1,400 of them were selected in 1984 and later in 1993; another 700 'hats' or 'bazaars' were added to this list to take the final number to 2,100. In the area, there are some growth centres that contain substantially the following characteristics:

- It incorporates a mix of uses that typically include or have the potential to include the following: retail, office, services, and other commercial, civic, recreational, industrial, and residential uses, within a densely developed, compact area;

- It incorporates existing or planned public spaces that promote social interaction, such as public parks, civic buildings (e.g., post office, municipal offices), community gardens, and other formal and informal places to gather.

- It is organized around one or more central places or focal points, such as prominent buildings of civic, cultural, or spiritual significance or a square.

- It results in compact concentrated areas of land development that are served by existing or planned infrastructure and are separated by rural country side or working landscape.

Rural commercial centre in the “Growth Centre” form is one important feature for rural economic sustainability. In growth centre, people can find their necessary service activity in an easy accessible distance apart from the urban area. In this sense the permitted uses in the growth centre are:

##### Table - 4.39: Landuse Permitted in Growth Center

| Accounting, Auditing or Bookkeeping Services | Garages |
| --- | --- |
| Addiction Treatment Center | Garden Center or Retail Nursery |
| Agricultural Chemicals, Pesticides or Fertilizers Shop | General Store |
| Agricultural Dwellings | Grain & Feed Mills |
| Agricultural Product Sales | Grocery Store |
| Agro-Based Industry (Rice Mill, Cold Storage) | High School |
| Agro-Based Industry (Feed mill) | Motel |
| Ambulance Service | Household Appliance and Furniture Repair Service |
| Amusement and Recreation (Indoors) | Housing For Seasonal Firm Labor |
| Ancillary Dwelling | Indoor Amusement Centers |
| Animal Husbandry | Indoor Theatre |
| Antique Store | Jewelry and Silverware Sales |
| Appliance Store | Katcha Bazar |
| Auction Market | Loom (Cottage Industry) |
| Auto Repair Shop (With Garage) | Mosque, Place of Worship |
| Bank & Financial Office | Newspaper Stand |
| Barber Shop | NGO/CBO Office |
| Billboards, Advertisements & Advertising Structure | Nursery School |
| Blacksmith | Open Theater |
| Boarding and Rooming House | Optical Goods Sales |
| Book or Stationery Store or Newsstand | Orphanage |
| Bus Passenger Shelter | Outdoor Fruit and Vegetable Markets |
| Cinema Hall | Outdoor Religious Facility (Eidgah) |
| Cleaning/Laundry Shop | Paints and Varnishes Shop |
| Coffee Shop/Tea Stall | Photo Studio |
| Cold Storage | Photocopying and Duplicating Services |
| College/Technical Training School/Centers/Agro Based Trading | Postal Facilities |
| Communication Service Facilities | Primary School |
| Communication Tower Within Permitted Height | Public Utility Stations & Substations |
| Community Center | Research organization (Agriculture/Fisheries) |
| Computer Maintenance and Repair | Restaurant |
| Computer Sales & Services | Retail Shops/Facilities |
| Confectionery | Rickshaw/Auto Rickshaw Stand |
| Courier Service | Satellite Dish Antenna |
| Craft Workshop | Sawmill |
| Cyber Cafe | Shoe Repair or Shoeshine Shop (Small) |
| Dairy Firming | Signboard/Billboard |
| Daycare Center (Commercial or Nonprofit) | Slaughter House |
| Department Stores | Small Workshop |

| Doctor/Dentist Chamber | Social Forestry |
| --- | --- |
| Dormitory | Specialized School: Dance, Art, Music & Others |
| Drug Store or Pharmacy | Specialized School: Dance, Art, Music, Physically Challenged & Others |
| Electrical and Electronic Equipment and Instruments Sales | Static Transformer Stations |
| Emergency Shelter | Storage & Warehousing |
| Energy Installation | Taxi Stand |
| Fabric Store | Television, Radio or Electronics Repair |
| Family Welfare Center | Theater (Indoor) |
| Fast Food Establishment/Food Kiosk | Transmission Lines |
| Farm Equipment Sales & Service | Truck Stand & Freight Terminal |
| Fish Hatchery | Utility Lines |
| Fitness Centre | Variety Stores |
| Flowers, Nursery Stock | Vehicle, Leasing or Rental Service |
| Freight Transport Facility | Veterinary Centre |
| Fuel and Ice Dealers | Veterinary Clinic/Center |
| Funeral Services | Wood Products |
| Furniture Stores | Woodlot |

Landuse Conditionally Permitted

The following uses may be permitted or disallowed in the zone after review and approval by the authority/committee following appropriate procedure while the application meets the criteria mentioned in the requirement.

##### Table - 4.40: Landuse Conditionally Permitted in Growth Center

| Artisan’s Shop | Library |
| --- | --- |
| Auto Leasing or Rental Office | Motor Vehicle Fuelling Station/Gas Station |
| Beauty Parlor | Musical Instrument Sales or Repair |
| Building Maintenance/Cleaning Services (No Outside Storage) | Pet Store |
| Building Material Sales or Storage (Indoors) | Plantation (Except Narcotic Plant) |
| Carnivals and Fairs | Police Station |
| Circus | Poultry |
| Correctional Institution | Preserved Fruits and Vegetables Facility/Cold Storage |
| Fire Brigade Station | Sports and Recreation Club |
| Freight Handling, Storage & Distribution | Stage Shows |
| Freight Yard | Temporary Rescue Shed |
| Guest House | Trade Shows |

*All of the Commercial Activities shall be located at Growth Centres

##### Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Water Retention Area

Retaining water is the main purpose of this type of Landuse.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.41: Landuse Permitted in Water Retention Area

| Aquatic Recreation Facility (Without Structure) | Utility Lines |
| --- | --- |
| Fishing Club (non-strucural) | Water Parks |

Landuse Conditionally Permitted

The following uses may be permitted or denied in the zone after review and approval by the authority/committee.

##### Table - 4.42: Landuse Conditionally Permitted in Water Retention Area

| Marina/Boating Facility | Water based Recreation |
| --- | --- |

##### Waterbody

Water body serves the natural flow of water along their alignment.

##### Purpose

The zone intends to preserve flow and level of water across all seasons along their natural alignment. DMDP area is covered by a network of natural water body including river, canal and pond. To promote and provide environmentally safe and sound living for the people of the area and for the future generation, water body has an immense importance. Water body should be preserved not only for environmental concern but also for active and passive recreation purposes.

##### Landuse Permitted

The following uses in the table are proposed to be applicable for the zone.

##### Table - 4.43: Landuse Permitted for Waterbody

| Aquatic Recreation Facility (Without Structure) | Utility Lines |
| --- | --- |
| Fishing Club (Non-structural) | Water Parks |

Landuse Conditionally Permitted

The following uses may be permitted or denied in the zone after review and approval by the authority/committee.

##### Table - 4.44: Landuse Conditionally Permitted for Waterbody

| Marina/Boating Facility | Water based Recreation |
| --- | --- |

Restricted Uses

All uses except permitted and conditionally permitted uses.

##### Special Functional Options

Besides above categories of landuse, some situations are not so infrequent in nature. Successful control of development needs specific regulations for handling such functional options too.

##### New Use

Suppose a new electric substation needs to be installed in a residential neighborhood to cover the additional load. The plot in which it is to be installed is of course, earmarked as urban residential landuse. But since this facility is not permitted within the urban residential zone, this electric substation cannot be legally installed in the zone unless the landuse of the plot is altered to allow it. This process of changing the permitted use of a particular plot or land parcel to allow uses not presently permitted in the corresponding landuse zone is termed as New Use. Conversion of the permitted use to accommodate a new function shall require formal permission of the Nagar Unnayan Committee (NUC).

##### Nonconforming Use

Some functions, structures or facilities may have been in the area prior to landuse zone declaration and the facility does not conform to the new landuse zone. Considering the degree of incompatibility or the adverse impact on the surrounding

landuse, such functions, structures or facilities have been termed as non-conforming to the permitted facilities of the zone. Nonconforming use/function, structure or facility may either be legal or illegal depending on the plan permit status but in either case they have to be discontinued in the corresponding zone and may be relocated in the prescribed zone in due course.

##### Basic Nonconforming Elements

Any nonconforming facility is comprised of one or a combination of two or three of basic nonconforming elements in varying combinations, namely, use/function, structure and development over the site. In case of relocation of many nonconforming use/functions, especially foot loose industries such as RMGs and RKGs, packaging industries, small engineering works etc., structure need not to be demolished since it can house zone permitted functions and continue. Few industries and plants such as chemical industries, rerolling mills, brick fields etc., may require demolition of structures or plants that are meant specifically for that facility. Besides these, some nonconforming developments, such as landfill in flood flow, water retention area and agricultural zones requires clearing of landfill materials to restore uninterrupted flow of annual flood water, to maintain design capacity for water retention and to safeguard food security respectively.

##### General Regulatory Provisions

The continued existence of nonconformities is inconsistent with the purposes for which the “zoning” of the area has been worked out and thus, the gradual elimination of such nonconformities is generally desirable.

The following provisions shall apply to all uses/functions, structures and/or developments set forth as nonconforming to the respective zone as per this Zoning Regulation.

- There must be a time frame of up to 5 years within which the non conforming uses have to be relocated to places where those specific uses are permitted according to this regulation. This time frame for relocating a particular non conforming use will be determined by RAJUK subject to approval by the Nagar Unnayan Committee (NUC). However, the time frame may vary depending on that particular use but shall not exceed five years unless decided otherwise by the Nagar Unnayan Committee.

- RAJUK shall formulate strict conditions for nonconforming facilities restricting enlargement, alteration, expansion, restoration of those uses within interim period till its relocation to permitted zone.

- RAJUK shall closely monitor non conforming facilities to guard against non-compliance of the abiding conditions and also to ensure relocation to permitted zone. In case of non compliance in terms of conditions or of relocation to permitted location within set time frame, action to ensure immediate discontinuation or to restrict the facility or to relocate it in the permitted zone shall be initiated by RAJUK.

- To expedite relocation of nonconforming facilities to permitted zones, imposing higher tax for continuation in the present location as opposed to incentives in the form of subsidy, tax holiday etc. if relocated to the prescribed zone may be actively considered by the government.

##### Identification and Registration of Non-conforming Uses and Structures

All uses and structures existing at the effective date of this Regulation which do not conform to the requirements set forth in this Regulation, or any amendments thereto, should be identified and registered by RAJUK. Failure or negligence of the authority to identify or register uses or structures which are non-conforming as defined in this Regulation or subsequent amendments does not exempt such use or structures from the applications of this Regulation.

##### Continuation of Non-conforming Uses and Structures

Any non-conforming use or structure may be continued up to their time frame, provided such use or structure:

- Shall not be moved to another location where such use or structure would be non-conforming.

- Shall not be changed to another non-conforming use or structure without approval of NUC and then only to a use or structure which, in the opinion of the NUC is of the same or of a more restricted nature in accordance with the restrictions above.

- Shall not be re-established if such use or structure has been discontinued for any reason for a period of twelve

(12) months or more, or has been changed to, or replaced by, a conforming use or structure. Intent to resume a non-conforming use shall not confer the right to do so.

- Shall not be restored for other than a conforming use after damage from any cause, unless the non-conforming use or structure is reinstated within twelve (12) months of such damage; if the restoration of such structure is not completed within the said twelve (12) month period, the non-conforming use of such structure shall be deemed to have been discontinued, unless such non-conforming use is carried on without interruption in the undamaged portion of such structure.

##### Maintenance

Maintenance of a building or other structure containing or used for a non-conforming use will be permitted when it includes necessary non-structural repairs and incidental alterations which do not extend or intensify the non-conforming building or use.

##### Reconstruction

A non-conforming use which has been damaged by fire, explosion, or an act of God, to the extent of sixty (60%) percent of its reproduction value at the time of damage shall be restored in conformity with the current regulations of the zone in which it is located. When damaged by less than sixty (60%) percent of its reproduction value a non-conforming use may be repaired or reconstructed and used as before the time of damage, provided such repairs or reconstruction are started within one (1) year from the date of such damage.

##### Discontinuance or Abandonment

A non-conforming structure occupied by a non-conforming use may be re-occupied by a conforming use following relocation of the nonconforming use. If a non-conforming use is abandoned for a period of two (2) years, any future use of the premises shall be in conformity with the provisions of this Regulation and shall not again be used except in conformity with the current regulations of the zone in which it is located.

##### Unsafe Structure

Any non-conforming structure or portion thereof containing a non-conforming use is allowed to undergo routine and ordinary maintenance, repairs or replacement of non-load bearing walls, fixtures, wiring, or plumbing, provided that the floor area and floor height shall not be increased. However, only in case the structure/s or a portion thereof declared to be unsafe by any official in charge of protecting public safety, strengthening or restoring of any building or part thereof to make it safe upon order of such official shall be allowed. But in such cases it will require approval of the Nagar Unnayan Committee.

##### Termination and Removal of Non-Conforming uses, Structures or Developments

Uses, structures or developments of land listed as non-conforming shall be brought into compliance with the Zoning Regulation or be completely removed from the premises on or before the expiry within maximum five year period.

##### Principal Use and Accessory Use

Use of any facility may be categorized under two broad heads: Principal Use and Accessory Use. Any use may be principal in one zone while accessory in another or vice versa. Also, same use may simultaneously be principal and accessory in the same zone based on the nature of facility that house it. Principal Use indicates functions which the facility is primarily intended for. The Academic Building in a school is definitely its principal use. But an entire primary school would fall under accessory use category when it is located in the university complex to facilitate children of the teachers and officials working there. Ancillary functions of any facility are grouped as accessory use. A garage of a residence is an accessory use. But multi storied commercial parking in motijheel is definitely its primary function.

##### Development Permit

Development Permit is the most important function of RAJUK and for that matter of any City Development Authority. DAP will have no bearing unless development can be channelized to its desired direction through effective permit procedure. DAP has developed its plan using GIS database and other advanced computer software of world standard. Urban Planners to make use of the database is now available in the country. This combination provides RAJUK the unique opportunity to make its plan permit procedure fast, well managed and transparent. This is also in line with the idea of digital Bangladesh.

##### Computerization of the Permit procedure

Maintaining information of all the development activities within the DMDP jurisdiction is a mammoth task and maintain them in the present manual method is neither possible nor necessary. Consultant recommends development of customize software for the purpose. Necessary software experts are available locally. The system would prove worthwhile by saving in the form of time, cost, ease of management, ease of upgrading of information, control of corruption and so forth.

##### Landuse Permit

RAJUK has the legal responsibility to develop plan for the well-being of the citizens within its jurisdiction and implement the same by channelizing all developments through appropriate control mechanisms. Issuance of Plan Permit to private plot owner/s or developers working with the consent of the owner/s that comply the set regulations constitute the most part of development control activity conducted by RAJUK. In the following paragraphs the structures of the proposed Landuse Control Authority has been elaborated considering that RAJUK shall be strengthened adequately and in that situation Planners’ ranks would be as under [in light of the Sthapatya Adhidaptar]:

- Chief Town Planner

- Additional Chief Town Planner

- Deputy Chief Town Planner

- Assistant Chief Town Planner

- Assistant Town Planner

##### Structure of Landuse Permit Authority

The Landuse Permit Authority shall be comprised of three vertically linked tiers:

- At the entry level Landuse Permit Planner [LPP]

- At the mid level Landuse Permit Committee [LPC] to control LPPs, clarify legal provisions regarding landuse permit decisions on a case to case basis, and

- At the top level Nagar Unnayan Committee comprised of representatives from planning departments, professional institutions, eminent scholars and citizens of the city.

Figure No. 4.1 shows the tiers along with their inter links.

Landuse Permit Planner

Landuse Permit will be issued with the sign of Landuse Permit Planner [LPP] appointed by the Chairman, RAJUK from among the Planners not below the rank of Assistant Town Planner. Landuse Permit issued by the LPP/s shall be null and void, even it signed, unless the use sought for is permitted in the landuse options of the respective zone that contains the plot.

To cover the vast DMDP area under RAJUK jurisdiction, Paurashava Planners [PP] working in the Paurashavas within the DMDP area, in addition to RAJUK’s main stream LPPs, may be delegated with the power to act as LPP and issue Plan Permit and control development within the provision of DAP on RAJUK’s behalf. He/she must have needful Inspectors and GIS facility with logistics and knowledgeable personnel to operate so as to accomplish such responsible job. For all plan permit activities PPs shall be accountable to, controlled by and act in close communication with Landuse Permit Committee at RAJUK.

Landuse Permit Committee

At the mid level Landuse Permit Committee [LPC] shall function for effective control of LPPs [both main stream and Paurashava] and to clarify legal provisions regarding landuse permit decisions on a case to case basis. LPC shall be a maximum seven member committee headed by RAJUK’s Chief Town Planner [CTP]. LPC Members shall be not below the rank of Assistant Chief Town Planner while Deputy Chief Town Planner, Development Control shall act as the Member Secretary of this Committee. The activities of LPC will include

- Clarify the legal provisions for the LPPs as per their request.

- Make recommendations in case of New Use or Conditional Use and send it to the Nagar Unnayan Committee [NUC] for decision.

- Earmark plot numbers under non-conforming uses and notify the owners about the time span to relocate the facility, procedure and conditions that must be strictly maintained to avoid immediate eviction.

To Court

Applicant

Landuse Permit Planner

Landuse Permit Committee

Nagar Unnayan Committee

##### Figure - 4.1: Structure of Landuse Permit Authority Showing Linkages

Nagar Unnayan Committee

Nagar Unnayan Committee [NUC] shall be the supreme authority regarding Landuse Permit within RAJUK jurisdiction. The Members of the Nagar Unnayan Committee are (not as per the protocol):

- Head	: Secretary, Ministry of Housing and Public Works, GoB

- Members	: Chairman, RAJUK

: Head, Department of URP, BUET

: President, BIP

: President, IAB

: Chief Town Planner, Dhaka City Corporation

: Mayors, Paurashavas [within RAJUK jurisdiction]

: Chief of Concerned Line Agencies

: Three Eminent citizens of Dhaka

- Member Secretary	: Chief Town Planner, RAJUK

NUC shall be well supported by a secretariat and shall be empowered both authoritatively and financially to carry out study and/or survey, arrange public hearing, round table conference, seminar, or if necessary, engage experts.

NUC shall decide whether the proposed New Uses should be permitted or denied; in case of Conditional Permit impose the conditions to comply; accept variances for specific cases and so forth. Besides, NUC shall decide strict conditions to nonconforming uses and the action against its violation if so happens.

##### Landuse Permit Option

For a plot seeking landuse permit there can be three possible options:

- Landuse permitted

- Landuse conditionally permitted, or

- Landuse restricted

Landuse Permitted

Landuse that unconditionally permitted in the zone are listed in this category. When permission is sought for a residential landuse on a plot earmarked as urban residential zone then it falls under this category.

Landuse Permitted with Condition

Landuse that generally not incompatible or harmful for the community but whose number, location or specific use nature may pose threat to community’s life style, privacy, safety or security etc. then the landuse is permitted but with a condition to fulfill so that the potential threat is avoided. For example, in a Residential-General Industrial Mixed use zone a request is made seeking landuse permit for a composite textile mill with a dyeing unit. Since the use is compatible in the zone except for the release of noxious effluent to the surrounding, the permit may be issued with a condition to exclude the dyeing unit in order to get landuse permit. Now, following the formal agreement by the applicant to comply the condition the permit is issued against the plot. A list of such conditional uses is maintained in this category.

Landuse Restricted

Landuse that is harmful for the community are restricted by law. Such harmful landuse is listed in this category. A cinema hall in a neighborhood may be cited as an example under this category. But for convenience, any use not listed in the permitted and conditionally permitted use category is considered as restricted for the zone.

##### Landuse Permit Procedures

Landuse permit procedure is a product of a number of interlinking activities. The whole process has been shown in a flow diagram for clearer understanding in Figure 4.2.

The procedure is commenced with the submission of formal application by the applicant to the Chairman of RAJUK. The applicant must submit along with other information and documents a mauza map showing his plot including plot no, mauza name etc. The concerned official designated as Landuse Permit Planner (LPP), will then check the compliance of the landuse desired by the applicant with the landuse zone containing his plot and the uses permitted therein.

Four situations may be possible:

- Desired Use is listed as Permitted in the zone

- Desired Use overlaid

- Desired Use is listed as Conditionally Permitted in the zone, and

- Desired is not listed under any of the categories and may be permitted as New Use

Formally Applied

Checked for

Landuse Zone

The Process ends finally

Appealed

to Higher Court

Verdict in Against the Plaintiff

Landuse

Zone Complied

Landuse

Zone Not Complied

Appealed to Nagar Unnayan Committee

Proposed Landuse Overlayed

Appealed to Court

Verdict in favor of Plaintiff

Checked for Permitted Functions

Landuse

Rejected with Reason

No more Applied

Proposed Landuse Permitted

Proposed Landuse need further Evaluation

New Landuse

Verdict to correct and reapply

Returned with Data and/or Information

Seek Feedback Data

and/or Information

Evaluated in the Landuse Permit Committee

Proposed Landuse permitted

Proposed Landuse Rejected and Returned with Reason

Proposed Landuse permitted with abiding conditions to meet

Returned with Decision

Referred to Nagar Unnayan Committee

Proposed Landuse Permitted

Nagar Unnayan Committee

Appealed to Nagar Unnayan

Committee

Decision in favor of Applicant

No more Applied

Decision not accepted by

the Applicant

Condition Accepted and formally agreed by the Applicant

Landuse permitted with abiding conditions to meet

The Process ends finally

Decision Against the Applicant

The Process

Decision to correct and re-apply

ends finally

Corrected and reapplied

##### Figure - 4.2: Flow Diagram Showing Activity Linkage of Land Permit Procedure

If the desired use is listed as Permitted or Overlaid then it will be permitted without any question. If the desired use is listed as Conditionally Permitted the LPP would refer it to the LPC for further action.

In case of desired use not found in the permitted or conditionally permitted lists of the zone, the LPP shall reject the desired landuse as it is not allowed in the zone. At this stage if the rejection decision taken by the LPP is not satisfactory to the applicant, he/she can appeal to the NUC. If the decision of the NUC goes in favor of the applicant, LPP shall then issue the permit. NUC may also ask the applicant to make some modifications to make his/her claim appropriate for approval. The applicant may comply accordingly and apply afresh.

If the applicant is not satisfied with the decision of the NUC he/she may go to the court for decision.

If the LPP is convinced that the desired use should be allowed for the greater interest of the people and therefore deserves to be considered under New Use category, he may recommend it to the LPC furnishing reasons in favour. The LPC if convinced by the reasoning will send the case to the NUC with recommendation to permit desired landuse in the New Use category. Following necessary study and investigation if the NUC is also convinced about permitting the use as recommended by the LPC, they may decide so and authorize the LPP to issue permit for the desired landuse in the New Use category.

##### Field Level Vigilance

Development cannot be channelized as per the plan unless an effective control at the field level is achieved. Due to unhealthy concentration of political, administrative, financial and even judicial power in the capital city, almost entire in- migrant population from all over the country rush to Dhaka to try their luck. This is the root cause of all the problems Dhaka has been and will be facing. Land value and rent of space within the city soaring sky high exerting tremendous development pressure that tempts opportunists to fill out low lying areas completely disregarding its legality provisions set by the plan. Land classified in the plan as agriculture, flood flow or retention pond is indiscriminately filled out for making plots every day. Such environmentally catastrophic activities must be guarded to save the city through effective Field level vigilance. And as a custodian of DAP the responsibility falls on to RAJUK’s shoulder.

Another kind of damage to plan takes place at the implementation level in the form of illegal structures, unauthorized structures, deviation from the approved plan and so forth. These too need close vigilance from RAJUK’s end.

Vigilance is very vital for identification of overlay and non-conforming structures and to ensure their compliance to the plan permitted use.

Consultants recommend a large, strong, committed and effective department with appropriate personnel and logistics to guard against all such evil activities.

##### Interaction with People

Plans and Reports furnish various information that has direct bearing on the people since they need to follow the guidelines provided in these documents while developing their parcel of land/s. Presently RAJUK makes it difficult for the common people to collect information that are necessary for various development ventures on their land parcel/s. This hiding attitude makes way for unplanned developments and people feel reluctant to come to RAJUK for plan permit. But in reality availability of information has a vital role to play towards effective plan implementation.

Planning Department of RAJUK should have close interaction with the citizen of Dhaka in order to make people aware of the benefits of a good plan which in turn steer people’s social responsibility to promote plan implementation and resist contraventions. A specific Interactive Cell is recommended to be put to operation in this respect with following objectives:

- Listen to people informing about contraventions, deviations to approved plans, damage to planned developments, obstruction to flow of water or filling out of retention areas for personal or group benefits and

inform them about the remedial measures taken and thereby encouraging inclusion of people in guarding the implementation of the plan.

- Apprise the citizens about the planning and landuse management related legislation and zoning regulations.

- Investigate and report to the higher authority about complaints made by the citizens about plan violation, violation of landuse of a zone and miscellaneous complaints.

- Provide pre-application advice to residents, consultants and developers about landuse management issues and application procedures for the submission of development applications.

- Interact with communities, especially spontaneously developed ones, lacking specific needs and provide them with proper planning guidance to overcome them through collective efforts.

Such interactive windows may be opened at convenient locations, thus, making of services available at the door steps of the citizens. Answers to frequently asked questions may be provided in the internet. Besides, these may be publicized in the print and electronic media time to time.

RAJUK should immediately open up in all possible ways to reach people and earn their confidence.