## Chapter- 3 DEVELOPMENT PLAN PROPOSALS

### Abiding Policy Frameworks of Higher Level Plans

The Project Area of Group-B covers seven Strategic Planning Zones (SPZ), six fully and one partially, each having diversified characteristics. DMDP Structure Plan and Urban Area Plan provide different policies for their development depending on the nature and character of the area. For the planning purposes, these SPZs are again regrouped into Detailed Planning Zones (DPZ). The regrouping has been done according to the local condition considering their contiguousness. As such, all the Detailed Planning Zones are continuous, separated from each other either by natural barriers or administrative boundaries.

This section deals with the regrouped areas to see the policies that have been adopted by higher level plans as policy guidelines for development control before the preparation of Detailed Area Plans for that particular area. The proposed Detailed Planning Zones are:

DPZ-01 Shyampur-Dania-Matuail

DPZ-02 Kutubpur-Fatullah-Enayet Nagar DPZ-03 Siddhirganj Paurashava

DPZ-04 Baktabali-Alirtek

DPZ-05 Narayanganj Paurashava and Adjoining Areas DPZ-06 Kadam Rasul Paurashava

DPZ-07 Bandar-Sonargaon

##### DPZ-01 Shyampur-Dania-Matuail

Detailed Planning Zone-01 consists of three unions namely: Shyampur, Dania and Matuail. It extends from Sitalakhya river in the east to Buriganga river in the west. In the north, it is bounded by the Jatrabari-Demra road and parts of Kutubpur union and the Siddhirganj Paurashava are located in the south. It is one of the fastest growing areas of the DND and houses several important establishments like Dhaka Central Sewage Treatment Plant, RAJUK Shyampur Industrial Park and parts of Adamjee-Demra Industrial zone and the intake canal for Syedabad Water Treatment Plant. A part of the National Highway No.1 Dhaka-Chittagang road passes through this zone. Regional roads like Dhaka-Demra road, Old Narayanganj road and Demra-Adamjee road serve the area. This fast growing area is characterized by sprawl development, poor access facilities and inadequate drainage provision. There are pockets of undeveloped land in the central and the south-western part. Development pressure is high along the major arterial roads as mentioned above.

##### Relevant Structure Plan Policies

- Rural and Special Area Policy RS/5 – Flood Retention Ponds control will be maintained over the areas in order to ensure that they remain capable of fulfilling their primary function of water storage at times of flooding.

- Urban Area Policy UA/6 for New Urban Land growth promotion seeks to initiate and coordinate a range of measures aimed at stimulating and promoting the rate of development in the designated areas of the urban fringe.

- Urban Area Policy UA/7 Infrastructure Initiatives seek to promote, through the DMDP Structure Plan, an orderly sequence of new area development by means of mutually reinforcing and coordinated public sector investment programs, spearheaded by drainage, flood protection and transport development.

- Sectoral Policy SE/4 advocates for an integrated policy of the incremental environmental upgrading and relocation, where necessary, of Dhaka’s polluting industries, in a manner commensurate with sound environmental practice and cost-effectiveness.

- Infrastructure Policy IN/2 promotes for incremental network development in the transport sector in order to conserve resources and being responsive to proven demand for the service being offered.

##### Urban Area Plan Recommendations

- Adequate control should be imposed on the intake canals of Sayedabad Water Treatment Plant.

- Sitalakhya river front should be provided with incentives for industrial development.

- Adequate measures to ensure the realization of Retention Ponds should be undertaken.

- FAP-8A project will provide drainage facilities and retention ponds in the area making the area free from internal flooding.

- Detailed Area Plan to steer development in the area is required.

- A study should be undertaken to provide the slum population with minimum basic services.

##### DPZ-02 Kutubpur-Fatullah-Enayet Nagar

Kutubpur, Fatullah and Enayet nagar unions constitute the Detailed Planning Zone no.02. Old Dhaka-Narayanganj road and the Buriganga river are the important elements of the zone. Shyampur and Matuail unions of DPZ-01 are located in the north, in the south it is bordered with Kashipur union, in the east Narayanganj and Siddhiganj paurashavas are its border while Buriganga river makes its western border. Settlements have mostly developed along the Old Dhaka- Narayanganj road. There are several pockets of industrial agglomeration in the zone. A very large section of river bank is serving as loading, unloading and processing centre of construction materials. BIWTA maintains a jetty for VIPs and two large Oil Depots are located here. Old Dhaka-Narayanganj road provides connection with Munshiganj district through Panchabati-Mukterpur road. Dhaka-Narayanganj rail link is through this zone with a station at Fatullah and a flag station at Pagla.

##### Relevant Structure Plan Policies

- Rural and Special Area Policy RS/5 – Flood Retention Ponds control will be maintained over the areas in order to ensure that they remain capable of fulfilling their primary function of water storage at times of flooding.

- Urban Area Policy UA/6 for New Urban Land growth promotion seeks to initiate and coordinate a range of measures aimed at stimulating and promoting the rate of development in the designated areas of the urban fringe.

- Urban Area Policy UA/7 Infrastructure Initiatives seek to promote, through the DMDP Structure Plan, an orderly sequence of new area development by means of mutually reinforcing and coordinated public sector investment programs, spearheaded by drainage, flood protection and transport development.

- Sectoral Policy SE/4 advocates for an integrated policy of the incremental environmental upgrading and relocation, where necessary, polluting industries of Dhaka’s, in a manner commensurate with sound environmental practice and cost-effectiveness.

- Infrastructure Policy IN/2 promotes for incremental network development in the transport sector in order to conserve resources and responsive to proven demand for the service being offered.

##### Urban Area Plan Recommendations

- A full study of the Pagla Fault line which passes through this zone is required to ascertain the measures that are required in the development work of the zone.

- Projects to increase the capacity of Sewerage Treatment Plant should be undertaken.

- RAJUK developed Industrial Area in Kadamtali should be steered to achieve full utilization in order to provide economic base to nearby people.

- Adequate measures to ensure the realization of Retention Ponds should be undertaken.

- FAP-8A project will provide drainage facilities and retention ponds in the area making the area free from internal flooding.

- Detailed Area Plan to steer development in the area required.

- A study should be undertaken to provide the slum population with minimum basic services.

- Given the facilities and opportunities in this zone, study should be made to relocating the Inland Container Depot in this zone.

##### DPZ-03 Siddhirganj Paurashava

Siddhirganj Paurashava was created in the year 1995 to provide municipal umbrella to the industrial area along the west bank of the Sitalakhya river stretching from Godenail (northern end of Narayanganj Paurashava ) to Demra in the north and west of Demra–Adamjee road almost up to DND Spine Road covering Siddhirganj, Sumilpara and Godenail unions. The area is fast growing characterized by unplanned and sprawl development. The eastern part of the Paurashava along the west bank of Sitalakhya river is dominated by industrial use. The DND portion of the Paurashava is mostly residential and with patches of vacant land either used for cultivation or remaining fallow. The major elements of the Paurashava are Adamjee EPZ, Siddhiganj Power Station, the Grain Silo, the Pump House, DND Canals (a portion of the DND canal is serving as intake channel of Sayedabad Water Treatment Plant) etc.

##### Relevant Structure Plan Policies

- Urban Area Policy UA/3 emphasizes community led initiatives through the participation in the land development process.

- Sectoral Plans, Policies and Proposals Policy SE/1 seeks to designate Narayanganj area as a Special Rehabilitation Incentive Zone to revitalize the manufacturing sector to recover from decline of the jute sector

- Sectoral Policy SE/4 advocates for an integrated policy of the incremental environmental upgrading and relocation, where necessary, polluting industries of Dhaka, in a manner commensurate with sound environmental practice and cost-effectiveness.

- Infrastructure Policy IN/2 promotes for incremental network development in the transport sector in order to conserve resources and being responsive to proven demand for the service being offered.

- Infrastructure Policy IN/5 promotes incremental flood protection strategy to employ means of implementation of components of FAP-8A which will spread costs over time and capture benefits for other development sectors.

- Rural and Special Area Policy RS/4 envisages river pollution control for environmental protection to prevent pollution of the Sitalakhya river and its tributary, the Balu River, in order to ensure that it remains viable long-term source of potable water.

- Policy RS/5 proposes flood retention ponds for retaining water at times of flooding. The Structure Plan states that ponds have to be maintained primarily by landuse controls since acquisition of retention pond areas is unfeasible.

##### Urban Area Plan Recommendations

- On completion of DAP, the embargo on development in this area should be lifted to ensure planned development of the Zone.

- Sitalakhya river front should be provided with incentive for industrial development.

- More projects to enable low income people to get hold of serviced plots should be taken.

- Land Readjustment Projects to realize planning standards in the established areas should be taken.

- A new municipality or a specific DND Triangle Development Board should be undertaken.

##### DPZ-04 Baktabali-Alirtek

DPZ-04 is constituted with Baktabali union and Alirtek union. It makes the south and south-western boundary of RAJUK jurisdiction. The northern boundary is made by Enayet Nagar union and the Dhaleshwari river flows through the eastern boundary. The zone is low lying and part of flood flow zone. It is accessible through Old Dhaka-Narayanganj road and Mukterpur road.

##### Relevant Structure Plan Policies

- Structure Plan Policy RS/3 seeks to protect flood flow zones from land development in order to avoid obstructions to flood flow, which otherwise result, in adverse hydraulic effects, such as, the rise of flood water levels and changes in flood direction.

- Structure Plan Policy RS/6- Special Areas seeks to respect the integrity of the functions of the Special Area designations and maintain the land areas of their respective domain free from urban incursions.

##### Urban Area Plan Recommendations

- For providing improved access, various khals in the zone may be re-excavated which may then also be used for retaining irrigation water.

- Measures should be taken to maintain the flood plain free from development.

##### DPZ-05 Narayanganj Paurashava Area and Adjoining Areas

DPZ-05 encompasses Narayanganj Paurashava and its immediate surroundings, is the oldest urban area of the Group-B. The paurashava part of the zone is an important center of trade, commerce and industries. At the time of creation, Narayanganthis paurashava covered areas of both the banks of the Sitalakhya river. In the year 1993 the east bank area was separated from it and a new paurashava was created. The other areas included in the zone are Fatullah and Enayet Nagar unions in parts and Kashipur and Gognagar unions in full. The paurashava part is more or less developed. But development has taken place haphazardly resulting in ill-drawn and inadequate circulation system where paucity of urban services is a common factor.

##### Relevant Structure Plan Policies

Narayanganj Paurashava and its adjoining areas are composed of established urban areas and urbanizing areas. The Structure Plan policies for the areas are:

- Urban Area Policy UA/1 seek to optimize land resources within the defined established urban area by encouraging the infilling of vacant, under-utilized land by allowing vertical development up to four-six floors by redevelopment or re-subdivision of land within lower density communities.

- Urban Area Policy UA/2 promotes densification at community level accompanied by infrastructure consolidation.

- Urban Area Policy UA/3 emphasizes community led initiatives through participation in the land development process.

- Under Sectoral Plans, Policies and Proposals Policy SE/1 seeks to designate Narayanganj area as a Special Rehabilitation Incentive Zone to revitalize the manufacturing sector to recover from decline of the jute sector.

- Sectoral Policy SE/4 advocates for an integrated policy of the incremental environmental upgrading and relocation, where necessary, of Dhaka’s polluting industries, in a manner commensurate with sound environmental practice and cost-effectiveness.

- Sectoral Policy SE/5 seeks to promote the gradual dispersal of commercial activity to the existing suburbs and new growth areas.

- Sectoral Policy SE/10 envisages augmentation of the city’s existing stock of major recreational facilities by means of exploiting the resource of vacant and/or under-utilized government land within established urban area.

- Sectoral Policy SE/11 intends to identify and secure sites for major recreational use in all DMDP priority areas.

- Infrastructure Policy IN/2 promotes for incremental network development in the transport sector in order to conserve resources and responsive to proven demand for the service being offered.

- Infrastructure Policy IN/5 promotes incremental flood protection strategy to employ means of implementation of components of FAP-8A which will spread costs over time and capture benefits for other development sectors.

- Rural and Special Area Policy RS/4 envisages river pollution control for environmental protection to prevent pollution of Sitalakhya river and its tributary, Balu river, in order to ensure that it remains viable, long-term source of potable water.

- Policy RS/5 proposes flood retention ponds for retaining water at times of flooding. The Structure Plan states that ponds have to be maintained primarily by landuse controls since acquisition of retention ponds areas is unfeasible.

##### Urban Area Plan Recommendations

- Proposed additional access facilities with Dhaka should be realized as early as possible.

- Study to improve drainage facilities has to be taken up on priority basis.

- Telecommunication facilities of international standard have to be developed.

- To improve circulation within the city, a study for traffic management should be undertaken and efforts should be made to implement the recommendations with priority.

- Container handling capacity should be installed in the river port.

- BSCIC sponsored and on-going work of Hosiery Estate should be expedited for providing supplementary facilities to the growing export-oriented Knit Garment Industries.

- For the promotion of tourism Fort and its adjacent areas of historical importance should be developed. Fort area with river front provides excellent opportunity for development of a recreation centre.

##### DPZ-6 Kadam Rasul Paurashava

Kadam Rasul Paurshava area is also an old and important urban center. In fact, before 1993 it was a part of Narayanganj paurashava. The areas under Narayanganj paurashava in the east bank of the Sitalakhya river has been renamed as Kadam Rasul Paurashava. In the process, some of the areas of the old paurshava have been left out and new areas included. Thus the existing Kadam Rasul Paurshava covers areas along the eastern bank of the Sitalakhya river extending from Madanpur in the north to Madanganj in the south and almost the entire length of land between the old railway tract and the Sitalakhya river. There are thick patches of low-lying vacant lands in between settlements. Development pressure is slow and the area is being developed as residential areas for middle and low income people. With the decline of Jute related activities, the economy of the area has substantially declined. Here it may be mentioned that the Bawa Jute Mills, the first Jute Mills of the country was set up here which has long been closed and machineries scrapped. The Sonakanda Dockyard, the Institute of Marine Technology, the Haripur Power Station and some other textile and jute mills are important elements of the paurashava.

##### Relevant Structure Plan Policies

- Urban Area Policy UA/1 seeks to optimize land resources within the defined established urban area by encouraging the infilling of vacant, under utilized land by allowing vertical development up to four-six floors by redevelopment or re-subdivision of land within lower density communities.

- Urban Area Policy UA/7 Infrastructure Initiatives seek to promote an orderly sequence of new area development by means of mutually reinforcing and coordinated public sector investment programs, spearheaded by drainage, flood protection and transport development.

- Rural and Special Area Policy RS/4- Environmental protection measures will be taken to prevent pollution of the Sitalakhya river and its tributary, the Balu river, in order to ensure that it remains a viable, long-term source of potable water for Dhaka city.

- Under Sectoral Plans, Policies and Proposals Policy SE/1 seeks to designate Narayanganj area as a Special Rehabilitation Incentive Zone to revitalize the manufacturing sector to recover from decline of the jute sector.

- Sectoral Policy SE/4 advocates for an integrated policy of the incremental environmental upgrading and relocation, where necessary, polluting industries of Dhaka, in a manner commensurate with sound environmental practice and cost-effectiveness.

- Infrastructure Policy IN/2 promotes for incremental network development in the transport sector in order to conserve resources and being responsive to proven demand for the service being offered.

##### Urban Area Plan Recommendations

- Feasibility study of a second bridge over river Sitalakhya to provide direct road connection with Narayanganj should be undertaken.

- As proposed in the Structure Plan, the zone should be given the economic incentives zone status.

- To create job facilities, the communication facilities should be developed to help rehabilitate trade, commerce and industry. As one of the measures, construction of the road on the abandoned railway track should be completed on priority basis.

- Services like electricity, water, gas etc and drainage facilities should be brought to adequate scale and service level.

##### DPZ-07 Bandar-Sonargaon

Detailed Planning Zone-07 is constituted with Bandar, Kalagachia, Musapur, Madanpur, Dhamgar, Mograpara, Sadipur, Aminpur Pirizpur and Shambhupura unions. The area is generally rural characterized by scattered rural homesteads. Sporadic industrial activities are found along the Dhaka-Chittagong Highway. The Langalbandh area on the bank of Old Brahmaputra river is an important center of pilgrimage for the Hindu community.

##### Relevant Structure Plan Policies

- Sectoral Policy SE/4 advocates for an integrated policy of the incremental environmental upgrading and relocation, where necessary, of Dhaka’s polluting industries, in a manner commensurate with sound environmental practice and cost-effectiveness.

##### Urban Area Plan Recommendations

- Steer development on Dhaka-Chittagong Highway so as to maintain the integrity of highway’s function.

### Planning Principles and Standards

##### Guiding Principles

Proposed landuses for the area has been prepared on the basis of the following principles:

- Environment friendly sustainable development of the area.

- City function to develop as per major landuse zones.

- Effective drainage through minimum hindrance to Flood Flow zones.

- Safe residential areas at proximity to place of work or major communication routes.

- Smooth and effective functioning of industries, especially export oriented industries.

- Safe yet faster connectivity.

- Develop to serve the surrounding hinterlands.

##### Planning Standards

Many standards and codes for guiding and controlling development are found in Bangladesh National Building Code (BNBC), 2006, Dhaka Mahanagar Imarat Nirman Bidhimala 2008 etc. For ascertaining minimum space requirement for different physical features and functions Besarkari Abashik Prokolper Bhumi Unnayan Bidhimala 2004 and for density control of an area ‘Floor Area Ratio [FAR]’ are the two very important legal instruments.

DMDP Structure Plan and Urban Area Plan too have fixed minimum standards for certain facilities. In today’s reality of congested unhygienic laissez-faire construction race where planning is a far cry, land is obviously the most scarce and hence most valuable asset.

##### Facility Standard

Planned development ensuring community’s active participation is the key to successful transformation of today’s Dhaka into tomorrow’s adorned green Dhaka. Keeping this vision in mind, the Consultants developed an optimum standard for the amenities and community facilities that the city dwellers deserve. Table No. 3.1 shows standard population served with corresponding area requirement for educational institutions as these data is frequently needed for physical planning decisions. Double shift for Primary School is discouraged due to odd timing hampering healthy grooming of the children. However, school of performing or fine arts for the children to complement the learning process is recommended in the school premise in the second half. Double shift for Colleges having Honours and Masters programme is also discouraged

for making way to carryout library work, sessionals/practicals and other extra curricular activities by the students. Data on university has not been included in this time saver chart as the catchment area of university extends far beyond the region it belongs.

##### Table - 3.1: Standard Population Served and Area Required per Educational Institute

| Facility |  | Population/Facility |  | Area (Acre) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Single Shift | Double Shift | Minimum | Optimum |
| Primary School |  | 5000 | 10000 | 1 | 1.5 |
| High School | Std.VI- Std. X | 12000 | 24000 | 1.5 | 2 |
|  | Std.VI- Std. XII | 16000 | 32000 | 3 | 4 |
| College | Std.XI- Std. XII | 12500 | 25000 | 3 | 5 |
|  | Std.XI- Upto Hons./Masters | 22000 | 44000 | 4 | 6 |

Note: Double shift for Primary School and college with Hons./Masters is discouraged.

Neighbourhood concept of residential development is recommended in the DAP as strategy. So, the facilities required for a neighbourhood development deserves special mention (Table No. 3.2).

##### Table - 3.2: Facility Standard at Neighbourhood Level or for 12500 People

| Sl. No. | Name of the Facility | Quantity |  | Area |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Min. | Max. | Minimum for Unit Facility | Sub Class Total | Class Total |
|  |  | (No.) | (No.) |  |  | (Acre) |
| 1 | Primary School (Public or private) | 2 | 3 | 1 Acre |  | 3 |
| 2 | High School (Public or private) | 1 | 2 | 1.5 Acre |  | 3 |
| 3 | Open space |  |  | 10 Acre |  | 12 |
|  | i)Park/children’s park | 1 | 2 | 0.3 Acre | 1 Acre |  |
|  | ii)Water body/Canal/Pond | As per Planner |  | 1.5 Acre | 6 Acre |  |
|  | iii)Play field | 2 | 3 | 1 Acre | 3 Acre |  |
|  | iv) Green/Vegetation/Water Front | As per Planner |  | 0.5 Acre | 2 Acre |  |
| 4 | Mosque and Maktab/ Worship Places | 2 | 3 | 0.2 Acre |  | 0.6 |
| 5 | Library (central) | 1 | 1 | 0.1 Acre |  | 0.2 |
| 6 | Services |  |  | 0.3 Acre |  | 0.5 |
|  | i)Dentist/Doctor's Chamber | 2 | 3 | 40 sq.m | 120 sq.m |  |
|  | ii) Beauty Parlour | 1 | 2 | 50 sq.m | 100 sq.m |  |
|  | iii) Laundry | 2 | 3 | 16 sq.m | 50 sq.m |  |
|  | iv) Hair Dresser | 2 | 3 | 12 sq.m | 40 sq.m |  |
|  | v) Cyber Cafe/Internet service provider | 1 | 2 | 50 sq.m | 100 sq.m |  |
|  | vi) Photocopy/mobile/land phone/fax | 2 | 2 | 12 sq.m | 40 sq.m |  |
|  | vii) Computer based (word processing, printing etc) services | 1 | 1 | 30 sq.m | 30 sq.m |  |
|  | viii) Motor bike Repair, vulcanising etc.(optional) | 1 | 1 | 50 sq.m | 50 sq.m |  |
|  | ix) NMT repair service (Rickshaw, bicycle etc) | 1 | 2 | 30 sq.m | 60 sq.m |  |
|  | x) Post Office/Courier Services | 1 | 2 | 20 sq.m | 40 sq.m |  |

| Sl. No. | Name of the Facility | Quantity |  | Area |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Min. | Max. | Minimum for Unit Facility | Sub Class Total | Class Total |
|  |  | (No.) | (No.) |  |  | (Acre) |
|  | xi) Sports/Recreational facilities (games, indoor games etc) | 1 | 2 | 50 sq.m | 100 sq.m |  |
|  | xii) Rickshaw/Auto stand (General) | 2 | 4 | 100 sq.m | 400 sq.m |  |
|  | xiii) Restaurant, Tea bar, Fast food | 2 | 4 | 10 sq.m | 100 sq.m |  |
|  | xiv) Tailoring | 1 | 2 | 20 sq.m | 40 sq.m |  |
| 7 | Solid waste transfer station(may also small scale processing) | 1 | 1 | 0.5 Acre |  | 1 |
| 8 | Utility Facilities |  |  |  |  | 1* |
| 9 | Neighborhood Co-operative Office Complex |  |  | 0.33 Acre |  | 0.5 |
|  | i) Offices | 2 | 4 | 15 sq.m | 60 sq.m |  |
|  | ii) Committee rooms | 2 | 3 | 40 sq.m | 120 sq.m |  |
|  | iv) Community Club including indoor games (male and female) | 2 | 2 | 200 sq.m | 400 sq.m |  |
|  | v) Cultural Facilities (Rehearsal, Music room etc) | 1 | 2 | 30 sq.m | 60 sq.m |  |
|  | vi) Community Police Barrack | 1 | 1 | 40 sq.m | 50 sq.m |  |
|  | Vii) Technician Service (Electrical, Plumber, AC, Freeze etc.) | 2 | 4 | 25 sq.m | 100 sq.m |  |
| 10 | Community Hall | 1 | 2 | 0.33 Acre |  | 0.5 |
| 11 | Shops |  |  | 0.33 Acre |  | 0.5 |
|  | i) General store | 3 | 4 | 25 sq.m | 100 sq.m |  |
|  | ii) Grocery | 4 | 6 | 25 sq.m | 150 sq.m |  |
|  | iii) Stationery | 2 | 3 | 25 sq.m | 150 sq.m |  |
|  | iv) Confectionary/Bakery | 2 | 3 | 25 sq.m | 80 sq.m |  |
|  | v) Departmental Store** | 1 | 2 | 100 sq.m | 200 sq.m |  |
|  | vi) Medicine Shop | 2 | 3 | 25 sq.m | 80 sq.m |  |
|  | vii) Sweet Meat Shop | 2 | 3 | 25 sq.m | 80 sq.m |  |
|  | viii) Book/Newspaper Stall | 2 | 3 | 10 sq.m | 30 sq.m |  |
|  | ix) Fresh Corner (Vegetable, fish, meat, egg, chicken etc.) | 2 | 3 | 12 sq.m | 40 sq.m |  |
|  | x) Fruit Shop | 2 | 3 | 10 sq.m | 30 sq.m |  |
|  | xi) Flower Stall | 2 | 2 | 10 sq.m | 30 sq.m |  |
|  | xii) Gift shop | 1 | 2 | 10 sq.m | 30 sq.m |  |
| Total Area for the Neighbourhood Facilities |  |  |  | 22.8 Acres (approx.) |  |  |

- May be added as per decision of the Nagar Unnayan Committee under New use category.

**Area under Departmental Store shall be calculated on the basis of the spaces allocated against one of the corresponding services in this table (cumulative area).

Higher level plan identified Dhaka-Narayanganj-Demra [DND] triangle due to its flood free nature as one of the most potential areas for development. This area shall accommodate not only the naturally growing population of its own rather it is to house additional population from already choked built up Dhaka and inmigrants. Urban residential zone shall be developed in neighbourhood concept with following approximate standard that will be free of through traffic.

Gross area of neighbourhood	: 50 acres [approx.]

Gross density	: 225 to 250 persons per acre.

##### Road Standard

Roads are categorised on the basis of both function and hierarchy which is presented in Table 3.3 showing their corresponding standard widths. Same road has more than one width to make way for laying them as per availability of land.

Table 3.3: Standard Width of Road

| Functional Basis | RoW | Hierarchical Basis |
| --- | --- | --- |
| Arterial I | 30 m | Primary Road I |
| Arterial II | 27 m | Primary Road II |
| Distributor I | 25 m | Secondary Road I |
| Distributor II | 20 m | Secondary Road II |
| Collector I | 15 m | Tertiary Road I |
| Collector II | 12.5 m | Tertiary Road II |
| Access I | 10 m | Access I |
| Access II | 8 m | Access II |

### Preferred Development Strategies

Hydrological aspects dominated the issue of development of a suitable strategy for the physical development of the area so near to the heart of the capital city. All the higher level plans and studies carried out at varying points of time converged to the same conclusion that the drainage function that low lying areas and several rivers carryout, must not be obstructed by any development. Despite this unanimous expert cautions, the area is experiencing a tremendous development pressure with complete disregard to drainage requirement. DAP consultants for the area has worked out some effective strategy/policy for various issues that hinder the desiarable living environment. These are described under some basic heads:

##### Drainage

- Non-continuous smaller rural settlements above flood level surrounded by ample low lying areas (agriculture, flood flow) allowing uninterrupted flow of water to pass through.

- Minimize obstruction of flood water as is practicable.

- Appropriate connectivity by roads having sufficient openings to ensure needful flow of water across them as well as uninterrupted traditional water based connectivity by keeping appropriate navigation clearance at the bridges. This would help maintain the biodiversity of the area and contribute to sustainable environment in turn.

- Retention of excess water in the canal system in the DND area.

##### Residential Development

- Sub-divide Residential Landuse Zone based on the potentiality, trend and opportunity.

- Adopt Neighbourhood concept for new residential developments and provide community facilities as per need of the neighbourhood.

- Minimize through traffic and heavy vehicles within the neighbourhoods.

- Provide adequate safe and easy to move footpaths.

- Ensure community facilities and services of appropriate scale at neighbourhood level.

##### Industrial Development

- Ensure provision of central effluent treatment plant in case of industrial clusters.

- Ensure own treatment plant in case of individual facilities.

- Prohibit high hazard industries within the area.

- Relocate industries from predominantly residential zones in phases.

- Provide essential support facilities for effective functioning of the industries.

##### Mixed Use Development

- Relocate Noxious and heavy industries [red category as per DoE] to Heavy Industrial Zone as soon as practicable.

- Allow the red industries to maintain their status under strict abiding conditions until shifting.

- Ensure adequate safety and security of the people especially of the families residing in such mixed zones.

- Provide sufficient quantity of wide, easy to use and safe footpaths.

- Provide zebra crossing at road crossings instead of foot over bridge to ease the lives of major portion of low income workers likely to traverse on foot to reach their abode in the busy town.

- Ensure adequate utility services to ensure uninterrupted production.

##### Transport and Connectivity

- Provide safe, adequate and comfortable pedestrian paths.

- Provide appropriate and effective public transport routes with sufficient number of quality public transport to carry passenger.

- Grade separation of National and Regional Highways from the local main roads, the latter being at grade and the other two above grades.

##### Flood Flow Zones

- Strictly preserve the zone area as per the higher level plans and DAP.

- Promote agricultural and passive recreational use of the area during dry season.

##### Non-urban Areas

- Strictly preserve agriculture land from conversion into non-agricultural use.

- Promote traditional waterways in the low lying areas by constructing sub-merged road for dry season connectivity.

- Promote rural characteristics in the isolated homesteads keeping mandatory buffer to make way for the flood water to pass through.

##### Water Body and Open Spaces

- Strictly protect canal networks as per the DAP.

- Make provision for open spaces and water body at the neighbourhood level.

- Strictly protect the river fronts and open it to city dwellers for serene passive recreation.

- Make city scale open space with easy accessibility especially for people of densely populated areas with meagre scope for open space.

| 3.3.9 | Amenities and Community Facilities |
| --- | --- |
|  | Consider neighbourhood concept of residential development for estimating community facilities and amenities |
|  | requirement. Prohibit construction of religious structure unless built on its own land. |
|  | Relocate unauthorized religious structures from road right of way to safe-guard greater interest of the people |
|  | especially the city dwellers. Evacuate unauthorized structures and uses from road right of way to safe-guard greater interest of the people |
|  | especially the city dwellers. Close/relocate existing schools with highly inadequate class rooms, play field and essential facilities and |
|  | gradually replace them with standard one i.e. one Primary School of minimum one acre area for 12500 population or neighbourhood of approx. 50 acres. |
| 3.3.10 | Solid Waste Management |
|  | No more conventional disposal through dumping |
|  | Solid Waste processing to ensure recycling |
|  | Conversion of Solid Waste in to fertilizer |
|  | Door to door collection instead of road side bin disposal |
|  | Disposal of Hospital and other hazardous waste in the proposed DCC Regional Disposal sites |
| 3.3.11 | Water Supply |
|  | Harness Surface Water source instead of Ground water |
|  | Explore possibility of processing DND Retention Canal Water |
|  | Continuous monitoring of Tube well water to check Arsenic contamination |
| 3.3.12 | Electricity |
|  | Priority will be given to industry and irrigation pumps in the wake of short generation |
|  | Gradual coverage of the whole area with the increase of generation |
|  | Gradually electricity network will be through underground system |
|  | Explore the possibility of using renewable energy source in order to minimize cost of distribution network |
| 3.3.13 | Gas Supply |
|  | Recommend use of gas in cylinder for domestic purposes |
| 3.3.14 | Conservation of Monument and Heritage |
|  | Identify and record all historical sites and monuments |
|  | Conserve and restore with standard procedure all the historical sites and monuments |
|  | Evict illegal occupants of the historical sites |
|  | Restore the original character of and Sonakanda Fort sites |
|  | Develop the Historical Sites as Tourist spot |
| 3.3.15 | Environmental Management |
|  | Grouping of Hazardous Industries |
|  | Establishment of Common Effluent Treatment Plant |

- Adoption of Neighbourhood Concepts for New Residential Development

- Waste Water Treatment Plant

##### 3.3.16	Supporting the Surrounding Hinterland

- Easy accessibility from the surrounding hinterlands especially growth centers.

- Ensure facilities such as cold storage godowns, wholesale/retail market facilities for commodities like fertilizer, insecticide, agro-machineries etc. and shopping centers of regional standards to support population living in the surrounding hinterlands.

### Major Infrastructure Proposals

##### Transport

Among the vital infrastructure supports required for the smooth functioning of urban life, the foremost is the accessibility. The physical interpretation of this requirement is an effective road network having a range of roads with proper hierarchy starting with major traffic arteries, connectors, distributors and finally down to access roads. Almost always these road networks physically house other vital utilities of urban life – electricity, tap water, gas, sewage disposal, storm water drainage and so forth so that the households come under their services. This calls for careful planning of the road sections off-setting the perennial problem of cutting of urban roads which causes tremendous discomfort to the citizens.

The National Highway No.1 connecting the port city Chittagong with the capital city Dhaka and several other Regional Highways pass through the DAP Group-B area. As maintenance of design speed is of prime importance for these highways, local traffic is disallowed completely. Settlements here have been developing at a rapid pace on both sides of the highways and they require frequent to and fro movement and at present creating tremendous threat to traffic safety and speed of the highway traffic as, such movements are allowed through the highways.

In order to address the above realities, proposed road infrastructure plan has recommended several measures under following three heads:

- National and Regional Highway related issues

- Road hierarchy, and

- Geometric road sections.

##### National and Regional Highway Related Issues

It has been observed that the traffic at the national and regional highways can more or less maintain the design speed all along the highway only except when it passes through some important settlements. The speed at these points is sharply slowed down due to crowding of local traffic on the highway. Following recommendations have been made to overcome the traffic congestion, maintain design speed and ensure safety:

- Primary roads on both sides of the national and regional highways, the later being suggested at a higher grade. These roads can well be accommodated within the existing right of way of the highways. Thus the cost and practical hassle associated with the land acquisition process will not be needed. Layouts are shown in Figure 3.1.

- To ease the frequent movement between homogeneous communities separated by the highways, the primary roads along the highways are linked at grade allowing uninterrupted highway traffic to pass over at a higher grade keeping necessary clearance below for all types of vehicles to pass through (Figure No. 3.3 and 3.4).

- At such points bus bays are proposed beside the highway on both sides equipped with ticket counters, stairs for

connecting the levels. Public toilets for both male and female have also been suggested. Besides, large areas under the ramp of highway may be used by the related line agencies or Paurashava. Figure No. 3.5 shows such provisions.

- Entry to the highways is denied completely only except at about 5 km intervals within the city area. At the entry and exit point too elaborate design measures are proposed for the vehicles to control their speed. Highway Police Box and provision for their petrol vehicles have been made. Figure 3.5 shows a layout of such an entry- exit point.

##### Road Hierarchy

Basically three types of roads make up the network in a city:

- Main traffic arteries providing connectivity among the various parts of the city. The roads serving this purpose are called Primary roads. The consultants suggest 3 standard primary roads.

- The next level roads establish the link between the access roads with the primary roads. These are called Secondary roads. The consultants suggest 2 standards for Secondary roads.

- The last class of roads is access roads. This road provides access to the individual households and link into secondary roads. The consultants suggest 2 standards for Access roads.

All these standards are of course meant for the areas where practicable preferably new development areas. But in many parts which are already built up, these cannot be achieved.

##### Geometric Road Section

Geometric section for all these categories of proposed standard roads have been proposed keeping in mind the following issues:

- Safety and ease of the pedestrian traffic through segregation

- Utility layout in a certain track which would make part of the footpath. It would save the costly road crest as well as the sufferings of the people for maintenance or other works related to utility lines which otherwise done by cutting road crests so frequently.

- Road width has been determined as multiple of standard lane widths not on arbitrary basis.

A standard road section with utility channels on both sides of the road stretching under the foot path is developed for the whole range of roads (Figure No. 3.1, Figure No. 3.2 and Figure No. 3.3).

FOOT PATH WITH DRAIN

FENCING

FOOT PATH WITH DRAIN

DOUBLE LANE

MEDIAN

MEDIAN

DOUBLE LANE

UTILITY CHANNEL WITH SLOPE GREEN TOP

PARKING BAY

FOOT PATH WITH DRAIN

MEDIAN

DOUBLE LANE

UTILITY CHANNEL

SIDE PRIMARY ROAD-1 ALONG NATIONAL/ REGIONAL HIGH WAY ABOVE GRADE

PARKING BAY

DOUBLE LANE

MEDIAN

FENCING

UTILITY CHANNEL

FOOT PATH WITH DRAIN

UTILITY CHANNEL

SIDE PRIMARY ROAD-2 AT GRADE ALONG NATIONAL/ REGIONAL HIGH WAY

##### Figure - 3.1: Standard Road Sections

ME D IA N

PA R K IN G BA Y

DO U B L E L A NE

FO O T P A T H W ITH DR A IN

UT IL IT Y CHA N N E L

PR IM AR Y R O AD - 3

PA R K IN G BA Y

ME D IA N

DO U B L E L A N E

FO O T P A TH W ITH D R A IN

UT IL IT Y  CHA N N E L

SE C O N D A R Y R O A D - 1

Figure - 3.2: Standard Road Sections

PARKING BAY

DOUBLE LANE

FOOT PATH WITHDRAIN

UTILITY CHANNEL

DOUBLE LANE

PARKING BAY

FOOT PATH WITH DRAIN UTILITY CHANNEL

SECONDARY ROAD - 2

SECONDARY ROAD - 3

8m

1m

1m

DOUBLE LANE

FOOT PATH WITH DRAIN

UTILITY CHANNEL

2 x 3 m

DOUBLE LANE

FOOT PATH WITH UTILITY LINES

ACCESS ROAD - 1

ACCESS ROAD -2

##### Figure - 3.3: Standard Road Sections

Detailed Area Plan Transport Network

The Consultants have proposed a network of Primary roads to provide access facilities in the whole of the Group-B area. The proposals are made for short term and the long term. In proposing the network the higher level plan guidelines have been followed and proposed north-south roads have been provided in 1000-1200m interval and east-west roads have been provided in 1500m interval. The proposed network included proposals for the entire area including agricultural zones to secure right of way for future use. The new north-south and east-west road proposed by the consultants are listed in the Table No 3.4.

##### Table - 3.4: DAP Road Proposals

| Sl. No. | Road Name | Length (Km) | RoW (m) | Lane |
| --- | --- | --- | --- | --- |
| 1 | Adamjee Nagar-Sayedpur River Front Road | 11.25 | 27 | 2x21 |
| 2 | Adamjee Road | 7.41 | 30 | 2x2 |
| 3 | Aliganj-Godenail Road | 5.57 | 30 | 2x2 |
| 4 | Aliganj-Jalkundi Road | 2.04 | 18 | 2 |
| 5 | Amirabad-Nabiganj Road | 0.51 | 18 | 2 |
| 6 | Amoir-Gharmora Road | 10.11 | 30 | 2x2 |
| 7 | Bandar Bazar-Koratpara Road | 1.33 | 18 | 2 |
| 8 | Bara Vanga-Demra Road | 1.08 | 12 | 2 |
| 9 | Bhuigar-Ati Road | 3.93 | 27 | 2x2 |
| 10 | Bhuigar-Kutubail Road | 0.69 | 18 | 2 |
| 11 | Bilashnagar-Talla Road | 0.89 | 27 | 2x2 |
| 12 | Kadamtali-Shyampur Road | 0.36 | 12 | 2 |
| 13 | Chashara-Khanpur Road | 2.28 | 12 | 2 |

| Sl. No. | Road Name | Length (Km) | RoW (m) | Lane |
| --- | --- | --- | --- | --- |
| 14 | Dakshin Matuail-Demra Road | 3.79 | 30 | 2x2 |
| 15 | Dapa Idrakpur-Godenail Road | 4.88 | 27 | 2x2 |
| 16 | Dapa Idrakpur-Siachar Road | 1.84 | 18 | 2 |
| 17 | Demra-Sharif Bag Road | 4.26 | 30 | 2x2 |
| 18 | Demra-Siddhirganj Road | 1.88 | 18 | 2 |
| 19 | Demra-Simrail Road | 3.02 | 30 | 2x2 |
| 20 | Deuli-Barapara Road | 4.1 | 30 | 2x2 |
| 21 | Deulpara-Adamjee Road | 4.42 | 27 | 2x2 |
| 22 | Deulpara-North Siachar Road | 1.03 | 18 | 2 |
| 23 | Deulpara Road | 2.02 | 18 | 2 |
| 24 | Dhaka-Chittagong Road | 18.92 | 85 | 2x2+2x2+2x2 |
| 25 | DND Spine Road (Dhaka-Narayanganj Link Road) | 7.67 | 85 | 2x2+2x2+2x2 |
| 26 | Dhakeshwari Road | 0.64 | 18 | 2 |
| 27 | Dhamgar-Madanganj Road | 8.23 | 18 | 2 |
| 28 | Dharmaganj-Nabinagar Road I | 0.67 | 18 | 2 |
| 29 | Dharmaganj-Nabinagar Road II | 0.68 | 18 | 2 |
| 30 | Dudhghata Road | 1.47 | 27 | 2x2 |
| 31 | East Bamail-East Sanarpar Road | 2.71 | 27 | 2x2 |
| 32 | East Kadamtali-Shahidnagar Road | 1.22 | 18 | 2 |
| 33 | East Lamapara Road | 0.36 | 18 | 2 |
| 34 | Ekrampur-Chota Baraikhali Road | 1.14 | 30 | 2x2 |
| 35 | Fatulla-Godenail Link Road | 4.4 | 30 | 2x2 |
| 36 | Fatulla-Lamapara Road | 2.16 | 12 | 2 |
| 37 | Fatulla Bazar-Dharmaganj-Shashangao Road | 5.26 | 30 | 2x2 |
| 38 | General Industry Road 1 | 0.63 | 18 | 2 |
| 39 | Ghognagar-Sayedpur Road | 1.01 | 18 | 2 |
| 40 | Gobindapur-Dakshin Matuail Road | 1.36 | 18 | 2 |
| 41 | Godenail- -Talla Road | 1.82 | 18 | 2 |
| 42 | Godenail Road | 0.38 | 18 | 2 |
| 43 | -Chashara Railway Road | 2.06 | 30 | 2x2 |
| 44 | Heavy Industry Road 3 | 1.12 | 30 | 2x2 |
| 45 | Ispahani Bazar Road | 0.34 | 18 | 2 |
| 46 | Jalkundi-Godenail Road | 2.92 | 18 | 2 |
| 47 | Jalkundi-Kutubpur Road | 0.52 | 18 | 2 |
| 48 | Jalkundi-Lamapara Road | 2.82 | 18 | 2 |
| 49 | Jalkundi-Nayapara Road | 2.81 | 18 | 2 |
| 50 | Jalkundi Road | 0.65 | 18 | 2 |
| 51 | Jamtala-Paikpara Road | 2.08 | 18 | 2 |
| 52 | Jatrabari-Demra Road | 5.97 | 50 | 2+2x2+2 |

| Sl. No. | Road Name | Length (Km) | RoW (m) | Lane |
| --- | --- | --- | --- | --- |
| 53 | Jurain-Chandmari Railway Road | 8.92 | 30 | 2x2 |
| 54 | Jurain-South Matuail Road | 2.09 | 18 | 2 |
| 55 | Kanchpur-Chapatali Road | 1.19 | 27 | 2x2 |
| 56 | Kanchpur-Madanganj Road | 11.38 | 27 | 2x2 |
| 57 | Kajla-Rasulnagar Road | 4.66 | 18 | 2 |
| 58 | Kashipur-Gogenagar Road | 2.62 | 18 | 2 |
| 59 | Kashipur-Narayanganj Road | 2.3 | 30 | 2x2 |
| 60 | Kashipur-Netaiganj-Kashipur Link Road | 3.53 | 18 | 2 |
| 61 | Kayempur Road | 0.64 | 18 | 2 |
| 62 | Khanpur- Road | 0.5 | 30 | 2x2 |
| 63 | Konapara-Rayer Bag Road | 2.24 | 30 | 2x2 |
| 64 | Konapara-Saddam Market Road | 2.71 | 18 | 2 |
| 65 | Konapara-South Matuail Road | 2.56 | 18 | 2 |
| 66 | Kuripara-Gokuldaser Bag Road | 0.54 | 18 | 2 |
| 67 | Kuripara-Malibag Road | 2.59 | 30 | 2x2 |
| 68 | Kutubpur-Enayetnagar Road | 0.68 | 18 | 2 |
| 69 | Lakshmankhola-Nangalbanda Bazaar Road | 3.62 | 30 | 2x2 |
| 70 | Laksmankhola Road 1 | 0.75 | 18 | 2 |
| 71 | Laksmankhola Road 2 | 0.93 | 18 | 2 |
| 72 | Laksmankhola Road 3 | 0.66 | 18 | 2 |
| 73 | Lal Krishnadi-Gaziarpar Road | 2.21 | 27 | 2x2 |
| 74 | Lalpur-Panchabati Road | 1.25 | 18 | 2 |
| 75 | Madanganj-Kabilganj Road | 9.31 | 27 | 2x2 |
| 76 | Madanpur-Madanganj Road | 12.65 | 85 | 2x2+2x2+2x2 |
| 77 | Masdair-Kashipur Hatkhola Road | 3.15 | 30 | 2x2 |
| 78 | Masdair-Chandmari Road | 1.25 | 18 | 2 |
| 79 | Matuail-Baniar Bag Road | 0.88 | 18 | 2 |
| 80 | Matuail Road | 1.53 | 18 | 2 |
| 81 | Mijmiji-Dhankunda Road | 4.22 | 18 | 2 |
| 82 | Mijmiji-Jalkundi Road | 2.21 | 18 | 2 |
| 83 | Mijmiji Godenail Road | 5.14 | 30 | 2x2 |
| 84 | Mijmiji Road 1 | 1.22 | 18 | 2 |
| 85 | Mijmiji Road 2 | 0.57 | 18 | 2 |
| 86 | Mohammadbag-South Matuail Road | 2.44 | 27 | 2x2 |
| 87 | Mozahidnagar-East Kadamtali Road | 1.88 | 27 | 2x2 |
| 88 | Munshikhola-Siddhirganj Road | 7.16 | 30 | 2x2 |
| 89 | Nabiganj Road | 0.91 | 18 | 2 |
| 90 | Netaiganj-Syedpur Road | 1.75 | 18 | 2 |
| 91 | Nandalapur-South Siachar Road | 1.78 | 18 | 2 |

| Sl. No. | Road Name | Length (Km) | RoW (m) | Lane |
| --- | --- | --- | --- | --- |
| 92 | Nandan Kanon-Kalagachia Bazar-Farajikanda Road | 12.59 | 30 | 2x2 |
| 93 | Nischintapur-Kadamtali Road | 5.73 | 18 | 2 |
| 94 | North Kashipur-West Deobhog Road | 1.35 | 18 | 2 |
| 95 | North Laksmankhola Road | 0.7 | 18 | 2 |
| 96 | Old Narayanganj Road | 9.17 | 50 | 2+2x2+2 |
| 97 | Omarpur-Godenail Road | 2.98 | 18 | 2 |
| 98 | Paikpara-Muslimnagar Road | 0.49 | 18 | 2 |
| 99 | Panchabati-Chashara Road | 2.62 | 50 | 2+2x2+2 |
| 100 | Panchabati-Dharmaganj Road | 1.06 | 12 | 2 |
| 101 | Panchabati-Shashangao Road | 0.52 | 18 | 2 |
| 102 | Panchabati-Muktarpur Road | 7 | 50 | 2+2x2+2 |
| 103 | Power Station Road | 1.2 | 18 | 2 |
| 104 | Rajarbag-Godenail Road | 2.05 | 30 | 2x2 |
| 105 | Rasulbag-South Matuail Road | 1.72 | 18 | 2 |
| 106 | Roghunathpur Mijmiji Purbapara Road | 2.68 | 18 | 2 |
| 107 | Sanarpar-Bhuigar Road | 2.75 | 27 | 2x2 |
| 108 | Sarulia–Adamjee Road | 5.06 | 27 | 2x2 |
| 109 | Sarulia Road | 0.67 | 18 | 2 |
| 110 | Sastapur Road | 0.65 | 18 | 2 |
| 111 | Shahebpara-Madaninagar Road | 2.13 | 18 | 2 |
| 112 | Shahidnagar-Bhuigar Road | 1.35 | 18 | 2 |
| 113 | Shantinagar Road | 0.9 | 18 | 2 |
| 114 | Sharifbag-Enayetnagar Road | 1.03 | 12 | 2 |
| 115 | Shashangao-Masdair Road | 1.25 | 27 | 2x2 |
| 116 | Shyampur Road | 0.59 | 27 | 2x2 |
| 117 | Siachar-Godenail Road | 3.94 | 18 | 2 |
| 118 | Siddhirganj-Godenail Road | 2.5 | 27 | 2x2 |
| 119 | Sign Board-Bhuigar Road | 1.59 | 27 | 2x2 |
| 120 | Sonakanda-Mirkandi Road | 4.65 | 30 | 2x2 |
| 121 | South Dania-Siddhirganj | 4.22 | 18 | 2 |
| 122 | South Kutubkhali-East Jurain Road | 1.7 | 12 | 2 |
| 123 | South Matuail-Bhuigar Road | 1.34 | 18 | 2 |
| 124 | South Matuail-Dagar Road | 0.62 | 18 | 2 |
| 125 | South Matuail-Deulpara Road | 3.6 | 30 | 2x2 |
| 126 | South Matuail-Nandalalpur Road | 4.07 | 18 | 2 |
| 127 | South Sastapur Road | 0.69 | 18 | 2 |
| 128 | Stadium Road | 1.85 | 30 | 2x2 |
| 129 | DND Spine Road | 2.65 | 85 | 2x2+2x2+2x2 |
| 130 | Sukurshi Road | 0.9 | 18 | 2 |

| Sl. No. | Road Name | Length (Km) | RoW (m) | Lane |
| --- | --- | --- | --- | --- |
| 131 | Syedpur Road | 1.21 | 18 | 2 |
| 132 | Talla Road | 1.05 | 18 | 2 |
| 133 | Taltala-Bhuigar Road | 3.24 | 27 | 2x2 |
| 134 | Tigerpara-West Deobhog Road | 2.21 | 18 | 2 |
| 135 | Tushardhara-Deulapara Road | 2.03 | 18 | 2 |
| 136 | West Bamail-West Sanarpur Road | 2.65 | 18 | 2 |
| 137 | West Deobhog Road | 0.46 | 18 | 2 |
| 138 | West Masdair Road | 0.55 | 18 | 2 |
| 139 | Nabiganj-Kaikertek-Sonargaon Road | 8 | 30 | 2x2 |
| 140 | Bhuigar-Siachar Road | 2.99 | 18 | 2 |
| 141 | Chashara-Khanpur Road | 1.45 | 18 | 2 |
| 142 | East Siachar-Kutubail Road | 0.8 | 18 | 2 |
| 143 | Goladi-Tipodi Road | 1.43 | 30 | 2x2 |

Source: DAP Proposal

*1= 2x2: 2 lanes x 2 sides of the median;

*2= for highway section, see Figure 3.1

Here it may be mentioned that the proposal includes all the Structure Plan proposed roads excepting one east-west road of the DND area. Actually this road is a side walk of an irrigation canal, which the consultants have proposed to retain as khal for drainage purposes. Map No. 3.1 shows proposed road network of the area.

HIGHWAY POLICE BOX

HIGH WAY AT ABOVE GRADE

HIGH WAY AT ABOVE GRADE

PRIMARY ROAD - 1 AT GRADE WITH THE SETTLEMENT

HIGHWAY POLICE BOX

PRIMARY ROAD - 1 AT GRADE WITH THE SETTLEMENT

PRIMARY ROAD - 1 AT GRADE WITH THE SETTLEMENT

PRIMARY ROAD - 1 AT GRADE WITH THE SETTLEMENT

##### Figure - 3.4: Entry Exit Point to and from National and Regional Highways

RICKSHAWPARKING

PRIMARY ROAD - 1 AT GRADE WITHTHE SETTLEMENT

ZEBRA CROSSING

PUBLIC TOILET (Male) BELOW

DN

TICKETS (UP)

ZEBRA CROSSING

DROOPING AREA (UP)

PUBLIC TOILET (Female) BELOW

DN

TICKETS (UP)

HIGH WAY AT ABOVE GRADE

LIGHT WALLS

MEDIAN	FENCE ON MEDIAN

LINK ROAD BELOW

HIGH WAY AT ABOVE GRADE

TICKETS (UP)

TICKETS (UP)

DN

DROOPING AREA (UP)

DN

ZEBRA CROSSING

PUBLIC TOILET (Male) BELOW

PUBLIC TOILET (Female) BELOW

ZEBRA CROSSING

PRIMARY ROAD - 1 AT GRADE WITHTHE SETTLEMENT

RICKSHAWPARKING

SECTIONALONGTHEHIGHWAY

Figure - 3.5: Link Between Primary Roads Along Either Side of the Highway

SECTION ALONG THE HIGHWAY

SECTION ACROSS THE HIGHWAY

SECTIONACCROSSTHEHIGHWAY

Figure - 3.6: Section of Highway In-between Roads Along Either Side

Commuter Rail Network

The consultants adopt the DMDP proposal for future Commuter Rail Network. A branch of the same, as proposed by the DMDP covers the DND area running eastward from Fatullah Railway Station then turning to north cover the eastern fringe area. The Consultants propose that the commuter rail should be built on the DND spine road right of way. The DND spine road right of way has capacity to accommodate a rail section in it. The Consultants feel that this will help avoid land acquisition for commuter rail in the Group-B area. However, a detailed study is required in this respect.

##### Utility Services

Water Supply

Discussion has been made about the existing water supply system of the area in Chapter-2. It is found that Narayanganj Paurashava, Kadam Rasul Paurashava and a very small part of DND area is covered by piped water supply system. There is no piped water supply system in the Siddhirganj Paurashava and rural parts of the area. The area will continue with present system of collecting water from own or community tube wells. It is proposed that piped water network to be introduced incrementally in the Siddhirganj Paurashava and urban parts of DND during the second phase of the plan period.

##### Total quantity of water requirement

The quantity of water requirement can be computed by using the method: Qr = Pr x q Where, Qr = The quantity of water required per day

Pr = The projected population at the end of design period q = The rate of water consumption per day

##### Assumptions

- Per day per capita water consumption 160 liters

- Technical loss 20%

- Industrial/Commercial demand 20%

##### Water Requirement for Narayanganj Paurashava

Projected demand	= 137 mld Present Production Capacity, Surface	= 45 mld Deep tube-well	= 18 mld

##### Water Requirement for Kadam Rasul Paurashava

Projected demand	= 73 mld Present Production Capacity, Surface water = nil

Deep tube-well water	= 4.56 mld

From the above analysis, it is found that there is a huge gap between the projected demand and the existing installed capacity. To improve the situation huge investment from the public fund will be required for existing urban areas alone. On the other hand, it will require more investment for expansion of piped water for new urban areas mostly areas of Siddhirganj Paurashava, DND and Narayanganj west area.

In this situation, pollution of Sitalakhya river and Buriganga river should be controlled to enable use of their water with minimum cost. The whole matter depends on the policy of the central government. It is beyond the capacity of the local government institutions. The idea of bringing water from Meghna or Mawa ghats may prove very expensive in the long run.

In the prevailing situation, the consultants recommend that both surface and ground water sources should be exploited with major emphasis on surface water.

In established urban areas it is difficult to secure enough green/uncovered area for enabling percolation of water for ground water recharge. Provision has been made to open up the existing canals and create open spaces which will help percolation of rain water. In the DND area the proposed canal system provides openings for percolation of water. However, a separate study is recommended for investigating the possibility of use of water to be retained in the canal system for treatment for the human consumption. For existing urban area of Narayanganj Paurashava and Kadam Rasul Paurashava, there is no scope for treatment of waste water.

##### Solid Waste Management

Recycling of solid waste is gradually becoming popular in the wake of its potentiality for reuse as well as to avoid problems associated with disposal through dumping method. In many countries this recycling process is encouraged through healthy incentives too.

Solid wastes are mostly generated from kitchen wastes and from use of day to day essentials. They generally have two parts: perishable and non-perishable. The non-perishable part is sent directly to the factories as raw materials and it comes back to the people as new product. The perishable portion constitutes the major share of the solid waste. With a simple technology this huge waste can be transformed into quality natural fertilizer. It effectively saves from the adverse effect of chemical fertilizers and raises agricultural production in a very eco-friendly way.

A vast land area in Group-B jurisdiction has been recommended to continue as its present agricultural use. The Group-B adjoining Munshiganj area is renowned for its potato production.

Considering all these, the consultants recommend that there should be no more dumping of solid waste in the low lying areas as is practiced presently causing huge pollution, filling out low lying natural water retention areas and so forth. Instead, entire solid waste management should focus on recycling and composting of such waste to capture above benefits.

Solid waste processing area has been earmarked in the proposed landuse map. The proposed landuse control procedures, has necessary provision for accommodating solid waste processing areas as may be required by the relevant agency.

It may be mentioned here that, Waste Concern, a local organisation, active with the research and implementation of such recycling and composting procedure in the Dhaka Metropolitan area headed by two successful Urban Planners has already earned very respectable UNICEF award for their discovery and effective contribution in this field.

##### Drainage

Drainage is one of the major concerns of Narayanganj people. Once very successful Dhaka-Narayanganj-Demra (DND) Irrigation Project has now turned into a curse for the inhabitants. DND Drainage improvement is nowadays synonymous to drainage problem of the whole Narayanganj area. This has been specially considered in the drainage mitigation proposal.

##### Drainage Improvement: DND Area Background

DND is a leading irrigation project executed by BWDB during 1962-1968 to fulfil national objectives of attaining self

reliance in food grain production. The DND area covers 56.79 km2. The area is spread over two districts, they are Demra & Shyampur thanas of the Dhaka district and Fatullah & Siddirganj thanas of the Narayanganj district.

Since 2004 the DND area is experiencing protracted water logging causing immense sufferings to the inhabitants. Description of the DND area has been made in chapter-II, as such in this section attempt has been made to explain the devise for pumping out the accumulated water to retrieve the area from water logging. Finally recommendations have been made to adopt for mitigation measures of the problem. However, it is evident that the mitigation of the problem will need substantial investment from public fund and further study will be required for taking up projects for mitigation of the problem.

##### Compartmentalization of DND

According to JICA proposals, DND is divided into three drainage compartments for efficient discharge of accumulated water from precipitation and accommodate the time lag required for pumping out of the excess water. The proposal includes provision of separate Pump Houses for each compartment, located in three convenient locations for quick discharge in to outfall (river). Considering the run off distance to travel, a slight modification of the compartments has been proposed in DAP Plan.

##### Planning Options

  - Widening of existing khals

  - Excavation of existing khals for required depth (3.5 m)

  - Installation of additional Pumps (improvement of drainage discharge capacity)

  - Demarcation of retention areas, if needed

  - Connection of missing links (khals)

  - Storm water drainage ( more than required section) to get rid of temporary inundation

  - Restoration of canal (according to RS Mauza)

  - Storage and retention ponds

##### Recommended Options

  - Use the existing canals as Retention Area instead of separate Retention Ponds

  - Re-delineation of the boundaries of compartment C and B

  - Restoration of encroached canal

  - Excavation of existing canal for design depth

  - Connection of missing links of canals

  - Road side storm water drainage (more than required section)

  - Recycling of stored water for supply of drinking water within DND area

  - Installation of additional pumps (As per JICA proposal)

##### Advantages of canal proposal

Canal ensures

  - Retention

  - Drainage of water

  - Intake at every point

  - Evaporation & percolation of water

  - Minimum of acquisition

To avoid acquisition of land for retention pond it has been decided to utilize the existing canals for retention of water. Stakeholders are also against retention pond system, as this will result in large scale eviction which will give rise to social problem and general resentment in the community. On the other hand, the existing canals, if re-excavated to proper sections are capable of retaining water for the time lag required to pump out.

### Amenities and Urban Facilities Proposals

Attempts have been made to provide amenities and facilities for the entire Group-B area people. The proposal for amenities and facilities has been made in line with people’s aspirations quantified through stakeholders’ consultation. Moreover, despite nation wide reputation of Narayanganj for national level sportsmen, Narayanganj lacks in facilities for the promotion of games and sports. Provisions have made to mitigate this problem also. The proposals are compiled below:

##### Academics

Public Medical College at Old District Court and Jail premises Site for University in the DND Area

##### Park and Open Space

Restoration of and Sonakanda Forts as regional tourist spots Regional Park at Sign Board area of DND

Theme Park at Jimkhana

Restoration of Khanpur Children Park

River front development along the Sitalakhya and the Buriganga rivers Walkway with vegetation along the DND canals

New Park in the Khas land south of Labour Welfare Centre, Nawab Salimullah Road, Narayanganj

##### Sports and Recreation

International standard Sports Complex at Chandmari

Development of Jimkhana Play Field as Specialized Football Stadium Regional level aquatic recreational centre at Charargope

### Description of the Plan

##### Proposed Plan

Plan proposal for the Group-B area includes various types of landuses. The DND area is designated as one of the future urban expansion areas of the DMDP area. It is expected that at the end of the plan period the area will accommodate about 1 million additional people. To meet the future requirement of circulation network and right of ways of roads with appropriate widths (in order to accommodate utility provision) have been proposed. Summary of the proposed landuse is provided in the Table No. 3.5. From the table it is found that Urban Residential Zone and Rural Settlement Zone together occupies the highest amount of land followed by the Agricultural land. The volume of Agricultural land and Flood Flow Zone indicates that a substantial area of the Group-B will remain rural at the end of the project period. Detailed description of the plan has been made on the basis of Detailed Planning Zone (DPZ). Landuses of the entire Group-B area is shown in Integrated Planning Map No. 3.10.

##### Table - 3.5: Summary of Proposed Landuse Zone

| Landuse | Area (Acres) | Area (Hectares) | Area (Percentage) |
| --- | --- | --- | --- |
| Administrative Zone | 105.46 | 42.68 | 0.21 |
| Agricultural Zone | 12125.99 | 4907.22 | 23.94 |
| Commercial Zone (Business) | 365.05 | 147.73 | 0.72 |
| Flood Flow Zone | 5912.03 | 2392.52 | 11.67 |
| General Industrial Zone | 2089.86 | 845.74 | 4.13 |

| Landuse | Area (Acres) | Area (Hectares) | Area (Percentage) |
| --- | --- | --- | --- |
| Heavy Industrial Zone | 164.71 | 66.66 | 0.33 |
| Institutional Zone | 125.2 | 50.67 | 0.25 |
| Mixed Use Zone (Commercial-General Industrial) | 241.64 | 97.79 | 0.48 |
| Mixed Use Zone (Residential-Commercial) | 1463.32 | 592.19 | 2.89 |
| Mixed Use Zone (Residential-Commercial-General Industrial) | 2246.16 | 908.99 | 4.43 |
| Open Space | 205.92 | 83.33 | 0.41 |
| Overlay Zone | 381.26 | 154.29 | 0.75 |
| Proposed Road | 3135.71 | 1268.98 | 6.19 |
| Rural Settlement Zone | 5302.53 | 2145.86 | 10.47 |
| Transport & Communication | 570.26 | 230.78 | 1.13 |
| Urban Residential Zone | 11310.95 | 4577.39 | 22.31 |
| Water Retention Area | 781.53 | 316.27 | 1.54 |
| Waterbody | 4136.09 | 1673.82 | 8.17 |
| Total | 50663.67 | 20502.89 | 100 |

DPZ-01 Shyampur-Dania-Matuail

Detailed Planning Zone-01 consists of three unions namely: Shyampur, Dania, Matuail and covers about 5283.97 acres. According to 2001 Census population of the area is 370232; projection made for the year 2015 indicates an increase of 300145 persons making the total 670377.

The DMDP higher level plans designated this area as new urban area to be mostly used for residential purposes. The development trend indicates that the area is developing as residential areas. There are large tracts of undeveloped land which can be developed in a planned manner. In order to arrest the present sprawl development trend RAJUK should intervene and impose strict development control measure and initiate development through people’s participation.

The RAJUK sponsored Kadamtali-Shympur Industrial Park is located here. Other than this industrial park, there are several clusters where industrial activities are carried out. The hazardous industries of the area have been recommended to be shifted to the designated area gradually. The only sewage treatment plant of Dhaka is also located here though it does not serve the area.

Pagla Fault which runs east-west along the Pagla canal is a potential source of natural disaster and for which necessity of separate study has been pointed out by the consultants. Proposed landuse for the zone is presented by the following Table 3.6.

##### Table - 3.6: Proposed Landuse of DPZ-01

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 1 | General Industrial Zone | 442.82 | 179.2 | 8.38 | Ghop Dakshin | 8 | 35.9 | 14.53 |
|  |  |  |  |  | Joka | 425 | 108.52 | 43.91 |
|  |  |  |  |  | Kadamtali | 515 | 193.23 | 78.2 |
|  |  |  |  |  | Sarlia | 474 | 105.18 | 42.56 |
| 2 | Institutional Zone | 25.65 | 10.38 | 0.49 | Kadamtali | 74 | 25.65 | 10.38 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 3 | Mixed Use Zone (Commercial-General Industrial) | 44.71 | 18.09 | 0.85 | Ailbahar | 24 | 17.46 | 7.06 |
|  |  |  |  |  | Kadamtali | 61 | 27.25 | 11.03 |
| 4 | Mixed Use Zone (Residential- Commercial) | 178.5 | 72.22 | 3.38 | Dagar | 194 | 63.47 | 25.68 |
|  |  |  |  |  | Joka | 43 | 10.82 | 4.38 |
|  |  |  |  |  | Matuail | 323 | 104.2 | 42.17 |
| 5 | Mixed Use Zone (Residential- Commercial-General Industrial) | 462.9 | 187.32 | 8.76 | Ailbahar | 171 | 39.61 | 16.03 |
|  |  |  |  |  | Dagar | 65 | 26.02 | 10.53 |
|  |  |  |  |  | Dania | 445 | 136.62 | 55.29 |
|  |  |  |  |  | Kadamtali | 121 | 15.58 | 6.31 |
|  |  |  |  |  | Matuail | 712 | 212.17 | 85.86 |
|  |  |  |  |  | Syampur | 120 | 32.89 | 13.31 |
| 6 | Open Space | 174.2 | 70.51 | 3.3 | Ailbahar | 18 | 29.61 | 11.98 |
|  |  |  |  |  | Dagar | 157 | 52.15 | 21.11 |
|  |  |  |  |  | Kadamtali | 25 | 65.06 | 26.33 |
|  |  |  |  |  | Matuail | 92 | 27.39 | 11.08 |
| 7 | Overlay Zone | 189.58 | 76.72 | 3.59 | Kadamtali | 9 | 170.37 | 68.95 |
|  |  |  |  |  | Syampur | 3 | 19.21 | 7.77 |
| 8 | Proposed Road | 566.58 | 229.3 | 10.72 | Ailbahar | 40 | 7.58 | 3.06 |
|  |  |  |  |  | Bamail | 18 | 1.76 | 0.71 |
|  |  |  |  |  | Dagar | 651 | 91.13 | 36.87 |
|  |  |  |  |  | Dania | 511 | 79.85 | 32.31 |
|  |  |  |  |  | Deilla | 23 | 9.11 | 3.69 |
|  |  |  |  |  | Ghop Dakshin | 29 | 15.69 | 6.35 |
|  |  |  |  |  | Joka | 219 | 23.77 | 9.62 |
|  |  |  |  |  | Kadamtali | 445 | 80.15 | 32.43 |
|  |  |  |  |  | Matuail | 958 | 165.47 | 66.98 |
|  |  |  |  |  | Paradagar | 197 | 42.32 | 17.13 |
|  |  |  |  |  | Payati | 29 | 4.77 | 1.93 |
|  |  |  |  |  | Sarlia | 358 | 32.42 | 13.13 |
|  |  |  |  |  | Syampur | 102 | 12.59 | 5.1 |
| 9 | Transport & Communication | 62.89 | 25.45 | 1.19 | Ailbahar | 21 | 2.53 | 1.03 |
|  |  |  |  |  | Bamail | 8 | 0.78 | 0.32 |
|  |  |  |  |  | Dagar | 29 | 5.96 | 2.41 |
|  |  |  |  |  | Dania | 44 | 8.27 | 3.34 |
|  |  |  |  |  | Deilla | 18 | 3.11 | 1.26 |
|  |  |  |  |  | Ghop Dakshin | 11 | 1.07 | 0.43 |
|  |  |  |  |  | Joka | 14 | 2.1 | 0.85 |
|  |  |  |  |  | Kadamtali | 16 | 6.73 | 2.73 |
|  |  |  |  |  | Matuail | 90 | 17.96 | 7.27 |
|  |  |  |  |  | Paradagar | 23 | 7.65 | 3.1 |
|  |  |  |  |  | Payati | 3 | 0.26 | 0.11 |
|  |  |  |  |  | Sarlia | 24 | 2.79 | 1.13 |
|  |  |  |  |  | Syampur | 5 | 3.67 | 1.48 |
| 10 | Urban Residential Zone | 2903.58 | 1175.02 | 54.95 | Bamail | 181 | 51.76 | 20.95 |
|  |  |  |  |  | Dagar | 1821 | 508.66 | 205.85 |
|  |  |  |  |  | Dania | 2400 | 572.51 | 231.66 |
|  |  |  |  |  | Deilla | 40 | 9.55 | 3.87 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Urban Residential Zone |  |  |  | Ghop Dakshin | 151 | 45.68 | 18.49 |
|  |  |  |  |  | Joka | 467 | 155.25 | 62.83 |
|  |  |  |  |  | Kadamtali | 531 | 211.63 | 85.66 |
|  |  |  |  |  | Matuail | 2699 | 689.84 | 279.16 |
|  |  |  |  |  | Paradagar | 1285 | 324.59 | 131.36 |
|  |  |  |  |  | Payati | 32 | 6.17 | 2.5 |
|  |  |  |  |  | Sarlia | 950 | 218.22 | 88.31 |
|  |  |  |  |  | Syampur | 311 | 109.71 | 44.4 |
| 11 | Water Retention Area | 150.41 | 60.88 | 2.85 | Bamail | 28 | 1.91 | 0.77 |
|  |  |  |  |  | Dagar | 262 | 32.31 | 13.09 |
|  |  |  |  |  | Dania | 150 | 13.38 | 5.41 |
|  |  |  |  |  | Deilla | 10 | 0.38 | 0.15 |
|  |  |  |  |  | Ghop Dakshin | 23 | 1.83 | 0.74 |
|  |  |  |  |  | Joka | 80 | 8.15 | 3.3 |
|  |  |  |  |  | Kadamtali | 138 | 13.75 | 5.57 |
|  |  |  |  |  | Matuail | 451 | 49.22 | 19.92 |
|  |  |  |  |  | Paradagar | 128 | 8.76 | 3.55 |
|  |  |  |  |  | Sarlia | 68 | 7.71 | 3.12 |
|  |  |  |  |  | Syampur | 120 | 13 | 5.26 |
| 12 | Waterbody | 82.16 | 33.24 | 1.55 | Ailbahar | 2 | 0.13 | 0.05 |
|  |  |  |  |  | Bamail | 3 | 0.04 | 0.02 |
|  |  |  |  |  | Dagar | 26 | 4.26 | 1.72 |
|  |  |  |  |  | Dania | 14 | 10.45 | 4.23 |
|  |  |  |  |  | Deilla | 18 | 4.5 | 1.82 |
|  |  |  |  |  | Ghop Dakshin | 16 | 7.31 | 2.96 |
|  |  |  |  |  | Joka | 4 | 1.24 | 0.5 |
|  |  |  |  |  | Kadamtali | 12 | 4.15 | 1.68 |
|  |  |  |  |  | Matuail | 56 | 20.16 | 8.15 |
|  |  |  |  |  | Paradagar | 20 | 15.62 | 6.32 |
|  |  |  |  |  | Payati | 10 | 1.54 | 0.62 |
|  |  |  |  |  | Sarlia | 41 | 8.44 | 3.42 |
|  |  |  |  |  | Syampur | 13 | 4.33 | 1.75 |
| Total |  | 5283.97 | 2138.33 | 100 |  | 20526 | 5283.97 | 2138.33 |

This fast growing area is not covered by municipal administration. The consultants propose that a new municipality namely ‘Matuail Paurashava.’ be created to provide municipal services in the area. The consultants also propose to include in the said municipality the area east of Jatrabar-Postagala road now under DCC. It is recommended that the area should not be included in the DCC, rather itself should become a separate municipality.

The zone is served by Dhaka-Chittagong, Jatrabari-Demra, Old Dhaka-Narayanganj and Demra-Adamjee roads. However, the existing internal roads are narrow and ill drawn. In order to provide road network capable of serving the projected traffic several north-south and east-west roads have been proposed in the zone. Road hierarchy has been maintained and for smooth functioning of national and regional roads proposal has been made for separate lanes for highway traffic and local traffic. It is also proposed that the leased out plots of the right of way of Dhaka-Chittagong road of the zone should be reverted in order to reconstruct it according to the design road section for this road.

The proposed Mass Rail Transit route to follow the Link Road right of way (quite a substantial length of the road runs through the zone. However, right of way of the road in this zone has not been acquired yet).

For drainage mitigation, widening of existing khals and link khals to enable water to reach the pumping station has been proposed. Integrated Planning Map No. 3.2 shows the proposed landuse along with the road network of DPZ-01. The proposed road network of the DPZ 01 is also presented in the Table 3.7.

##### Table - 3.7: Proposed New Roads of DPZ-01

| Road Name | Width (m) | Length (Km) |
| --- | --- | --- |
| Bara Vanga-Demra Road | 12 | 1.07 |
| DND Spine Road | 85 | 2.63 |
| Dakshin Matuail-Demra Road | 30 | 3.79 |
| Demra-Chittagong Highway Intersection Road | 30 | 1.85 |
| Demra-Siddhirganj Road | 18 | 1.17 |
| Dhaka-Chittagong Highway | 85 | 4.59 |
| East Bamail-East Sanarpar Road | 27 | 2.15 |
| East Kadamtali-Shahidnagar Road | 18 | 1.21 |
| Gobindapur-Dakshin Matuail Road | 18 | 1.36 |
| Jatrabari-Demra Road | 50 | 5.97 |
| Jurain-Chandmari Railway Road | 30 | 1.79 |
| Jurain-South Matuail Road | 18 | 2.04 |
| Kadamtali-Shyampur Road | 12 | 0.36 |
| Kajla-Rasulnagar Road | 18 | 4.66 |
| Konapara-Rayer Bag Road | 30 | 2.24 |
| Konapara-Saddam Market Road | 18 | 2.71 |
| Konapara-South Matuail Road | 18 | 2.59 |
| Matuail-Baniar Bag Road | 18 | 0.88 |
| Matuail Road | 18 | 1.53 |
| Mujahidnagar-East Kadamtali Road | 27 | 1.87 |
| Munshikhola-Siddhirganj Road | 30 | 0.54 |
| Old Dhaka-Narayanganj Road | 50 | 2.95 |
| Rasulbag-South Matuail Road | 18 | 1.72 |
| Sarulia-Adamjee Road | 27 | 2.20 |
| Sarulia Road | 18 | 0.67 |
| Shahidnagar-Bhuigar Road | 18 | 0.23 |
| Sharif Bag Road | 30 | 4.13 |
| Shyampur-South Matuail Road | 27 | 2.44 |
| Shyampur Road | 27 | 0.59 |
| Sign Board-Bhuigar Road | 27 | 0.28 |
| South Dania-Siddhirganj Road | 18 | 3.77 |
| South Kutubkhali-East Jurain Road | 12 | 1.54 |
| South Matuail-Bhuigar Road | 18 | 0.77 |
| South Matuail-Dagar Road | 18 | 1.13 |

| Road Name | Width (m) | Length (Km) |
| --- | --- | --- |
| South Matuail-Deulpara Road | 30 | 1.00 |
| South Matuail-Nandalalpur Road | 18 | 4.02 |
| West Bamail-West Sanarpur Road | 18 | 2.56 |

DPZ-02 Kutubpur-Fatullah-Enayet Nagar

Kutubpur union, Fatullah union and parts of Enayet Nagar union constitute the Detailed Planning Zone no.02. It covers an area of 5024.11 acres with a projected population of 546525 for 2015. According to 2001 Census population of the zone was 301831.

Pagla Fault which runs east-west along the Pagla canal is partially located in the zone. The consultants recommend that a separate study on the impact of the fault should be carried out by RAJUK. However, all construction works in the area of influence of the fault must adhere to the BNBC set out guidelines.

Proposed landuse for the zone is presented by the following Table 3.8

##### Table - 3.8: Proposed Landuse of DPZ-02

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 1 | Flood Flow Zone | 178.08 | 72.07 | 3.54 | Dharmaganj | 360 | 104.26 | 42.19 |
|  |  |  |  |  | Enayetnagar | 26 | 3.18 | 1.29 |
|  |  |  |  |  | Hariharpara | 114 | 28.35 | 11.47 |
|  |  |  |  |  | Sridhardi | 129 | 42.30 | 17.12 |
| 2 | General Industrial Zone | 245.88 | 99.50 | 4.89 | Aliganj | 203 | 52.36 | 21.19 |
|  |  |  |  |  | Dapa Indrakpur | 165 | 60.85 | 24.63 |
|  |  |  |  |  | Dhopatita | 143 | 50.88 | 20.59 |
|  |  |  |  |  | Hariharpara | 2 | 0.02 | 0.01 |
|  |  |  |  |  | Pagla | 235 | 72.63 | 29.39 |
|  |  |  |  |  | Pilkuni | 25 | 9.15 | 3.70 |
| 3 | Institutional Zone | 31.06 | 12.57 | 0.62 | Deulpara | 70 | 31.06 | 12.57 |
| 4 | Mixed Use Zone (Commercial -General Industrial) | 196.93 | 79.69 | 3.92 | Aliganj | 37 | 25.78 | 10.43 |
|  |  |  |  |  | Alinagar | 20 | 12.31 | 4.98 |
|  |  |  |  |  | Dapa Indrakpur | 48 | 18.41 | 7.45 |
|  |  |  |  |  | Dharmaganj | 247 | 74.72 | 30.24 |
|  |  |  |  |  | Dhopatita | 33 | 15.48 | 6.26 |
|  |  |  |  |  | Fatulla | 33 | 4.28 | 1.73 |
|  |  |  |  |  | Fazilpur | 61 | 20.70 | 8.38 |
|  |  |  |  |  | Hariharpara | 45 | 9.89 | 4.00 |
|  |  |  |  |  | Pagla | 22 | 7.45 | 3.01 |
|  |  |  |  |  | Uttar Azmatpur | 4 | 7.91 | 3.20 |
| 5 | Mixed Use Zone (Residential- Commercial) | 83.02 | 33.61 | 1.65 | Bhuigar | 161 | 50.07 | 20.27 |
|  |  |  |  |  | Hajeeiganj | 96 | 32.96 | 13.34 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 6 | Mixed Use Zone (Residential - Commercial -General Industrial) | 491.41 | 198.89 | 9.78 | Bag Benidas | 4 | 1.08 | 0.44 |
|  |  |  |  |  | Dapa Indrakpur | 429 | 144.74 | 58.58 |
|  |  |  |  |  | Deulpara | 435 | 130.63 | 52.87 |
|  |  |  |  |  | Dhopatita | 166 | 49.11 | 19.88 |
|  |  |  |  |  | Khizirpur | 235 | 31.95 | 12.93 |
|  |  |  |  |  | Kutubpur | 335 | 85.85 | 34.75 |
|  |  |  |  |  | Pagla | 48 | 9.43 | 3.82 |
|  |  |  |  |  | Siachar | 127 | 38.63 | 15.63 |
| 7 | Open Space | 9.54 | 3.86 | 0.19 | Bhuigar | 24 | 5.24 | 2.12 |
|  |  |  |  |  | Deulpara | 3 | 1.74 | 0.70 |
|  |  |  |  |  | Pagla | 2 | 2.55 | 1.03 |
| 8 | Overlay Zone | 65.77 | 26.62 | 1.31 | Deulpara | 10 | 3.03 | 1.22 |
|  |  |  |  |  | Kutubpur | 78 | 28.77 | 11.64 |
|  |  |  |  |  | Pagla | 4 | 33.97 | 13.75 |
| 9 | Proposed Road | 519.25 | 210.14 | 10.34 | Aliganj | 52 | 10.95 | 4.44 |
|  |  |  |  |  | Alinagar | 18 | 3.52 | 1.42 |
|  |  |  |  |  | Bhuigar | 1001 | 117.62 | 47.59 |
|  |  |  |  |  | Dapa Indrakpur | 300 | 39.70 | 16.06 |
|  |  |  |  |  | Deulpara | 1056 | 114.26 | 46.25 |
|  |  |  |  |  | Dharmaganj | 238 | 30.40 | 12.31 |
|  |  |  |  |  | Dhopatita | 118 | 23.64 | 9.57 |
|  |  |  |  |  | Enayetnagar | 51 | 6.32 | 2.56 |
|  |  |  |  |  | Fatulla | 125 | 10.10 | 4.09 |
|  |  |  |  |  | Fazilpur | 7 | 2.74 | 1.11 |
|  |  |  |  |  | Hariharpara | 179 | 23.68 | 9.58 |
|  |  |  |  |  | Hajeeganj | 31 | 3.33 | 1.35 |
|  |  |  |  |  | Kaempur | 12 | 0.10 | 0.04 |
|  |  |  |  |  | Khizirpur | 141 | 6.19 | 2.51 |
|  |  |  |  |  | Kutubpur | 290 | 44.66 | 18.07 |
|  |  |  |  |  | Lalpur | 14 | 0.43 | 0.17 |
|  |  |  |  |  | Pagla | 144 | 28.09 | 11.37 |
|  |  |  |  |  | Pilkuni | 194 | 20.98 | 8.48 |
|  |  |  |  |  | Siachar | 312 | 29.25 | 11.84 |
|  |  |  |  |  | Talla | 4 | 0.04 | 0.02 |
|  |  |  |  |  | Uttar Azmatpur | 35 | 3.29 | 1.33 |
| 10 | Rural Settlement Zone | 190.48 | 77.08 | 3.79 | Dharmaganj | 148 | 50.31 | 20.35 |
|  |  |  |  |  | Hariharpara | 144 | 46.30 | 18.74 |
|  |  |  |  |  | Sridhardi | 333 | 93.88 | 37.99 |
| 11 | Transport & Communication | 88.73 | 35.91 | 1.77 | Aliganj | 7 | 2.00 | 0.81 |
|  |  |  |  |  | Alinagar | 2 | 0.21 | 0.09 |
|  |  |  |  |  | Bhuigar | 24 | 4.31 | 1.74 |
|  |  |  |  |  | Dapa Indrakpur | 77 | 19.04 | 7.70 |
|  |  |  |  |  | Deulpara | 43 | 5.79 | 2.35 |
|  |  |  |  |  | Dharmaganj | 24 | 5.42 | 2.19 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Transport & Communication |  |  |  | Dhopatita | 35 | 11.20 | 4.53 |
|  |  |  |  |  | Fazilpur | 4 | 0.28 | 0.11 |
|  |  |  |  |  | Hariharpara | 20 | 2.39 | 0.97 |
|  |  |  |  |  | Hajeeganj | 3 | 0.93 | 0.37 |
|  |  |  |  |  | Khizirpur | 10 | 1.28 | 0.52 |
|  |  |  |  |  | Kutubpur | 42 | 5.69 | 2.30 |
|  |  |  |  |  | Pagla | 24 | 14.39 | 5.82 |
|  |  |  |  |  | Pilkuni | 32 | 6.73 | 2.72 |
|  |  |  |  |  | Siachar | 38 | 6.30 | 2.55 |
|  |  |  |  |  | Sridhardi | 6 | 1.50 | 0.61 |
|  |  |  |  |  | Uttar Azmatpur | 8 | 1.29 | 0.52 |
| 12 | Urban Residential Zone | 2576.59 | 1042.6 | 51.28 | Aliganj | 1 | 0.10 | 0.04 |
|  |  |  |  |  | Bhuigar | 3190 | 678.65 | 274.62 |
|  |  |  |  |  | Dapa Indrakpur | 293 | 91.72 | 37.12 |
|  |  |  |  |  | Deulpara | 2785 | 822.26 | 332.76 |
|  |  |  |  |  | Dharmaganj | 458 | 121.87 | 49.31 |
|  |  |  |  |  | Dhopatita | 67 | 17.34 | 7.01 |
|  |  |  |  |  | Hariharpara | 244 | 86.36 | 34.95 |
|  |  |  |  |  | Hajeeganj | 59 | 12.56 | 5.08 |
|  |  |  |  |  | Khizirpur | 29 | 4.24 | 1.72 |
|  |  |  |  |  | Kutubpur | 784 | 248.36 | 100.51 |
|  |  |  |  |  | Pagla | 263 | 87.91 | 35.57 |
|  |  |  |  |  | Pilkuni | 675 | 171.62 | 69.44 |
|  |  |  |  |  | Siachar | 761 | 227.29 | 91.98 |
|  |  |  |  |  | Uttar Azmatpur | 35 | 6.31 | 2.55 |
| 13 | Water Retention Area | 285.88 | 115.69 | 5.69 | Aliganj | 80 | 17.12 | 6.93 |
|  |  |  |  |  | Bag Benidas | 11 | 0.70 | 0.29 |
|  |  |  |  |  | Bhuigar | 530 | 44.11 | 17.84 |
|  |  |  |  |  | Dapa Indrakpur | 75 | 19.05 | 7.71 |
|  |  |  |  |  | Deulpara | 862 | 72.99 | 29.54 |
|  |  |  |  |  | Dhopatita | 146 | 70.31 | 28.46 |
|  |  |  |  |  | Fatulla | 51 | 1.96 | 0.79 |
|  |  |  |  |  | Godnail | 4 | 0.04 | 0.02 |
|  |  |  |  |  | Hajeeganj | 40 | 2.32 | 0.94 |
|  |  |  |  |  | Kaempur | 63 | 2.41 | 0.98 |
|  |  |  |  |  | Khizirpur | 77 | 2.78 | 1.13 |
|  |  |  |  |  | Kutubpur | 109 | 7.89 | 3.19 |
|  |  |  |  |  | Lalpur | 36 | 1.56 | 0.63 |
|  |  |  |  |  | Pagla | 91 | 16.89 | 6.83 |
|  |  |  |  |  | Pilkuni | 174 | 18.24 | 7.38 |
|  |  |  |  |  | Siachar | 134 | 7.03 | 2.84 |
|  |  |  |  |  | Talla | 16 | 0.50 | 0.20 |
| 14 | Waterbody | 61.49 | 24.89 | 1.22 | Aliganj | 9 | 0.29 | 0.12 |
|  |  |  |  |  | Alinagar | 1 | 0.07 | 0.03 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Waterbody |  |  |  | Bhuigar | 39 | 9.27 | 3.75 |
|  |  |  |  |  | Dapa Indrakpur | 2 | 3.95 | 1.60 |
|  |  |  |  |  | Deulpara | 24 | 2.97 | 1.20 |
|  |  |  |  |  | Dharmaganj | 42 | 30.51 | 12.35 |
|  |  |  |  |  | Dhopatita | 3 | 1.71 | 0.69 |
|  |  |  |  |  | Enayetnagar | 4 | 1.74 | 0.70 |
|  |  |  |  |  | Fazilpur | 7 | 0.66 | 0.27 |
|  |  |  |  |  | Hariharpara | 32 | 6.47 | 2.62 |
|  |  |  |  |  | Hajeeganj | 5 | 0.12 | 0.05 |
|  |  |  |  |  | Khizirpur | 7 | 0.49 | 0.20 |
|  |  |  |  |  | Kutubpur | 3 | 0.58 | 0.24 |
|  |  |  |  |  | Pilkuni | 10 | 0.03 | 0.01 |
|  |  |  |  |  | Siachar | 6 | 0.04 | 0.02 |
|  |  |  |  |  | Sridhardi | 6 | 2.58 | 1.04 |
|  |  |  |  |  | Talla | 3 | 0.02 | 0.01 |
| Total |  | 5024.11 | 2033.17 | 100.00 |  | 21871 | 5024.11 | 2033.17 |

The zone is fast growing and parts of the area are old settlement areas. However, the area is not covered by municipal administration. In order to ensure municipal services in the zone the consultants propose to create a new municipality covering the Kutubpur Union and parts of Fatullah and Enayet Nagar unions. The proposed municipality may be named as ‘Kutubpur Paurashava.’

The DND Drainage plan through a system of canal network in compartments with separate Pump Houses is expected to solve the water logging problem of the DND parts of the DPZ.

Proposed Railway Chord line should not be linked with Fatullah, which should follow a different alignment to connect Dhirasham directly.

The proposed Mass Rail Transit route to follow the Link Road right of way (quite a substantial length of the road runs through the zone).

Dyeing industries located in the DND area is proposed to be gradually relocated in the extended general industrial zone around the BSCIC industrial estate. A central effluent treatment plant to treat the effluent of the dyeing industries is proposed to be set up by the government. Until shifting of the dyeing industries from the DND area, these will be treated as non conforming use in the DPZ and will not be allowed to expand.

The Buriganga river front is extensively used for storage of bulk construction materials and encroachment in to the river is common. It is proposed that BIWTA take effective measure to prevent such encroachment.

It is also recommended that Landing Stations should be provided in this part of Circular Water Way to enable people to avail the water way for travel to destinations around river front.

The zone is served by Old Dhaka-Narayanganj Road, Narayanganj Link Road and the Dhaka-Narayanganj Rail road. To improve the internal connectivity road network proposal includes several north-south and east-west roads. To increase the efficiency of Old Dhaka-Narayanganj road grade separation is proposed at places where right of way is difficult to secure. Here it may be mentioned that the acquired right of way of the road has been encroached at several points. It is also found that some section of the road has been leased out to private individuals. This will severely impede the proposed

improvement of the road. The proposed widening generally follows the acquisition line. But at places the acquired right of way is less than the proposed widening. The Panchabati intersection is proposed to be improved through grade separation. Map 3.3 shows the proposed landuse and road network of the DPZ 02.

The proposed road network of the DPZ 02 is also presented in the following Table 3.9.

##### Table - 3.9: Proposed New Roads of DPZ-02

| Road Name | Width (m.) | Length (Km.) |
| --- | --- | --- |
| Aliganj-Godenail Road | 30 | 2.37 |
| Aliganj-Jalkundi Road | 18 | 1.45 |
| BSCIC Road | 18 | 0.02 |
| Bhuigar-Ati Road | 27 | 1.05 |
| Bhuigar-Siachar Road | 18 | 2.21 |
| DND Spine Road | 85 | 0.00 |
| Dapa Idrakpur-Godenail Road | 27 | 1.07 |
| Dapa Idrakpur-Siachar Road | 18 | 1.84 |
| Deulpara-Adamjee Road | 27 | 0.32 |
| Deulpara-North Siachar Road | 18 | 1.03 |
| Deulpara Road | 18 | 1.72 |
| Dharmaganj-Arafatnagar Road | 18 | 0.67 |
| Dharmaganj Road | 18 | 0.68 |
| East Kadamtali-Shahidnagar Road | 18 | 1.21 |
| East Lamapara Road | 18 | 0.36 |
| East Siachar-Kutubail Road | 18 | 0.80 |
| Fatulla-Godenail Link Road | 30 | 0.29 |
| Fatulla-Lamapara Road | 12 | 2.16 |
| Fatulla Bazar-Dharmaganj-Shashangaon Road | 30 | 4.85 |
| Godenail--Talla Road | 18 | 0.30 |
| Godenail Road | 18 | 0.24 |
| Chashara Railway Road | 30 | 0.01 |
| Jalkundi-Kutubpur Road | 18 | 0.30 |
| Jalkundi-Lamapara Road | 18 | 0.28 |
| Jurain-Chandmari Railway Road | 30 | 4.55 |
| Kutubpur-Enayetnagar Road | 18 | 0.31 |
| Lalpur-Panchabati Road | 18 | 0.04 |
| Mujahidnagar-East Kadamtali Road | 27 | 1.87 |
| Munshikhola-Siddhirganj Road | 30 | 4.15 |
| Nandalalpur-South Siachar Road | 18 | 1.78 |
| Narayanganj Link Road | 85 | 3.98 |
| Nischintapur-Kadamtali Road | 18 | 2.88 |
| Old Dhaka-Narayanganj Road | 50 | 4.53 |
| Panchabati-Dharmaganj Road | 12 | 1.04 |
| Panchabati-Muktarpur Road | 50 | 1.66 |
| Panchabati-Shashangaon Road | 18 | 0.52 |
| Raghunathpur-Madani Nagar Road | 18 | 1.30 |
| Rajarbag-Godenail Road | 30 | 2.05 |
| Roghunathpur Mijmiji Purbopara Road | 18 | 0.70 |

| Road Name | Width (m.) | Length (Km.) |
| --- | --- | --- |
| Sahebpara-Raghunathpur Road | 18 | 0.42 |
| Sanarpar-Bhuigar Road | 27 | 1.66 |
| Sastapur Road | 18 | 0.08 |
| Shahidnagar-Bhuigar Road | 18 | 1.12 |
| Sharif Bag Road | 30 | 0.13 |
| Sharifbag-Enayet Nagar Road | 12 | 1.03 |
| Shashangaon-Masdair Road | 27 | 0.02 |
| Siachar-Godenail Road | 18 | 2.53 |
| Sign Board-Bhuigar Road | 27 | 1.30 |
| South Matuail-Bhuigar Road | 18 | 0.57 |
| South Matuail-Deulpara Road | 30 | 2.60 |
| South Matuail-Nandalalpur Road | 18 | 4.06 |
| Stadium Road | 30 | 1.63 |
| Talla Road | 18 | 0.50 |
| Taltala-Bhuigar Road | 27 | 3.24 |
| Tushardhara-Deulapara Road | 18 | 2.03 |

DPZ-03 Siddhirganj Paurashava

Siddhirganj Paurashava has been recently created covering a large portion of the DND and most of the Adamjee-Demra industrial belt. It covers an area of 5849.56 acres with a population of 260687 according to 2001 Census report. The projected population in 2015 will be 472025.

In the higher level plans, the DND part of the Paurashava has been designated as New Urban Land Development Area, Sitalakhya river front as Industrial Moderate Hazard zone and established urban area. The development trend indicates that Bhuigar, Jalkundi and Godenyle areas are fast developing. However, a large area within the zone is agricultural land and under pressure of speculative land transfer through sub-division in to small plots. On the other hand the Old Adamjee- Demra industrial belt is rejuvenating with the establishment of an EPZ in the Adamjee premises. Besides the Siddhirganj Power House and the Grain Silo, several cotton spinning, composite knit garment and jute industries have been set up in the area. Sitalakhya river front at the foot of the Kanchpur Bridge is being encroached upon by traders of bulk building materials.

The zone is served by a part of the Dhaka-Chittagong road, Demra-Adamjee-Narayanganj road and Narayanganj Link road. To improve the internal connectivity road network proposal includes several north-south and east-west roads of appropriate widths. To increase the efficiency of the Chittagong road intersection and Narayanganj Link road intersection grade separated traffic crossing has been proposed. For providing Clover Leaf intersection separate study is required. Widening of the Demra-Adamjee road has been proposed. Leased out plots of right of way of Dhaka-Chittagong road is also proposed to be cancelled to enable the reconstruction of the road according to the design section proposed for the road.

The areas between Demra-Adamjee road and the Sitalakhya river will be developed through land fill above flood level. There is no proposal for the implementation of the FAP 8A compartment of this location. It is also not feasible in the context of present development trend and public fund requirement for implementation of such projects. However, the 2nd Pump House of the DND project will be located in the Adamjee area of the zone. Proposed landuse for the zone is presented in the Table 3.10.

##### Table - 3.10: Proposed Landuse of DPZ-03

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acres | Hectares | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 1 | General Industrial Zone | 477.36 | 193.18 | 8.16 | Shimrail | 291 | 116.52 | 47.16 |
|  |  |  |  |  | Siddhirganj | 309 | 360.84 | 146.03 |
| 2 | Institutional Zone | 58.17 | 23.54 | 0.99 | Jalkundi | 185 | 58.17 | 23.54 |
| 3 | Mixed Use Zone (Residential- Commercial) | 599.47 | 242.61 | 10.25 | Dhankunda | 160 | 35.78 | 14.47 |
|  |  |  |  |  | Godnail | 791 | 205.75 | 83.26 |
|  |  |  |  |  | Hajeeganj | 5 | 1.63 | 0.66 |
|  |  |  |  |  | Jalkundi | 372 | 95.73 | 38.75 |
|  |  |  |  |  | Khorda Ghoshpara | 407 | 148.11 | 59.94 |
|  |  |  |  |  | Siddhirganj | 485 | 112.47 | 45.52 |
| 4 | Mixed Use Zone (Residential- Commercial- General Industrial) | 323.83 | 131.04 | 5.54 | Ati | 58 | 14.25 | 5.77 |
|  |  |  |  |  | Dhankunda | 10 | 1.70 | 0.69 |
|  |  |  |  |  | Godnail | 320 | 102.95 | 41.66 |
|  |  |  |  |  | Khorda Ghoshpara | 118 | 38.68 | 15.66 |
|  |  |  |  |  | Shimrail | 223 | 70.89 | 28.68 |
|  |  |  |  |  | Siddhirganj | 537 | 95.36 | 38.60 |
| 5 | Open Space | 6.81 | 2.76 | 0.12 | Shimrail | 3 | 1.07 | 0.43 |
|  |  |  |  |  | Siddhirganj | 16 | 5.74 | 2.32 |
| 6 | Proposed Road | 531.12 | 214.93 | 9.08 | Ati | 183 | 39.07 | 15.81 |
|  |  |  |  |  | Azibpur | 45 | 5.50 | 2.22 |
|  |  |  |  |  | Dhankunda | 124 | 12.22 | 4.94 |
|  |  |  |  |  | Godnail | 561 | 70.51 | 28.52 |
|  |  |  |  |  | Hajeeganj | 4 | 0.48 | 0.19 |
|  |  |  |  |  | Jalkundi | 1384 | 161.52 | 65.37 |
|  |  |  |  |  | Khorda Ghoshpara | 313 | 64.39 | 26.07 |
|  |  |  |  |  | Kutubpur | 6 | 0.13 | 0.05 |
|  |  |  |  |  | Shimrail | 133 | 30.68 | 12.42 |
|  |  |  |  |  | Siddhirganj | 1350 | 146.63 | 59.33 |
| 7 | Transport & Communication | 56.73 | 22.95 | 0.97 | Ati | 19 | 7.10 | 2.87 |
|  |  |  |  |  | Azibpur | 17 | 3.63 | 1.47 |
|  |  |  |  |  | Dhankunda | 4 | 1.53 | 0.62 |
|  |  |  |  |  | Godnail | 18 | 6.81 | 2.76 |
|  |  |  |  |  | Jalkundi | 30 | 5.65 | 2.28 |
|  |  |  |  |  | Khorda Ghoshpara | 19 | 5.22 | 2.11 |
|  |  |  |  |  | Shimrail | 19 | 4.50 | 1.82 |
|  |  |  |  |  | Siddhirganj | 111 | 22.29 | 9.02 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acres | Hectares | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 8 | Urban Residential Zone | 2938.38 | 1189.13 | 50.23 | Ati | 606 | 237.21 | 95.99 |
|  |  |  |  |  | Azibpur | 405 | 79.04 | 31.99 |
|  |  |  |  |  | Dhankunda | 187 | 51.28 | 20.75 |
|  |  |  |  |  | Godnail | 1129 | 285.32 | 115.48 |
|  |  |  |  |  | Hajeeganj | 6 | 0.26 | 0.10 |
|  |  |  |  |  | Jalkundi | 3810 | 989.96 | 400.61 |
|  |  |  |  |  | Khorda Ghoshpara | 736 | 179.66 | 72.70 |
|  |  |  |  |  | Kutubpur | 41 | 0.42 | 0.17 |
|  |  |  |  |  | Shimrail | 145 | 46.93 | 19.00 |
|  |  |  |  |  | Siddhirganj | 4789 | 1068.33 | 432.34 |
| 9 | Water Retention Area | 312.96 | 126.61 | 5.35 | Ati | 62 | 15.07 | 6.10 |
|  |  |  |  |  | Azibpur | 3 | 0.35 | 0.14 |
|  |  |  |  |  | Dhankunda | 61 | 5.50 | 2.22 |
|  |  |  |  |  | Godnail | 516 | 54.73 | 22.15 |
|  |  |  |  |  | Hajeeganj | 3 | 0.10 | 0.04 |
|  |  |  |  |  | Jalkundi | 937 | 83.71 | 33.86 |
|  |  |  |  |  | Khorda Ghoshpara | 117 | 30.66 | 12.41 |
|  |  |  |  |  | Kutubpur | 18 | 0.04 | 0.02 |
|  |  |  |  |  | Shimrail | 31 | 11.65 | 4.71 |
|  |  |  |  |  | Siddhirganj | 966 | 111.16 | 44.96 |
| 10 | Waterbody | 544.71 | 220.43 | 9.31 | Ati | 20 | 100.70 | 40.75 |
|  |  |  |  |  | Azibpur | 3 | 27.31 | 11.05 |
|  |  |  |  |  | Dhankunda | 11 | 0.11 | 0.04 |
|  |  |  |  |  | Godnail | 26 | 30.62 | 12.39 |
|  |  |  |  |  | Jalkundi | 61 | 3.91 | 1.58 |
|  |  |  |  |  | Khorda Ghoshpara | 25 | 1.68 | 0.69 |
|  |  |  |  |  | Shimrail | 64 | 82.31 | 33.31 |
|  |  |  |  |  | Siddhirganj | 77 | 298.08 | 120.63 |
| Total |  | 5849.56 | 2367.20 | 100.00 |  | 23485 | 5849.56 | 2367.20 |

Landuse proposal for the zone includes a site of Institutional use especially setting up of a Public University to cater to the need of higher education of the people of DND and Narayanganj area. Other proposals include areas of mixed use development and General Industrial Zone. Proposal has been made to relocate the hazardous industries of the area to the designated areas of this type of industries. The stakeholders’ wish to set up a private Inland Container Depot (ICD) in the Adamjee Premises does not hold good as that has been converted to a EPZ. On the other hand river front and the undeveloped areas between settlements offer opportunity for setting up of an ICD in the zone.

The proposed Mass Rail Transit route to follow the Link road right of way (a very small portion runs through the zone). It is also proposed that the now abandoned railway track connecting with a loop with Adamjee rail to be connected with

Narayanganj Rail Station through now abandoned railway track along the proposed road on the west bank of Sitalakhya river.

On completion of Detailed Area Plan project RAJUK should undertake Action Area Plan for the development of large pockets of undeveloped land within the zone through people’s participation (Land Readjustment Technique).

The proposed road network of the DPZ 03 is also presented in the following Table 3.11.

##### Table - 3.11: Proposed New Roads of DPZ-03

| Road Name | Width (m.) | Length (Km.) |
| --- | --- | --- |
| Adamjee Nagar-Sayedpur River Front Road | 27 | 1.01 |
| Adamjee Road | 30 | 6.91 |
| Aliganj-Godenail Road | 30 | 3.20 |
| Aliganj-Jalkundi Road | 18 | 0.59 |
| Bhuigar-Ati Road | 27 | 3.21 |
| Bhuigar-Jalkundi Road | 18 | 0.69 |
| Bhuigar-Siachar Road | 18 | 0.78 |
| DND Spine Road | 85 | 0.02 |
| Dapa Idrakpur-Godenail Road | 27 | 3.36 |
| Demra-Chittagong Highway Intersection Road | 30 | 1.87 |
| Demra-Siddhirganj Road | 18 | 0.71 |
| Deulpara-Adamjee Road | 27 | 4.10 |
| Deulpara Road | 18 | 0.30 |
| Dhaka-Chittagong Highway | 85 | 4.15 |
| East Bamail-East Sanarpar Road | 27 | 0.57 |
| Fatulla-Godenail Link Road | 30 | 0.62 |
| Godenail--Talla Road | 18 | 0.76 |
| Godenail Road | 18 | 0.15 |
| Jalkundi -Godenail Road | 18 | 2.44 |
| Jalkundi-Kutubpur Road | 18 | 0.22 |
| Jalkundi-Lamapara Road | 18 | 2.55 |
| Jalkundi Nayapara Road | 18 | 2.81 |
| Jalkundi Road | 18 | 0.65 |
| Kutubpur-Enayet Nagar Road | 18 | 0.37 |
| Mijmiji-Dhankunda Road | 18 | 4.22 |
| Mijmiji-Jalkundi Road | 18 | 2.21 |
| Mijmiji Godenail Road | 30 | 5.14 |
| Mijmiji Road 1 | 18 | 1.22 |
| Mijmiji Road 2 | 18 | 0.57 |
| Munshikhola-Siddhirganj Road | 30 | 2.46 |
| Narayanganj Link Road | 85 | 1.80 |
| Nischintapur-Kadamtali Road | 18 | 2.85 |

| Road Name | Width (m.) | Length (Km.) |
| --- | --- | --- |
| Omarpur-Godenail Road | 18 | 2.98 |
| Power Station Road | 18 | 1.20 |
| Rajarbag-Godenail Road | 30 | 2.04 |
| Roghunathpur Mijmiji Purbopara Road | 18 | 1.98 |
| Sahebpara-Raghunathpur Road | 18 | 0.41 |
| Sanarpar-Bhuigar Road | 27 | 1.09 |
| Sarulia-Adamjee Road | 27 | 2.91 |
| Shantinagar Road | 18 | 0.90 |
| Siachar-Godenail Road | 18 | 0.99 |
| Siddhirganj-Godenail Road | 27 | 2.50 |
| South Dania-Siddhirganj Road | 18 | 0.45 |
| Stadium Road | 30 | 0.22 |
| Sukurshi Road | 18 | 0.90 |
| West Bamail-West Sanarpur Road | 18 | 0.10 |

Map No. 3.4 shows the landuse and road network of DPZ-03.

##### DPZ-04 Baktabali-Alirtek

DPZ-04 is constituted with Baktabali union and Alirtek union. Baktabali and Alirtek unions are low lying and part of flood flow zone. The zone is accessible through Old Dhaka-Narayanganj and Mukterpur road. The DPZ-04 covers an area of 7911.16 acres with a population of 49930 according to 2001 Census report. The projected population in 2015 will be 84847.

No change of the landuse has been proposed in this part of the Dhaleshwari Flood Plain. Landuse for the zone is presented in the following Table 3.12. Map No. 3.5 shows the landuse and road network of DPZ-04.

##### Table - 3.12: Proposed Landuse of DPZ-04

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acres | Hectares | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 1 | Flood Flow Zone | 5214.35 | 2110.11 | 65.91 | Alirtek | 796 | 266.86 | 107.98 |
|  |  |  |  |  | Char Bairagadi | 315 | 146.78 | 59.40 |
|  |  |  |  |  | Char Baktaballi | 6739 | 2250.36 | 910.64 |
|  |  |  |  |  | Char Gangaprashad | 751 | 237.71 | 96.19 |
|  |  |  |  |  | Char Garkul | 1342 | 467.82 | 189.32 |
|  |  |  |  |  | Char Indradi | 308 | 86.90 | 35.16 |
|  |  |  |  |  | Gopchar | 3238 | 1243.84 | 503.36 |
|  |  |  |  |  | Kashipur | 1386 | 493.06 | 199.55 |
|  |  |  |  |  | Masinabanda | 13 | 1.96 | 0.79 |
|  |  |  |  |  | Mirganj | 38 | 17.42 | 7.05 |
|  |  |  |  |  | Saiyadpur | 12 | 1.64 | 0.67 |
| 2 | Proposed Road | 65.33 | 26.44 | 0.83 | Bara Deobhog | 44 | 3.22 | 1.31 |
|  |  |  |  |  | Kashipur | 257 | 26.92 | 10.90 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acres | Hectares | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Proposed Road |  |  |  | Masinabanda | 47 | 9.03 | 3.66 |
|  |  |  |  |  | Saiyadpur | 185 | 26.15 | 10.58 |
| 3 | Rural Settlement Zone | 1331.31 | 538.77 | 16.83 | Alirtek | 228 | 54.77 | 22.16 |
|  |  |  |  |  | Char Bairagadi | 68 | 26.31 | 10.65 |
|  |  |  |  |  | Char Baktaballi | 2512 | 639.26 | 258.72 |
|  |  |  |  |  | Char Gangaprashad | 235 | 43.66 | 17.67 |
|  |  |  |  |  | Char Garkul | 370 | 77.79 | 31.48 |
|  |  |  |  |  | Char Indradi | 143 | 25.93 | 10.50 |
|  |  |  |  |  | Gopchar | 836 | 182.50 | 73.84 |
|  |  |  |  |  | Kashipur | 950 | 274.83 | 111.22 |
|  |  |  |  |  | Mirganj | 19 | 6.26 | 2.53 |
| 4 | Transport & Communication | 21.53 | 8.71 | 0.27 | Alirtek | 1 | 0.50 | 0.20 |
|  |  |  |  |  | Char Baktaballi | 21 | 12.64 | 5.12 |
|  |  |  |  |  | Char Garkul | 5 | 1.87 | 0.76 |
|  |  |  |  |  | Char Indradi | 1 | 0.45 | 0.18 |
|  |  |  |  |  | Gopchar | 4 | 3.23 | 1.31 |
|  |  |  |  |  | Kashipur | 7 | 2.23 | 0.90 |
|  |  |  |  |  | Masinabanda | 2 | 0.61 | 0.25 |
| 5 | Urban Residential Zone | 0.31 | 0.13 | 0.01 | Masinabanda | 6 | 0.21 | 0.08 |
|  |  |  |  |  | Saiyadpur | 7 | 0.10 | 0.04 |
| 6 | Waterbody | 1278.32 | 517.32 | 16.16 | Alirtek | 6 | 65.36 | 26.45 |
|  |  |  |  |  | Char Bairagadi | 2 | 24.17 | 9.78 |
|  |  |  |  |  | Char Baktaballi | 153 | 665.31 | 269.25 |
|  |  |  |  |  | Char Gangaprashad | 4 | 3.76 | 1.52 |
|  |  |  |  |  | Char Garkul | 4 | 43.38 | 17.56 |
|  |  |  |  |  | Char Indradi | 2 | 17.33 | 7.01 |
|  |  |  |  |  | Gopchar | 407 | 323.15 | 130.78 |
|  |  |  |  |  | Kashipur | 34 | 78.22 | 31.65 |
|  |  |  |  |  | Masinabanda | 30 | 11.45 | 4.63 |
|  |  |  |  |  | Saiyadpur | 128 | 46.20 | 18.69 |
| Total |  | 7911.16 | 3201.49 | 100 |  | 21656 | 7911.16 | 3201.49 |

As the zone is a part of Flood Plain, no new road has been proposed for it to discourage development in the area which in turn will protect its main function of drainage without impediments.

The detail of the only proposed road (actually extension of the existing Panchabati-Mukterpur Road) of the DPZ 04 is presented in the following Table 3.13.

##### Table - 3.13: Proposed New Road of DPZ-04

| Road Name | Width (m) | Length (Km) |
| --- | --- | --- |
| Panchabati - Muktarpur Road | 50 | 5.25 |

DPZ-05 Narayanganj Paurashava and Adjoining Areas

Narayanganj is one of the oldest urban centers of the country and the largest river port. Activities related to jute processing and transportation, for which Narayanganj came to be known as Dundee of the east, is waning. Historically Narayanganj was a salt trading center which is continuing till today in much larger scale. Flour milling and small-scale engineering workshops to meet the local needs are also found here. For its easy transportation linkages Narayanganj is one of important centers of the whole sale trading of all types of commodities, especially textile products. Traders from all around the country collect their needed commodities like yarn, dyes, chemicals, hosiery items, flour, edible oil, salt cement rod etc. from Narayanganj. Narayanganj is housing garment factory or garment factory related activities extensively.

Though Narayanganj is one of the old urban centers of the country, the development is haphazard and sprawl. There is no planned residential area within the zone. Roads are mostly narrow and houses being built covering maximum plot area leaving no space for road widening. There are several spots in the municipality where water logging takes place with every drop of rainfall. There is no play ground and park while utility provision is poor. The city seriously lacks standard academic institution. It takes unnecessarily longer time in commuting to Dhaka due to congestion on roads. There is no town service in the city. Right of ways of the important roads is occupied by hawkers. The city is overwhelmed by non motorized transport like rickshaws which make road crossing at several points very difficult. Water way is not properly used due to government policies which make huge infrastructure remaining underutilized. The zone houses the new District Administration complex, District Jail and the Narayanganj Sadar Upazila complex. The BSCIC Knit Garments estate is another element of the zone. The Kashipur union is developing fast as new urban area. Industrial developments are also found in the union.

Development proposals for the zone includes several project based uses ranging from academics to historical site preservation.

##### Medical College

A Public Medical College is proposed to be set up in the Narayanganj Old D.C. Office Complex including the abandoned sub-jail structure. This premise is bounded in the west by Institute Road, in the north by Shaista Khan Road, in the east by Narayanganj General Post Office, Bangladesh Telecommunication Company Ltd. office and Narayanganj College and in the south by Paurashava owned new katcha bazaar building.

##### Chandmari Sports Complex and Civic Centre

Chandmari Play Ground in the Chashara area along the Dhaka-Narayanganj Link road is to become a Sport Complex and Civic Centre of international standard. Here it may be mentioned that this was one of the prime sports ground of the Narayanganj Town until late eighties of the last century when it became a slum. During the last Caretaker Government’s rule slums were removed and it is now under military control. This Railway property was given to the Police Department for shooting practice, used to be extensively used by the local people as sports ground and jogging site also. This traditional play field is to become a sports complex with facilities to hold shooting and other indoor games competition of Commonwealth Games standard. This will also provide jogging facilities for the residents of Narayanganj. A preliminary design of the Complex is provided in the Chapter-5.

##### Fort Complex

Fort area was a sports complex before the World War-II. Annual Sports of the local schools used to be held in the open ground around the Fort. During the World War-II it was converted to a temporary camp for the American soldiers with a Field Hospital built in a portion of the Fort. The Field Hospital was converted to a Maternity Hospital after the end of the war and later it was given to the Fire Brigade. Fire Brigade has been shifted to its own building nearby and the Archaeology Department has recently reconstructed the site to return it to its original shape. After partition of the country in 1947, Jute based activities started to shift from Kolkata to Narayanganj area in the newly created country. At that time to meet the demand for storage area, the Jute Board constructed a number of jute godowns in this open space. On the decline of jute based activities, Jute Board has leased out almost the entire godown area to private individuals. Thus the process of premature death of this open space was completed. The Dacca Master Plan 1959 earmarked this and 44 acres area around the Fort to develop as an open space and tourist spot. Proposal has been included in the Detailed Area Plan to return this place to its original use. A preliminary design has been included in Chapter-5.

##### Aquatic Sports Complex at Charargope

The Detailed Area Plan has included a scheme to convert the charagope area including parts of Narayanganj Railway Station and Kumudiny Welfare Trust premises into a Regional level Aquatic Sports Complex. A preliminary design has been included in the Chapter-5.

##### Theme Park at Jimkhana Railway Land

The traditional Jimkhana ground is proposed to be converted to a theme park for the recreation of the residents of the Narayanganj town. A preliminary design has been included in Chapter-5.

##### Alauddin Memorial Football Stadium

The traditional Jimkhana Play Ground is proposed to be converted to a Football Stadium in the memory of Late Alauddin Khan, a famous Football Player from Narayanganj town.

Other proposals include restoration of Khanpur Children’s Park in its original shape, a new ladies park in the khas land located at the southern boundary of the Labour Welfare Centre on the Nawab Salimullah road, a museum in the BIWTA Banglow in the Barafkal area, restoration of the Fatullah Cattle Hat as open space and play ground, Amusement Park at Saidpur and Isdair.

A number of new roads have been proposed to make the circulation network of the zone efficient. List of the roads are provided in the Table No 3.6. The widening of Old Dhaka-Narayanganj road affects some of the important structures at the Chasara point. The Baitul Aman building and its adjacent mosque, Santana Market, Chasara Police Out-Post and the Zila Parishad Dakbanglow are some of them. All these structures can be rebuilt in their existing premises with readjustment. In order to improve the connectivity and travel need of the town dwellers a serious thought over the issue has to be given so that the widening of the road can be possible in the long run. The Panchabati intersection is proposed to be improved through grade separation.

The new road along the Adamjee Railway tract connecting the Old Dhaka-Narayanganj road above grade will need further detailed study. The Dhaka-Narayanganj Link road at the Chashara point has been proposed to be widened to 150'. The recently constructed Military Market and the Rifle Club will be affected by this proposal. Rifle Club is proposed to to be shifted to the Chandmari proposed Sports Complex. A new above grade Bus Terminal is proposed in the present Chashara Bus Stand site extending up to Chashara Railway Station. For strategic reason Chashara Railway Station may have to be relocated. The treatment of this area is a complex one, but due to its importance as being the gateway to Narayanganj it cannot be ignored. Consultants recommend that a comprehensive study of this Road, Rail Communication be undertaken immediately.

On the other hand leased out land parcel of the Old Dhaka-Narayanganj road is proposed to be returned back to the road right of way and all incidence of encroachment removed. The religious structure in front of the DC Complex, the eastern portion of the Fatullah Stadium and several other structures have been built on the right of way of the Narayanganj Link Road. For future extension of the road this has to be stopped.

The jurisdiction of the Narayanganj Paurashava area (demand for making it a corporation is mounting every day) is proposed to be extended to the boundary of the Detailed Planning Zone-5 boundary. The zone covers an area of 4893.46 acres with a population of 254558 according to 2001 Census report. The projected population in 2015 will be 432577.

Proposed landuse for the zone is presented by the following Table 3.14.

##### Table - 3.14: Proposed Landuse of DPZ-05

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 1 | Administrative Zone | 105.47 | 42.69 | 2.16 | Fatulla | 10 | 2.35 | 0.95 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Administrative Zone |  |  |  | Isdair | 7 | 2.13 | 0.86 |
|  |  |  |  |  | Kaempur | 120 | 36.26 | 14.68 |
|  |  |  |  |  | Khanpur | 191 | 64.73 | 26.20 |
| 2 | Commercial Zone (Business) | 222.96 | 90.22 | 4.56 | Bhagabanganj (Ma) | 139 | 6.28 | 2.54 |
|  |  |  |  |  | Chasara (Ma) | 5 | 1.38 | 0.56 |
|  |  |  |  |  | Fatulla | 2 | 4.69 | 1.90 |
|  |  |  |  |  | Khanpur | 16 | 5.95 | 2.41 |
|  |  |  |  |  | Narayanganj | 1023 | 98.18 | 39.73 |
|  |  |  |  |  | Paikpara (Ma) | 77 | 3.23 | 1.31 |
|  |  |  |  |  | Saiyadpur (Ma) | 80 | 47.26 | 19.12 |
|  |  |  |  |  | Shitalakshya | 428 | 55.99 | 22.65 |
| 3 | Flood Flow Zone | 265.16 | 107.31 | 5.42 | Kashipur | 2 | 0.05 | 0.02 |
|  |  |  |  |  | Masinabanda | 6 | 1.64 | 0.66 |
|  |  |  |  |  | Saiyadpur | 600 | 226.86 | 91.81 |
|  |  |  |  |  | Saiyadpur (Ma) | 68 | 36.62 | 14.82 |
| 4 | General Industrial Zone | 404.23 | 163.58 | 8.26 | Bara Deobhog | 26 | 2.31 | 0.93 |
|  |  |  |  |  | Baraibhog | 184 | 57.85 | 23.41 |
|  |  |  |  |  | Enayetnagar | 81 | 17.00 | 6.89 |
|  |  |  |  |  | Godnail | 455 | 171.64 | 69.46 |
|  |  |  |  |  | Godnail (Ma) | 15 | 15.78 | 6.39 |
|  |  |  |  |  | Hariharpara | 356 | 126.49 | 51.19 |
|  |  |  |  |  | Kashipur | 15 | 3.04 | 1.23 |
|  |  |  |  |  | Masdair | 34 | 10.13 | 4.10 |
| 5 | Institutional Zone | 10.31 | 4.17 | 0.21 | Khanpur | 2 | 5.83 | 2.36 |
|  |  |  |  |  | Narayanganj | 11 | 4.48 | 1.81 |
| 6 | Mixed Use Zone (Residential-Commercial) | 363.37 | 147.05 | 7.43 | Chasara | 14 | 2.07 | 0.84 |
|  |  |  |  |  | Chasara (Ma) | 585 | 48.23 | 19.52 |
|  |  |  |  |  | Deobhog (Ma) | 2 | 0.83 | 0.34 |
|  |  |  |  |  | Fatulla | 27 | 10.02 | 4.06 |
|  |  |  |  |  | Hajeeganj | 147 | 38.33 | 15.51 |
|  |  |  |  |  | Isdair | 129 | 33.56 | 13.58 |
|  |  |  |  |  | Kaempur | 79 | 23.39 | 9.47 |
|  |  |  |  |  | Khanpur | 284 | 115.70 | 46.82 |
|  |  |  |  |  | Masdair | 169 | 52.67 | 21.31 |
|  |  |  |  |  | Mukraba | 9 | 3.89 | 1.57 |
|  |  |  |  |  | Narayanganj | 126 | 18.31 | 7.41 |
|  |  |  |  |  | Talla | 71 | 16.38 | 6.63 |

| Sl. No. | Lanuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 7 | Mixed Use Zone (Residential-Commercial- General Industrial) | 440.72 | 178.34 | 9.01 | Bag Benidas | 12 | 6.08 | 2.46 |
|  |  |  |  |  | Fatulla | 566 | 178.55 | 72.26 |
|  |  |  |  |  | Godnail | 327 | 119.33 | 48.29 |
|  |  |  |  |  | Godnail (Ma) | 17 | 18.45 | 7.47 |
|  |  |  |  |  | Hajeeganj | 32 | 8.77 | 3.55 |
|  |  |  |  |  | Hajeeganj (Ma) | 48 | 13.57 | 5.49 |
|  |  |  |  |  | Kaempur | 77 | 19.25 | 7.79 |
|  |  |  |  |  | Lalpur | 251 | 76.71 | 31.04 |
| 8 | Open Space | 15.37 | 6.22 | 0.31 | Hajeeganj | 8 | 2.06 | 0.83 |
|  |  |  |  |  | Kashipur | 1 | 0.75 | 0.30 |
|  |  |  |  |  | Khanpur | 3 | 2.66 | 1.08 |
|  |  |  |  |  | Mukraba | 4 | 2.98 | 1.21 |
|  |  |  |  |  | Saiyadpur (Ma) | 6 | 1.52 | 0.62 |
|  |  |  |  |  | Shitalakshya | 1 | 5.41 | 2.19 |
| 9 | Overlay Zone | 102.52 | 41.49 | 2.09 | Chasara (Ma) | 1 | 0.05 | 0.02 |
|  |  |  |  |  | Hajeeganj | 9 | 22.50 | 9.11 |
|  |  |  |  |  | Hajeeganj (Ma) | 27 | 7.76 | 3.14 |
|  |  |  |  |  | Isdair | 17 | 16.39 | 6.63 |
|  |  |  |  |  | Masdair | 28 | 15.16 | 6.13 |
|  |  |  |  |  | Mukraba | 1 | 1.26 | 0.51 |
|  |  |  |  |  | Narayanganj | 37 | 39.40 | 15.95 |
| 10 | Proposed Road | 387.47 | 156.80 | 7.92 | Bara Deobhog | 408 | 28.86 | 11.68 |
|  |  |  |  |  | Baraibhog | 65 | 6.66 | 2.69 |
|  |  |  |  |  | Bhagabanganj (Ma) | 88 | 2.03 | 0.82 |
|  |  |  |  |  | Chasara | 99 | 9.85 | 3.99 |
|  |  |  |  |  | Chasara (Ma) | 30 | 2.07 | 0.84 |
|  |  |  |  |  | Deobhog (Ma) | 56 | 4.13 | 1.67 |
|  |  |  |  |  | Enayetnagar | 24 | 0.79 | 0.32 |
|  |  |  |  |  | Fatulla | 305 | 56.23 | 22.76 |
|  |  |  |  |  | Goalbanda | 136 | 10.10 | 4.08 |
|  |  |  |  |  | Goalbanda (Ma) | 19 | 0.60 | 0.24 |
|  |  |  |  |  | Godnail | 118 | 24.83 | 10.05 |
|  |  |  |  |  | Godnail (Ma) | 31 | 5.96 | 2.41 |
|  |  |  |  |  | Hariharpara | 82 | 12.53 | 5.07 |
|  |  |  |  |  | Hajeeganj | 79 | 10.20 | 4.13 |
|  |  |  |  |  | Hajeeganj (Ma) | 26 | 4.61 | 1.87 |
|  |  |  |  |  | Isdair | 65 | 21.94 | 8.88 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Proposed Road |  |  |  | Kaempur | 98 | 8.36 | 3.38 |
|  |  |  |  |  | Kashipur | 183 | 4.86 | 1.97 |
|  |  |  |  |  | Khanpur | 267 | 44.73 | 18.10 |
|  |  |  |  |  | Lalpur | 62 | 6.49 | 2.63 |
|  |  |  |  |  | Masdair | 246 | 34.85 | 14.10 |
|  |  |  |  |  | Masinabanda | 79 | 2.74 | 1.11 |
|  |  |  |  |  | Mukraba | 24 | 1.71 | 0.69 |
|  |  |  |  |  | Narayanganj | 225 | 16.02 | 6.48 |
|  |  |  |  |  | Paikpara (Ma) | 171 | 7.32 | 2.96 |
|  |  |  |  |  | River | 13 | 0.39 | 0.16 |
|  |  |  |  |  | Saiyadpur | 379 | 18.34 | 7.42 |
|  |  |  |  |  | Saiyadpur (Ma) | 55 | 13.80 | 5.58 |
|  |  |  |  |  | Shitalakshya | 268 | 16.90 | 6.84 |
|  |  |  |  |  | Talla | 72 | 9.58 | 3.88 |
| 11 | Rural Settlement Zone | 63.99 | 25.89 | 1.31 | Masinabanda | 7 | 1.30 | 0.52 |
|  |  |  |  |  | Saiyadpur | 204 | 62.70 | 25.37 |
| 12 | Transport & Communication | 123.26 | 49.88 | 2.52 | Bag Benidas | 5 | 0.28 | 0.11 |
|  |  |  |  |  | Bara Deobhog | 21 | 4.13 | 1.67 |
|  |  |  |  |  | Baraibhog | 4 | 0.53 | 0.22 |
|  |  |  |  |  | Bhagabanganj (Ma) | 14 | 0.66 | 0.27 |
|  |  |  |  |  | Chasara | 37 | 6.07 | 2.46 |
|  |  |  |  |  | Chasara (Ma) | 32 | 6.48 | 2.62 |
|  |  |  |  |  | Deobhog (Ma) | 8 | 1.96 | 0.79 |
|  |  |  |  |  | Fatulla | 48 | 13.72 | 5.55 |
|  |  |  |  |  | Goalbanda | 19 | 2.64 | 1.07 |
|  |  |  |  |  | Goalbanda (Ma) | 2 | 0.08 | 0.03 |
|  |  |  |  |  | Godnail | 17 | 5.18 | 2.10 |
|  |  |  |  |  | Godnail (Ma) | 5 | 0.93 | 0.38 |
|  |  |  |  |  | Hariharpara | 3 | 0.12 | 0.05 |
|  |  |  |  |  | Hajeeganj | 16 | 2.10 | 0.85 |
|  |  |  |  |  | Hajeeganj (Ma) | 5 | 0.87 | 0.35 |
|  |  |  |  |  | Isdair | 36 | 13.76 | 5.57 |
|  |  |  |  |  | Kaempur | 10 | 1.03 | 0.42 |
|  |  |  |  |  | Kashipur | 5 | 0.46 | 0.18 |
|  |  |  |  |  | Khanpur | 120 | 16.75 | 6.78 |
|  |  |  |  |  | Lalpur | 14 | 2.78 | 1.13 |
|  |  |  |  |  | Masdair | 27 | 6.20 | 2.51 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Transport & Communication |  |  |  | Masinabanda | 8 | 1.02 | 0.41 |
|  |  |  |  |  | Masinabanda (Ma) | 2 | 0.40 | 0.16 |
|  |  |  |  |  | Mukraba | 3 | 1.22 | 0.49 |
|  |  |  |  |  | Narayanganj | 80 | 23.74 | 9.60 |
|  |  |  |  |  | Paikpara (Ma) | 32 | 2.07 | 0.83 |
|  |  |  |  |  | Saiyadpur | 4 | 0.61 | 0.25 |
|  |  |  |  |  | Saiyadpur (Ma) | 5 | 0.76 | 0.31 |
|  |  |  |  |  | Shitalakshya | 56 | 6.03 | 2.44 |
|  |  |  |  |  | Talla | 5 | 0.69 | 0.28 |
| 13 | Urban Residential Zone | 1974.16 | 798.91 | 40.34 | Bara Deobhog | 1859 | 315.76 | 127.78 |
|  |  |  |  |  | Baraibhog | 117 | 25.36 | 10.26 |
|  |  |  |  |  | Chasara | 206 | 33.14 | 13.41 |
|  |  |  |  |  | Chasara (Ma) | 139 | 22.37 | 9.05 |
|  |  |  |  |  | Deobhog (Ma) | 572 | 95.49 | 38.64 |
|  |  |  |  |  | Fatulla | 610 | 228.90 | 92.63 |
|  |  |  |  |  | Goalbanda | 604 | 135.64 | 54.89 |
|  |  |  |  |  | Goalbanda (Ma) | 69 | 19.91 | 8.06 |
|  |  |  |  |  | Hariharpara | 5 | 0.92 | 0.37 |
|  |  |  |  |  | Hajeeganj | 17 | 4.05 | 1.64 |
|  |  |  |  |  | Isdair | 131 | 38.13 | 15.43 |
|  |  |  |  |  | Kaempur | 85 | 18.82 | 7.62 |
|  |  |  |  |  | Kashipur | 519 | 94.33 | 38.17 |
|  |  |  |  |  | Khanpur | 980 | 119.91 | 48.52 |
|  |  |  |  |  | Lalpur | 7 | 1.47 | 0.59 |
|  |  |  |  |  | Masdair | 866 | 239.36 | 96.87 |
|  |  |  |  |  | Masinabanda | 380 | 67.12 | 27.16 |
|  |  |  |  |  | Masinabanda (Ma) | 129 | 22.72 | 9.20 |
|  |  |  |  |  | Mukraba | 130 | 19.64 | 7.95 |
|  |  |  |  |  | Narayanganj | 144 | 22.06 | 8.93 |
|  |  |  |  |  | Paikpara (Ma) | 518 | 99.52 | 40.27 |
|  |  |  |  |  | Saiyadpur | 749 | 172.33 | 69.74 |
|  |  |  |  |  | Saiyadpur (Ma) | 24 | 17.75 | 7.18 |
|  |  |  |  |  | Shitalakhya | 653 | 81.13 | 32.83 |
|  |  |  |  |  | Talla | 284 | 78.34 | 31.70 |
| 14 | Water Retention Area | 32.31 | 13.08 | 0.66 | Fatulla | 78 | 3.28 | 1.33 |
|  |  |  |  |  | Godnail | 8 | 14.95 | 6.05 |
|  |  |  |  |  | Hajeeganj | 48 | 3.23 | 1.31 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Water Retention Area |  |  |  | Isdair | 17 | 1.43 | 0.58 |
|  |  |  |  |  | Kaempur | 44 | 3.17 | 1.28 |
|  |  |  |  |  | Khanpur | 49 | 4.29 | 1.74 |
|  |  |  |  |  | Mukraba | 14 | 1.17 | 0.47 |
|  |  |  |  |  | Talla | 20 | 0.78 | 0.32 |
| 15 | Waterbody | 382.18 | 154.67 | 7.81 | Bara Deobhog | 20 | 12.06 | 4.88 |
|  |  |  |  |  | Baraibhog | 5 | 1.10 | 0.44 |
|  |  |  |  |  | Bhagabanganj (Ma) | 16 | 0.04 | 0.02 |
|  |  |  |  |  | Chasara | 1 | 0.85 | 0.34 |
|  |  |  |  |  | Chasara (Ma) | 1 | 1.15 | 0.47 |
|  |  |  |  |  | Deobhog (Ma) | 6 | 4.52 | 1.83 |
|  |  |  |  |  | Enayetnagar | 2 | 0.82 | 0.33 |
|  |  |  |  |  | Fatulla | 4 | 0.05 | 0.02 |
|  |  |  |  |  | Goalbanda | 9 | 1.46 | 0.59 |
|  |  |  |  |  | Godnail | 9 | 150.31 | 60.83 |
|  |  |  |  |  | Godnail (Ma) | 6 | 0.72 | 0.29 |
|  |  |  |  |  | Hariharpara | 10 | 4.67 | 1.89 |
|  |  |  |  |  | Hajeeganj | 6 | 0.24 | 0.10 |
|  |  |  |  |  | Isdair | 1 | 1.04 | 0.42 |
|  |  |  |  |  | Kashipur | 10 | 9.85 | 3.99 |
|  |  |  |  |  | Khanpur | 16 | 11.49 | 4.65 |
|  |  |  |  |  | Masdair | 8 | 1.42 | 0.58 |
|  |  |  |  |  | Mukraba | 9 | 2.23 | 0.90 |
|  |  |  |  |  | Narayanganj | 43 | 9.99 | 4.04 |
|  |  |  |  |  | Paikpara (Ma) | 15 | 4.60 | 1.86 |
|  |  |  |  |  | River | 13 | 135.13 | 54.69 |
|  |  |  |  |  | Saiyadpur | 3 | 0.65 | 0.26 |
|  |  |  |  |  | Saiyadpur (Ma) | 2 | 0.99 | 0.40 |
|  |  |  |  |  | Shitalakshya | 35 | 24.80 | 10.04 |
|  |  |  |  |  | Talla | 11 | 2.01 | 0.81 |
| Total |  | 4893.46 | 1980.28 | 100 |  | 22031 | 4893.46 | 1980.28 |

The proposed road network of the DPZ 05 is also presented in the following Table 3.15.

##### Table - 3.15: Proposed New Roads of DPZ-05

| Road Name | Width (m.) | Length (Km.) |
| --- | --- | --- |
| Adamjee Nagar-Sayedpur River Front Road | 27.00 | 10.68 |
| Adamjee Road | 30.00 | 0.50 |

| Road Name | Width (m.) | Length (Km.) |
| --- | --- | --- |
| BSCIC Road | 18 | 0.61 |
| Bilashnagar-Talla Road | 27 | 0.89 |
| Chashara-Railway Gate no. 1 Road | 12 | 1.28 |
| Dapa Idrakpur-Godenail Road | 27 | 0.45 |
| Fatulla-Godenail Link Road | 30 | 3.48 |
| Godenail- - Talla Road | 18 | 0.76 |
| Gognagar-Sayedpur Road | 18 | 0.99 |
| -Chashara Railway Road | 30 | 2.01 |
| Issa Khan Road | 18 | 0.83 |
| Jalkundi-Godenail Road | 18 | 0.48 |
| Jamtala-Paikpara Road | 18 | 2.08 |
| Jurain-Chandmari Railway Road | 30 | 2.23 |
| Kashipur-Gogenagar Road | 18 | 2.58 |
| Kashipur-Narayanganj Road | 30 | 2.29 |
| Kashipur-Netaiganj-Kashipur Link Road | 18 | 3.51 |
| Kayempur Road | 18 | 0.64 |
| Khanpur Road | 30 | 0.50 |
| Lalpur-Panchabati Road | 18 | 1.21 |
| Masdair-Chandmari Road | 18 | 1.25 |
| Masdair-Kashipur Hatkhola Road | 30 | 3.15 |
| Naluapara -Sayedpur Road | 18 | 1.75 |
| Narayanganj Link Road | 85 | 1.90 |
| North Kashipur-West Deobhog Road | 18 | 1.31 |
| Old Dhaka-Narayanganj Road | 50 | 2.94 |
| Paikpara-Muslimnagar Road | 18 | 0.49 |
| Panchabati-Dharmaganj Road | 12 | 0.02 |
| Panchabati-Muktarpur Road | 50 | 0.02 |
| Salimullah Road | 18 | 0.60 |
| Sastapur Road | 18 | 0.57 |
| Sayedpur Road | 18 | 1.21 |
| Shashangaon-Masdair Road | 27 | 1.23 |
| Siachar-Godenail Road | 18 | 0.42 |
| Sirajuddaula Road | 12 | 1.01 |
| South Sastapur Road | 18 | 0.69 |
| Talla Road | 18 | 0.56 |
| Tiger Para-West Deobhog Road | 18 | 2.21 |
| West Deobhog Road | 18 | 0.46 |
| West Masdair Road | 18 | 0.55 |

Map No. 3.6 shows the landuse and road network of DPZ-05.

##### DPZ-6 Kadam Rasul Paurashava

Kadamrasul Paurashava has been created by separating it from the Narayanganj Paurashava. Kadam Rasul Paurashava stands on the east bank of Sitalakhya river. The first jute mill of the country was set up here, though it has now been closed and scraped. The only national level institution of the entire Group-B area: The Institute of Marine Technology is located here. Other important establishments are Sonakanda Dockyard, Haripur Power Station, several cotton textile and jute mills. The famous Kadam Rasul darga and the Shahi Masjid, a very old mosque of the region are located here. The zone including the paurashava area covers 3585.91 acres with projected population of 161572 for the year 2015.

Development pressure in the zone is low and there are pockets of undeveloped land in the area. However, development is of sprawl nature. A sizable population of the zone works in adjacent Narayanganj paurashava area by crossing the Sitalakhya reiver through Bandar and Nabiganj ferry ghats. The zone is a part of the Narayanganj old industrial belt. The river front of the zone is dotted with various industries. Encroachment in to Sitalakhya river is very common. The Sitalakhya river front stretching from Bandar area in the south to Kuripara in the north has been designated as General Industrial Zone. A substantial part of the zone has been designated as urban residential zone.

The historical site of the Sonakanda Fort area is proposed to be developed as a regional level tourist spot.

There has been encroachment in to the Sitalakhya river at many points, especially by a cement factory opposite the Barafkal BIWTA jetty. It is proposed that all structures encroached in to the river must be removed.

Proposed landuse for the zone is presented by the following Table 3.16:

##### Table - 3.16: Proposed Landuse of DPZ-06

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 1 | Agricultural Zone | 3.47 | 1.40 | 0.10 | Amirabad | 1 | 0.10 | 0.04 |
|  |  |  |  |  | Bandar | 8 | 0.19 | 0.08 |
|  |  |  |  |  | Gobindamandali | 3 | 0.10 | 0.04 |
|  |  |  |  |  | Hazipur | 3 | 0.41 | 0.17 |
|  |  |  |  |  | Lakshman Khola | 1 | 0.02 | 0.01 |
|  |  |  |  |  | Madanganj | 1 | 0.01 | 0.01 |
|  |  |  |  |  | Madhabpasa | 3 | 2.08 | 0.84 |
|  |  |  |  |  | Nabiganj | 5 | 0.21 | 0.08 |
|  |  |  |  |  | Satanga | 1 | 0.03 | 0.01 |
|  |  |  |  |  | Sonapur | 5 | 0.31 | 0.13 |
| 2 | Commercial Zone (Business) | 142.29 | 57.59 | 3.97 | Ayama Kadamrasul (Ma) | 117 | 12.88 | 5.21 |
|  |  |  |  |  | Bandar Krishnapura | 1 | 0.15 | 0.06 |
|  |  |  |  |  | Dariakandi (Ma) | 1 | 0.04 | 0.02 |
|  |  |  |  |  | Gobindamandali | 3 | 0.13 | 0.05 |
|  |  |  |  |  | Nabiganj | 56 | 12.14 | 4.91 |
|  |  |  |  |  | Nabiganj (Ma) | 571 | 109.39 | 44.28 |
|  |  |  |  |  | Uttar Bandar Krishnapura (Ma) | 23 | 7.58 | 3.06 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 3 | Flood Flow Zone | 87.96 | 35.60 | 2.45 | Gharmora | 12 | 4.25 | 1.72 |
|  |  |  |  |  | Madanganj | 36 | 83.66 | 33.86 |
|  |  |  |  |  | Madanganj (Ma) | 1 | 0.04 | 0.02 |
| 4 | General Industrial Zone | 519.55 | 210.28 | 14.49 | Amirabad | 62 | 10.58 | 4.29 |
|  |  |  |  |  | Ayama Kadamrasul (Ma) | 41 | 7.53 | 3.05 |
|  |  |  |  |  | Bag Nayanagar | 108 | 20.92 | 8.47 |
|  |  |  |  |  | Banga Sason | 30 | 7.59 | 3.07 |
|  |  |  |  |  | Dariakandi (Ma) | 58 | 16.63 | 6.73 |
|  |  |  |  |  | Dasergaon | 12 | 3.56 | 1.44 |
|  |  |  |  |  | Deuli | 8 | 15.70 | 6.35 |
|  |  |  |  |  | Dhamgar | 10 | 33.01 | 13.36 |
|  |  |  |  |  | Ekrampur (Ma) | 120 | 41.91 | 16.96 |
|  |  |  |  |  | Enatnagar | 40 | 31.83 | 12.88 |
|  |  |  |  |  | Enatnagar (Ma) | 4 | 0.04 | 0.02 |
|  |  |  |  |  | Fular | 13 | 3.08 | 1.25 |
|  |  |  |  |  | Gangakul Bandar (Ma) | 313 | 99.08 | 40.10 |
|  |  |  |  |  | Haripur | 36 | 8.34 | 3.38 |
|  |  |  |  |  | Lakshman Khola | 279 | 61.15 | 24.76 |
|  |  |  |  |  | Manahar Khaner Bag | 241 | 65.99 | 26.70 |
|  |  |  |  |  | Mridhabari | 38 | 8.39 | 3.39 |
|  |  |  |  |  | Nabiganj | 14 | 1.24 | 0.50 |
|  |  |  |  |  | Nabiganj (Ma) | 163 | 53.51 | 21.65 |
|  |  |  |  |  | Sonakanda (Ma) | 19 | 7.20 | 2.91 |
|  |  |  |  |  | Uttar Bandar Krishnapura (Ma) | 4 | 0.82 | 0.33 |
|  |  |  |  |  | Uttor Nanda | 61 | 21.46 | 8.68 |
| 5 | Heavy Industrial Zone | 164.71 | 66.65 | 4.59 | Bag Nayanagar | 2 | 0.22 | 0.09 |
|  |  |  |  |  | Banga Sason | 150 | 36.09 | 14.60 |
|  |  |  |  |  | Haripur | 43 | 57.76 | 23.38 |
|  |  |  |  |  | Manahar Khaner Bag | 96 | 25.42 | 10.28 |
|  |  |  |  |  | Mridhabari | 146 | 45.22 | 18.30 |
| 6 | Mixed Use Zone (Residential- Commercial) | 238.97 | 96.69 | 6.66 | Bag Nayanagar | 37 | 7.89 | 3.19 |
|  |  |  |  |  | Chanpatali | 8 | 0.67 | 0.27 |
|  |  |  |  |  | Dhamgar | 167 | 56.38 | 22.81 |
|  |  |  |  |  | Fular | 170 | 39.28 | 15.90 |
|  |  |  |  |  | Lakshman Khola | 593 | 134.75 | 54.52 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 7 | Mixed Use Zone (Residential- Commercial- General Industrial) | 527.31 | 213.41 | 14.71 | Enatnagar (Ma) | 6 | 0.59 | 0.24 |
|  |  |  |  |  | Madanganj | 343 | 76.15 | 30.82 |
|  |  |  |  |  | Madanganj (Ma) | 1492 | 435.18 | 176.12 |
|  |  |  |  |  | Namchar (Ma) | 38 | 8.59 | 3.48 |
|  |  |  |  |  | Noadha (Ma) | 10 | 6.80 | 2.75 |
| 8 | Overlay Zone | 8.98 | 3.64 | 0.25 | Enatnagar | 12 | 0.91 | 0.37 |
|  |  |  |  |  | Enatnagar (Ma) | 18 | 1.23 | 0.50 |
|  |  |  |  |  | Sonakanda (Ma) | 53 | 6.84 | 2.77 |
| 9 | Proposed Road | 431.89 | 174.78 | 12.04 | Amirabad | 87 | 8.23 | 3.33 |
|  |  |  |  |  | Ayama Kadamrasul (Ma) | 77 | 3.88 | 1.57 |
|  |  |  |  |  | Bag Nayanagar | 34 | 3.26 | 1.32 |
|  |  |  |  |  | Bandar | 11 | 5.85 | 2.37 |
|  |  |  |  |  | Bandar Krishnapura | 7 | 1.05 | 0.42 |
|  |  |  |  |  | Banga Sason | 53 | 4.86 | 1.97 |
|  |  |  |  |  | Chanpatali | 38 | 8.73 | 3.53 |
|  |  |  |  |  | Chhota Baraikhali | 9 | 7.15 | 2.89 |
|  |  |  |  |  | Dakshin Bandar Krishnapura (Ma) | 27 | 1.30 | 0.53 |
|  |  |  |  |  | Dariakandi (Ma) | 43 | 3.34 | 1.35 |
|  |  |  |  |  | Dasergaon | 57 | 7.41 | 3.00 |
|  |  |  |  |  | Deuli | 107 | 19.09 | 7.73 |
|  |  |  |  |  | Dhamgar | 202 | 43.20 | 17.49 |
|  |  |  |  |  | Ekrampur (Ma) | 32 | 5.22 | 2.12 |
|  |  |  |  |  | Enatnagar | 27 | 5.81 | 2.35 |
|  |  |  |  |  | Enatnagar (Ma) | 68 | 3.40 | 1.38 |
|  |  |  |  |  | Fular | 73 | 26.69 | 10.80 |
|  |  |  |  |  | Gangakul Bandar (Ma) | 178 | 13.92 | 5.63 |
|  |  |  |  |  | Gharmora | 1 | 0.46 | 0.19 |
|  |  |  |  |  | Gobindamandali | 30 | 11.55 | 4.67 |
|  |  |  |  |  | Haripur | 37 | 8.30 | 3.36 |
|  |  |  |  |  | Hazipur | 37 | 16.33 | 6.61 |
|  |  |  |  |  | Kalyandi | 10 | 1.28 | 0.52 |
|  |  |  |  |  | Kuripara | 166 | 28.18 | 11.40 |
|  |  |  |  |  | Kusiara | 6 | 3.31 | 1.34 |
|  |  |  |  |  | Lakshman Khola | 413 | 52.82 | 21.37 |
|  |  |  |  |  | Madanganj | 98 | 5.82 | 2.36 |
|  |  |  |  |  | Madanganj (Ma) | 188 | 18.09 | 7.32 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Proposed Road |  |  |  | Madhabpasa | 43 | 13.40 | 5.42 |
|  |  |  |  |  | Manahar Khaner Bag | 61 | 15.81 | 6.40 |
|  |  |  |  |  | Mridhabari | 7 | 1.01 | 0.41 |
|  |  |  |  |  | Nabiganj | 97 | 27.40 | 11.09 |
|  |  |  |  |  | Nabiganj (Ma) | 318 | 29.83 | 12.07 |
|  |  |  |  |  | Namchar | 12 | 0.42 | 0.17 |
|  |  |  |  |  | Noadha (Ma) | 9 | 0.52 | 0.21 |
|  |  |  |  |  | Nondirgaon | 4 | 4.15 | 1.68 |
|  |  |  |  |  | River | 1 | 0.03 | 0.01 |
|  |  |  |  |  | Satanga | 7 | 2.63 | 1.07 |
|  |  |  |  |  | Sonakanda (Ma) | 39 | 4.38 | 1.77 |
|  |  |  |  |  | Sonapur | 22 | 5.71 | 2.31 |
|  |  |  |  |  | Uttar Bandar Krishnapura (Ma) | 29 | 2.22 | 0.90 |
|  |  |  |  |  | Uttar Chanpur | 3 | 0.78 | 0.32 |
|  |  |  |  |  | Uttor Nanda | 76 | 5.08 | 2.06 |
| 10 | Rural Settlement Zone | 10.10 | 4.08 | 0.28 | Madanganj | 4 | 5.70 | 2.31 |
|  |  |  |  |  | Nabiganj | 2 | 0.53 | 0.21 |
|  |  |  |  |  | Namchar | 18 | 2.83 | 1.14 |
|  |  |  |  |  | Nondirgaon | 6 | 1.05 | 0.42 |
| 11 | Transport & Communication | 63.70 | 25.77 | 1.78 | Amirabad | 9 | 4.10 | 1.66 |
|  |  |  |  |  | Ayama Kadamrasul (Ma) | 15 | 0.90 | 0.36 |
|  |  |  |  |  | Chanpatali | 9 | 1.75 | 0.71 |
|  |  |  |  |  | Chhota Baraikhali | 3 | 3.51 | 1.42 |
|  |  |  |  |  | Dakshin Bandar Krishnapura (Ma) | 12 | 0.83 | 0.34 |
|  |  |  |  |  | Dariakandi (Ma) | 8 | 0.27 | 0.11 |
|  |  |  |  |  | Deuli | 11 | 4.55 | 1.84 |
|  |  |  |  |  | Dhamgar | 15 | 2.20 | 0.89 |
|  |  |  |  |  | Ekrampur (Ma) | 9 | 1.08 | 0.44 |
|  |  |  |  |  | Enatnagar | 10 | 0.70 | 0.28 |
|  |  |  |  |  | Enatnagar (Ma) | 4 | 0.48 | 0.19 |
|  |  |  |  |  | Fular | 5 | 0.12 | 0.05 |
|  |  |  |  |  | Gangakul Bandar (Ma) | 48 | 3.70 | 1.50 |
|  |  |  |  |  | Gharmora | 1 | 0.49 | 0.20 |
|  |  |  |  |  | Gobindamandali | 5 | 0.22 | 0.09 |
|  |  |  |  |  | Haripur | 4 | 0.94 | 0.38 |
|  |  |  |  |  | Kuripara | 8 | 2.11 | 0.85 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Transport & Communication |  |  |  | Kusiara | 5 | 3.30 | 1.34 |
|  |  |  |  |  | Lakshman Khola | 26 | 2.13 | 0.86 |
|  |  |  |  |  | Madanganj | 5 | 1.08 | 0.44 |
|  |  |  |  |  | Madanganj (Ma) | 50 | 10.13 | 4.10 |
|  |  |  |  |  | Madhabpasa | 2 | 0.07 | 0.03 |
|  |  |  |  |  | Nabiganj | 27 | 4.65 | 1.88 |
|  |  |  |  |  | Nabiganj (Ma) | 26 | 2.09 | 0.84 |
|  |  |  |  |  | Namchar | 3 | 0.08 | 0.03 |
|  |  |  |  |  | Noadha (Ma) | 3 | 0.85 | 0.34 |
|  |  |  |  |  | Nondirgaon | 5 | 4.69 | 1.90 |
|  |  |  |  |  | Satanga | 3 | 4.05 | 1.64 |
|  |  |  |  |  | Sonakanda (Ma) | 7 | 0.39 | 0.16 |
|  |  |  |  |  | Sonapur | 7 | 1.11 | 0.45 |
|  |  |  |  |  | Sonapur (Ma) | 1 | 0.05 | 0.02 |
|  |  |  |  |  | Uttar Bandar Krishnapura (Ma) | 2 | 0.08 | 0.03 |
|  |  |  |  |  | Uttar Chanpur | 1 | 0.06 | 0.03 |
|  |  |  |  |  | Uttor Nanda | 6 | 0.97 | 0.39 |
| 12 | Urban Residential Zone | 908.38 | 367.61 | 25.33 | Amirabad | 147 | 30.82 | 12.48 |
|  |  |  |  |  | Bag Nayanagar | 5 | 0.57 | 0.23 |
|  |  |  |  |  | Bandar | 4 | 1.32 | 0.53 |
|  |  |  |  |  | Bandar Krishnapura | 136 | 46.77 | 18.92 |
|  |  |  |  |  | Chanpatali | 92 | 21.06 | 8.52 |
|  |  |  |  |  | Chhota Baraikhali | 1 | 0.04 | 0.01 |
|  |  |  |  |  | Dakshin Bandar Krishnapura (Ma) | 150 | 21.90 | 8.87 |
|  |  |  |  |  | Dariakandi | 22 | 2.86 | 1.16 |
|  |  |  |  |  | Dariakandi (Ma) | 51 | 13.16 | 5.33 |
|  |  |  |  |  | Dasergaon | 185 | 43.42 | 17.57 |
|  |  |  |  |  | Deuli | 284 | 50.20 | 20.31 |
|  |  |  |  |  | Dhamgar | 340 | 70.94 | 28.69 |
|  |  |  |  |  | Ekrampur (Ma) | 9 | 1.62 | 0.65 |
|  |  |  |  |  | Enatnagar | 107 | 24.34 | 9.85 |
|  |  |  |  |  | Enatnagar (Ma) | 164 | 20.93 | 8.47 |
|  |  |  |  |  | Fular | 4 | 0.15 | 0.06 |
|  |  |  |  |  | Gangakul Bandar | 32 | 8.30 | 3.36 |
|  |  |  |  |  | Gangakul Bandar (Ma) | 483 | 53.11 | 21.49 |
|  |  |  |  |  | Gobindamandali | 189 | 53.55 | 21.67 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Urban Residential Zone |  |  |  | Haripur | 59 | 12.40 | 5.02 |
|  |  |  |  |  | Hazipur | 159 | 53.35 | 21.60 |
|  |  |  |  |  | Hazipur (Ma) | 18 | 3.28 | 1.33 |
|  |  |  |  |  | Kuripara | 582 | 106.09 | 42.94 |
|  |  |  |  |  | Kusiara | 2 | 0.27 | 0.11 |
|  |  |  |  |  | Lakshman Khola | 292 | 59.74 | 24.18 |
|  |  |  |  |  | Madanganj | 149 | 44.42 | 17.98 |
|  |  |  |  |  | Madanganj (Ma) | 120 | 22.64 | 9.16 |
|  |  |  |  |  | Madhabpasa | 66 | 21.96 | 8.88 |
|  |  |  |  |  | Nabiganj | 228 | 36.38 | 14.72 |
|  |  |  |  |  | Nabiganj (Ma) | 150 | 11.87 | 4.80 |
|  |  |  |  |  | Nondirgaon | 57 | 15.70 | 6.36 |
|  |  |  |  |  | Satanga | 1 | 0.24 | 0.10 |
|  |  |  |  |  | Sonapur | 227 | 35.74 | 14.47 |
|  |  |  |  |  | Sonapur (Ma) | 54 | 4.40 | 1.78 |
|  |  |  |  |  | Uttar Bandar Krishnapura (Ma) | 22 | 2.47 | 1.00 |
|  |  |  |  |  | Uttor Nanda | 91 | 12.42 | 5.02 |
| 13 | Waterbody | 478.61 | 193.69 | 13.35 | Bag Nayanagar | 5 | 1.04 | 0.42 |
|  |  |  |  |  | Bandar | 2 | 0.17 | 0.07 |
|  |  |  |  |  | Banga Sason | 1 | 1.01 | 0.41 |
|  |  |  |  |  | Chanpatali | 2 | 0.47 | 0.19 |
|  |  |  |  |  | Dakshin Bandar Krishnapura (Ma) | 2 | 0.36 | 0.14 |
|  |  |  |  |  | Dariakandi (Ma) | 8 | 1.14 | 0.46 |
|  |  |  |  |  | Deuli | 6 | 0.33 | 0.13 |
|  |  |  |  |  | Dhamgar | 16 | 0.29 | 0.12 |
|  |  |  |  |  | Enatnagar | 9 | 1.75 | 0.71 |
|  |  |  |  |  | Enatnagar (Ma) | 4 | 1.06 | 0.43 |
|  |  |  |  |  | Fular | 3 | 1.80 | 0.73 |
|  |  |  |  |  | Gangakul Bandar (Ma) | 11 | 2.20 | 0.89 |
|  |  |  |  |  | Gobindamandali | 4 | 1.44 | 0.58 |
|  |  |  |  |  | Haripur | 7 | 3.54 | 1.43 |
|  |  |  |  |  | Hazipur | 2 | 0.98 | 0.40 |
|  |  |  |  |  | Kuripara | 7 | 2.59 | 1.05 |
|  |  |  |  |  | Lakshman Khola | 40 | 3.88 | 1.57 |
|  |  |  |  |  | Madanganj | 14 | 24.78 | 10.03 |
|  |  |  |  |  | Madanganj (Ma) | 40 | 26.17 | 10.59 |
|  |  |  |  |  | Madhabpasa | 2 | 1.81 | 0.73 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Waterbody |  |  |  | Manahar Khaner Bag | 2 | 1.91 | 0.77 |
|  |  |  |  |  | Mridhabari | 1 | 0.28 | 0.11 |
|  |  |  |  |  | Nabiganj | 4 | 0.57 | 0.23 |
|  |  |  |  |  | Nabiganj (Ma) | 15 | 4.24 | 1.72 |
|  |  |  |  |  | Namchar (Ma) | 12 | 392.05 | 158.66 |
|  |  |  |  |  | Sonakanda (Ma) | 10 | 2.04 | 0.83 |
|  |  |  |  |  | Sonapur | 5 | 0.67 | 0.27 |
|  |  |  |  |  | Sonapur (Ma) | 1 | 0.05 | 0.02 |
|  |  |  |  |  | Uttor Nanda | 10 | 0.01 | 0.01 |
| Total |  | 3585.91 | 1451.17 | 100 |  | 14066 | 3585.91 | 1451.17 |

To provide connection with the North-south spine road (Road connecting Chittagong Road with Madanganj in the south built on the now abandoned Narsingdi-Madanganj railway tract) several east west road has been proposed. Two bridges, one in Nabiganj and the other in Sayedpur area to facilitate river crossing of the people has been proposed too. A new road connecting the Dhaka-Chittagong Road in the Sonargaon area to by pass the central area of both Narayanganj and Kadam Rasul Paurashava is included in the new road proposal.

The proposed road network of the DPZ 06 is also presented in the following Table 3.17.

##### Table - 3.17: Proposed New Roads of DPZ-06

| Road Name | Width (m.) | Length (Km.) |
| --- | --- | --- |
| Amirabad - Nabiganj Road | 18 | 0.51 |
| Amoir - Gharmora Road | 30 | 0.00 |
| Bandar Bazar - Koratpara Road | 18 | 1.33 |
| Deuli - Barapara Road | 30 | 0.71 |
| Dhaka-Chittagong Highway | 85 | 1.09 |
| Dhakeshwari road | 18 | 0.62 |
| Dhamgar - Madanganj Road | 18 | 8.23 |
| Ekrampur - Chota Baraikhali Road | 30 | 1.13 |
| Heavy Industry Road | 30 | 1.12 |
| Ispahani Bazar Road | 18 | 0.33 |
| Kanchpur - Madanganj Road | 27 | 11.57 |
| Kanchpur - Chapatali Road | 27 | 1.19 |
| Kuripara - Gokuldaserbag Road | 18 | 0.54 |
| Kuripara - Malibag Road | 30 | 0.47 |
| Lakshmankhola - Langalband Bazar Road | 30 | 0.05 |
| Laksmankhola Road 1 | 18 | 0.75 |
| Laksmankhola Road 2 | 18 | 0.92 |
| Laksmankhola Road 3 | 18 | 0.66 |

| Road Name | Width (m.) | Length (Km.) |
| --- | --- | --- |
| Langalband - Kalagachia Bazar - Farajikanda Road | 30 | 0.82 |
| Madanganj - Kabilganj Road | 27 | 1.08 |
| Madanpur - Madanganj Road | 85 | 10.78 |
| Nabiganj - Kaikertek - Sonargaon Road | 30 | 0.73 |
| Nabiganj Road | 18 | 0.91 |
| North Laksmankhola Road | 18 | 0.70 |
| Sonakanda - Mirkandi Road | 30 | 1.21 |

Map No. 3.7 shows the landuse and road network in DPZ-06.

##### DPZ-07 Bandar-Sonargaon

Detailed Planning Zone-07 is constituted with Bandar, Kalagachia, Musapur, Madanpur, Dhamgar, Mograpara, Sadipur, Aminpur, Pirizpur and Shambhupura unions. The total population for this area was 189910 in the year 2001. The projected population of the area is 343870 for the year 2015 that shows an increase of 153960 people.

The area is generally rural in character and has been designated in the higher level plan as Agricultural Zone. In order to maintain the present character of the zone any development other than the infrastructure to support the agricultural practices has been prohibited.

The Langalbandh area, which is an old Pilgrimage Place of the Hindu community is to be developed as a religio-cultural center to enable perform the religious rites properly and safely.

Any development along Langalbandh-Minarbari road and along the Road connecting Nabiganj with Chittagong road at Sonargaon via Kaikertek will be discouraged.

To ensure food security rural settlements will not be allowed to be extended in the agricultural designated land.

Proposed landuse and road network is shown in Map 3.8 and statistics of landuse is presented in the following Table 3.18.

##### Table - 3.18: Proposed Landuse of DPZ-07

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
| 1 | Agricultural Zone | 12122.59 | 4905.84 | 66.92 | Alapdi | 60 | 10.50 | 4.25 |
|  |  |  |  |  | Amoir | 825 | 192.72 | 77.99 |
|  |  |  |  |  | Asrafdi | 262 | 83.59 | 33.83 |
|  |  |  |  |  | Babrakpur | 12 | 2.64 | 1.07 |
|  |  |  |  |  | Bag Nazirpur | 43 | 16.39 | 6.63 |
|  |  |  |  |  | Bagbari | 46 | 15.52 | 6.28 |
|  |  |  |  |  | Baler Chak | 202 | 50.11 | 20.27 |
|  |  |  |  |  | Balia | 52 | 12.43 | 5.03 |
|  |  |  |  |  | Baligaon | 237 | 55.68 | 22.54 |
|  |  |  |  |  | Bandar | 423 | 100.88 | 40.82 |
|  |  |  |  |  | Bandara | 113 | 28.93 | 11.72 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Agricultural Zone |  |  |  | Bara Baraikhali | 181 | 55.90 | 22.62 |
|  |  |  |  |  | Bara Bibi Khalisa | 50 | 16.85 | 6.82 |
|  |  |  |  |  | Bara Sadipur | 90 | 23.52 | 9.52 |
|  |  |  |  |  | Baranagar | 33 | 10.33 | 4.18 |
|  |  |  |  |  | Barapara | 2337 | 587.35 | 237.70 |
|  |  |  |  |  | Barimajlis | 10 | 1.82 | 0.73 |
|  |  |  |  |  | Barundi | 352 | 117.18 | 47.43 |
|  |  |  |  |  | Beghaiakandi | 74 | 17.92 | 7.25 |
|  |  |  |  |  | Bejergaon | 191 | 53.61 | 21.69 |
|  |  |  |  |  | Bhagalpur | 100 | 17.73 | 7.18 |
|  |  |  |  |  | Bhairabdi | 238 | 64.53 | 26.11 |
|  |  |  |  |  | Bhajandi | 91 | 27.23 | 11.02 |
|  |  |  |  |  | Bhatipara | 43 | 9.59 | 3.88 |
|  |  |  |  |  | Bhattacharjya Chak | 108 | 33.86 | 13.70 |
|  |  |  |  |  | Bhiticandi | 135 | 33.18 | 13.43 |
|  |  |  |  |  | Bibijhora | 161 | 35.20 | 14.25 |
|  |  |  |  |  | Bil Boroya | 23 | 6.81 | 2.76 |
|  |  |  |  |  | Binnipara | 75 | 9.80 | 3.97 |
|  |  |  |  |  | Birahmerkandi | 307 | 76.81 | 31.09 |
|  |  |  |  |  | Chairdona | 112 | 35.28 | 14.28 |
|  |  |  |  |  | Chak Baraia | 58 | 16.15 | 6.54 |
|  |  |  |  |  | Chak Karamulla | 40 | 12.56 | 5.08 |
|  |  |  |  |  | Chanpatali | 501 | 102.97 | 41.67 |
|  |  |  |  |  | Char Arjundi | 125 | 50.66 | 20.50 |
|  |  |  |  |  | Char Elahinagar | 293 | 64.11 | 25.95 |
|  |  |  |  |  | Char Fatepur | 42 | 6.83 | 2.76 |
|  |  |  |  |  | Char Safikhan | 301 | 77.49 | 31.35 |
|  |  |  |  |  | Chargoaldi | 345 | 84.06 | 34.02 |
|  |  |  |  |  | Chaudhurigaon | 409 | 100.66 | 40.73 |
|  |  |  |  |  | Chelar Char | 218 | 59.16 | 23.94 |
|  |  |  |  |  | Chhota Bandara | 18 | 6.22 | 2.51 |
|  |  |  |  |  | Chhota Baraikhali | 162 | 65.67 | 26.57 |
|  |  |  |  |  | Chhota Krishnadi | 105 | 27.51 | 11.13 |
|  |  |  |  |  | Chhota Majher Char | 222 | 62.22 | 25.19 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Agricultural Zone |  |  |  | Chhota Sadipur | 56 | 12.61 | 5.11 |
|  |  |  |  |  | Chhota Silmandi | 136 | 36.30 | 14.69 |
|  |  |  |  |  | Chilarbag | 56 | 23.74 | 9.60 |
|  |  |  |  |  | Chinardi | 230 | 78.01 | 31.57 |
|  |  |  |  |  | Dakshin Chandpur | 229 | 63.84 | 25.83 |
|  |  |  |  |  | Dakshin Char | 84 | 31.59 | 12.78 |
|  |  |  |  |  | Dakshin Kulcharitra | 562 | 144.02 | 58.27 |
|  |  |  |  |  | Dakshln Chandibardi | 46 | 13.35 | 5.41 |
|  |  |  |  |  | Darugola | 27 | 9.10 | 3.68 |
|  |  |  |  |  | Dasergaon | 201 | 52.39 | 21.20 |
|  |  |  |  |  | Dashdona | 150 | 38.07 | 15.40 |
|  |  |  |  |  | Decree Char | 66 | 16.63 | 6.73 |
|  |  |  |  |  | Deuli | 71 | 20.85 | 8.44 |
|  |  |  |  |  | Dhamgar | 1867 | 597.82 | 241.91 |
|  |  |  |  |  | Dighaldi | 413 | 138.81 | 56.17 |
|  |  |  |  |  | Dudhghata | 1063 | 271.85 | 110.02 |
|  |  |  |  |  | Durgaprasad Kandi | 227 | 48.35 | 19.56 |
|  |  |  |  |  | Ekrampur | 521 | 148.56 | 60.12 |
|  |  |  |  |  | Fardi | 22 | 3.42 | 1.38 |
|  |  |  |  |  | Fular | 332 | 89.72 | 36.31 |
|  |  |  |  |  | Gazariapar | 651 | 198.29 | 80.24 |
|  |  |  |  |  | Gharmora | 144 | 43.96 | 17.79 |
|  |  |  |  |  | Ghorir Chak | 51 | 11.64 | 4.71 |
|  |  |  |  |  | Ghosher Chak | 144 | 42.32 | 17.12 |
|  |  |  |  |  | Giribardi | 54 | 12.15 | 4.91 |
|  |  |  |  |  | Goaldi | 623 | 213.85 | 86.55 |
|  |  |  |  |  | Gobindakul | 679 | 204.63 | 82.79 |
|  |  |  |  |  | Gonpara | 103 | 32.05 | 12.97 |
|  |  |  |  |  | Gudaraghat | 288 | 77.31 | 31.28 |
|  |  |  |  |  | Habibpur | 82 | 20.11 | 8.13 |
|  |  |  |  |  | Hardi | 126 | 37.85 | 15.33 |
|  |  |  |  |  | Hazipur | 88 | 24.51 | 9.92 |
|  |  |  |  |  | Hazradi | 162 | 38.17 | 15.44 |
|  |  |  |  |  | Hosenpur | 207 | 77.81 | 31.48 |
|  |  |  |  |  | Hosenpur Chak | 83 | 27.23 | 11.02 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Agricultural Zone |  |  |  | Imamdir Chhat | 30 | 7.14 | 2.89 |
|  |  |  |  |  | Itkhola | 141 | 40.12 | 16.23 |
|  |  |  |  |  | Jagatdi | 44 | 11.79 | 4.77 |
|  |  |  |  |  | Jahgrakhola | 59 | 18.29 | 7.40 |
|  |  |  |  |  | Jangal | 1259 | 296.73 | 120.07 |
|  |  |  |  |  | Jhau Char | 110 | 26.24 | 10.62 |
|  |  |  |  |  | Jiudhara | 185 | 65.62 | 26.55 |
|  |  |  |  |  | Kabilganj | 164 | 20.49 | 8.29 |
|  |  |  |  |  | Kafurdi | 109 | 27.93 | 11.31 |
|  |  |  |  |  | Kajirgaon | 56 | 10.89 | 4.41 |
|  |  |  |  |  | Kakair Tek | 7 | 5.36 | 2.17 |
|  |  |  |  |  | Kaliganj | 294 | 81.30 | 32.90 |
|  |  |  |  |  | Kalyandi | 178 | 49.68 | 20.11 |
|  |  |  |  |  | Kamargaon | 81 | 26.84 | 10.86 |
|  |  |  |  |  | Kamtal | 791 | 204.77 | 82.87 |
|  |  |  |  |  | Keudhala | 2949 | 757.50 | 306.56 |
|  |  |  |  |  | Kuchiamora | 774 | 185.02 | 74.87 |
|  |  |  |  |  | Kulcharitra | 257 | 86.84 | 35.14 |
|  |  |  |  |  | Kuripara | 201 | 50.79 | 20.56 |
|  |  |  |  |  | Kusasan | 59 | 20.89 | 8.46 |
|  |  |  |  |  | Kusiara | 1244 | 433.11 | 175.27 |
|  |  |  |  |  | Lakshman Khola | 461 | 129.23 | 52.30 |
|  |  |  |  |  | Lal Alapdi | 45 | 14.56 | 5.89 |
|  |  |  |  |  | Lal Krishnadi | 42 | 13.59 | 5.50 |
|  |  |  |  |  | Lam Chariton | 303 | 77.93 | 31.54 |
|  |  |  |  |  | Lambadardi | 131 | 42.85 | 17.34 |
|  |  |  |  |  | Madanpur | 494 | 144.86 | 58.62 |
|  |  |  |  |  | Madhabpasa | 1116 | 363.26 | 147.03 |
|  |  |  |  |  | Madhabpur | 108 | 33.70 | 13.63 |
|  |  |  |  |  | Maghbazar | 111 | 31.30 | 12.66 |
|  |  |  |  |  | Majguni | 42 | 11.05 | 4.47 |
|  |  |  |  |  | Majidbari | 54 | 14.32 | 5.80 |
|  |  |  |  |  | Malikerpara | 47 | 9.97 | 4.03 |
|  |  |  |  |  | Manahar Khaner Bag | 257 | 68.37 | 27.68 |
|  |  |  |  |  | Mangolergaon | 562 | 143.04 | 57.89 |
|  |  |  |  |  | Menikhali | 255 | 42.94 | 17.39 |
|  |  |  |  |  | Mirbaharkandi | 5 | 1.54 | 0.62 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Agricultural Zone |  |  |  | Mirer Char | 282 | 76.23 | 30.85 |
|  |  |  |  |  | Mirkundi | 302 | 74.97 | 30.35 |
|  |  |  |  |  | Mirkundi Char | 265 | 61.32 | 24.83 |
|  |  |  |  |  | Mithadi | 36 | 13.58 | 5.49 |
|  |  |  |  |  | Mithapur | 30 | 7.05 | 2.86 |
|  |  |  |  |  | Mohanpur | 198 | 73.54 | 29.77 |
|  |  |  |  |  | Mohiskhola | 180 | 56.19 | 22.74 |
|  |  |  |  |  | Mugar Char | 547 | 115.73 | 46.84 |
|  |  |  |  |  | Mukfaldi | 329 | 115.06 | 46.57 |
|  |  |  |  |  | Mukteshpur | 61 | 18.69 | 7.56 |
|  |  |  |  |  | Musapur | 292 | 69.66 | 28.19 |
|  |  |  |  |  | Nabiganj | 68 | 19.26 | 7.80 |
|  |  |  |  |  | Nabinagar | 289 | 71.04 | 28.75 |
|  |  |  |  |  | Nabuchhra | 20 | 4.03 | 1.63 |
|  |  |  |  |  | Nangalban | 418 | 70.28 | 28.44 |
|  |  |  |  |  | Narayandiya | 108 | 31.12 | 12.60 |
|  |  |  |  |  | Nayachargobi ndopur | 22 | 5.29 | 2.14 |
|  |  |  |  |  | Nisang | 237 | 64.73 | 26.19 |
|  |  |  |  |  | Nurpaddi | 201 | 59.94 | 24.27 |
|  |  |  |  |  | Pachcani | 56 | 10.55 | 4.27 |
|  |  |  |  |  | Padmalabhdi | 31 | 5.33 | 2.15 |
|  |  |  |  |  | Panchamighat | 1268 | 413.42 | 167.31 |
|  |  |  |  |  | Panchani | 12 | 2.01 | 0.81 |
|  |  |  |  |  | Pirijpur | 883 | 278.66 | 112.77 |
|  |  |  |  |  | Raahdi | 268 | 75.89 | 30.71 |
|  |  |  |  |  | Rahamatpur | 79 | 15.28 | 6.18 |
|  |  |  |  |  | Ramgabindag aon | 133 | 37.37 | 15.12 |
|  |  |  |  |  | Ratandi | 176 | 36.01 | 14.57 |
|  |  |  |  |  | Sabdi | 403 | 111.83 | 45.26 |
|  |  |  |  |  | Sachinapur | 83 | 29.44 | 11.91 |
|  |  |  |  |  | Salsardi | 209 | 68.36 | 27.67 |
|  |  |  |  |  | Sambhupura | 183 | 55.99 | 22.66 |
|  |  |  |  |  | Sarail | 181 | 68.94 | 27.90 |
|  |  |  |  |  | Satanga | 375 | 142.78 | 57.78 |
|  |  |  |  |  | Serpur | 48 | 15.61 | 6.32 |
|  |  |  |  |  | Shatarajdhi | 63 | 15.95 | 6.46 |
|  |  |  |  |  | Silmandi | 55 | 16.58 | 6.71 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Agricultural Zone |  |  |  | Sonakhali | 47 | 6.11 | 2.47 |
|  |  |  |  |  | Srinibasdi | 209 | 62.10 | 25.13 |
|  |  |  |  |  | Srirampur | 519 | 107.33 | 43.43 |
|  |  |  |  |  | Subahakardi | 71 | 29.04 | 11.75 |
|  |  |  |  |  | Sukardi | 58 | 17.19 | 6.96 |
|  |  |  |  |  | Tahirpur Kazirgaon | 234 | 41.51 | 16.80 |
|  |  |  |  |  | Tamodardi | 126 | 34.05 | 13.78 |
|  |  |  |  |  | Tartaria | 182 | 70.21 | 28.42 |
|  |  |  |  |  | Tatuakandi | 100 | 27.47 | 11.12 |
|  |  |  |  |  | Terakahania | 72 | 32.44 | 13.13 |
|  |  |  |  |  | Tindona | 43 | 15.77 | 6.39 |
|  |  |  |  |  | Tingaon | 115 | 36.77 | 14.89 |
|  |  |  |  |  | Tipurdi | 195 | 51.65 | 20.91 |
|  |  |  |  |  | Ulukandi | 34 | 10.88 | 4.40 |
|  |  |  |  |  | Uttamdi | 46 | 14.25 | 5.77 |
|  |  |  |  |  | Uttar Chandibardi | 37 | 10.42 | 4.22 |
|  |  |  |  |  | Uttar Chanpur | 237 | 76.45 | 30.94 |
|  |  |  |  |  | Uttar Kulcharitra | 111 | 29.67 | 12.00 |
|  |  |  |  |  | Uttar Sabdi | 94 | 30.77 | 12.45 |
| 2 | Flood Flow Zone | 166.54 | 67.39 | 0.92 | Gharmora | 26 | 2.67 | 1.08 |
|  |  |  |  |  | Mohanpur | 162 | 53.69 | 21.73 |
|  |  |  |  |  | Nayachargobi ndopur | 13 | 4.41 | 1.79 |
|  |  |  |  |  | Nisang | 42 | 9.66 | 3.91 |
|  |  |  |  |  | Sambhupura | 127 | 92.66 | 37.50 |
|  |  |  |  |  | Subahakardi | 26 | 3.43 | 1.39 |
| 3 | Overlay Zone | 14.42 | 5.83 | 0.08 | Nangalban | 49 | 14.42 | 5.83 |
| 4 | Proposed Road | 643.21 | 260.25 | 3.55 | Alapdi | 16 | 1.80 | 0.73 |
|  |  |  |  |  | Amoir | 87 | 11.24 | 4.55 |
|  |  |  |  |  | Baler Chak | 2 | 0.20 | 0.08 |
|  |  |  |  |  | Balia | 24 | 2.23 | 0.90 |
|  |  |  |  |  | Baligaon | 65 | 7.50 | 3.04 |
|  |  |  |  |  | Bandar | 74 | 6.87 | 2.78 |
|  |  |  |  |  | Bara Bibi Khalisa | 16 | 2.12 | 0.86 |
|  |  |  |  |  | Baranagar | 47 | 5.31 | 2.15 |
|  |  |  |  |  | Barapara | 226 | 21.70 | 8.79 |
|  |  |  |  |  | Barimajlis | 25 | 7.53 | 3.05 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Proposed Road |  |  |  | Beghaiakandi | 22 | 2.28 | 0.92 |
|  |  |  |  |  | Bejergaon | 15 | 2.06 | 0.83 |
|  |  |  |  |  | Bhajandi | 8 | 0.77 | 0.31 |
|  |  |  |  |  | Bhatipara | 18 | 0.85 | 0.34 |
|  |  |  |  |  | Bibijhora | 58 | 4.57 | 1.85 |
|  |  |  |  |  | Binnipara | 12 | 1.12 | 0.45 |
|  |  |  |  |  | Chairdona | 1 | 0.99 | 0.40 |
|  |  |  |  |  | Chanpatali | 80 | 12.23 | 4.95 |
|  |  |  |  |  | Char Elahinagar | 4 | 2.21 | 0.90 |
|  |  |  |  |  | Char Safikhan | 27 | 6.35 | 2.57 |
|  |  |  |  |  | Chargoaldi | 14 | 1.25 | 0.51 |
|  |  |  |  |  | Chaudhurigaon | 4 | 0.81 | 0.33 |
|  |  |  |  |  | Chelar Char | 39 | 3.94 | 1.60 |
|  |  |  |  |  | Chhota Bandara | 9 | 4.88 | 1.97 |
|  |  |  |  |  | Chhota Baraikhali | 7 | 0.51 | 0.21 |
|  |  |  |  |  | Chhota Krishnadi | 17 | 4.20 | 1.70 |
|  |  |  |  |  | Chhota Sadipur | 16 | 4.05 | 1.64 |
|  |  |  |  |  | Chhota Silmandi | 33 | 5.77 | 2.33 |
|  |  |  |  |  | Chinardi | 77 | 7.96 | 3.22 |
|  |  |  |  |  | Dakshin Kulcharitra | 47 | 8.02 | 3.25 |
|  |  |  |  |  | Dasergaon | 53 | 6.54 | 2.65 |
|  |  |  |  |  | Deuli | 21 | 1.15 | 0.46 |
|  |  |  |  |  | Dhamgar | 196 | 20.81 | 8.42 |
|  |  |  |  |  | Dighaldi | 76 | 9.54 | 3.86 |
|  |  |  |  |  | Dudhghata | 53 | 6.17 | 2.50 |
|  |  |  |  |  | Durgaprasad Kandi | 96 | 9.07 | 3.67 |
|  |  |  |  |  | Fular | 91 | 12.79 | 5.18 |
|  |  |  |  |  | Gazariapar | 72 | 8.53 | 3.45 |
|  |  |  |  |  | Gharmora | 96 | 10.90 | 4.41 |
|  |  |  |  |  | Ghosher Chak | 14 | 2.28 | 0.92 |
|  |  |  |  |  | Giribardi | 21 | 4.16 | 1.68 |
|  |  |  |  |  | Goaldi | 34 | 4.60 | 1.87 |
|  |  |  |  |  | Gobindakul | 125 | 14.69 | 5.94 |
|  |  |  |  |  | Gudaraghat | 13 | 1.62 | 0.65 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Proposed Road |  |  |  | Habibpur | 88 | 13.99 | 5.66 |
|  |  |  |  |  | Hardi | 45 | 5.33 | 2.16 |
|  |  |  |  |  | Hazipur | 7 | 0.05 | 0.02 |
|  |  |  |  |  | Hazradi | 56 | 4.71 | 1.91 |
|  |  |  |  |  | Hosenpur | 1 | 0.00 | 0.00 |
|  |  |  |  |  | Itkhola | 24 | 2.07 | 0.83 |
|  |  |  |  |  | Jangal | 76 | 17.43 | 7.06 |
|  |  |  |  |  | Jhau Char | 8 | 1.31 | 0.53 |
|  |  |  |  |  | Jiudhara | 37 | 5.77 | 2.33 |
|  |  |  |  |  | Kabilganj | 33 | 2.83 | 1.14 |
|  |  |  |  |  | Kakair Tek | 5 | 0.72 | 0.29 |
|  |  |  |  |  | Kaliganj | 1 | 0.08 | 0.03 |
|  |  |  |  |  | Kalyandi | 95 | 8.89 | 3.59 |
|  |  |  |  |  | Kamtal | 60 | 6.43 | 2.60 |
|  |  |  |  |  | Keudhala | 169 | 40.30 | 16.31 |
|  |  |  |  |  | Kuripara | 62 | 6.10 | 2.47 |
|  |  |  |  |  | Kusiara | 171 | 24.07 | 9.75 |
|  |  |  |  |  | Lakshman Khola | 52 | 0.83 | 0.33 |
|  |  |  |  |  | Lal Alapdi | 28 | 3.00 | 1.22 |
|  |  |  |  |  | Lal Krishnadi | 40 | 4.42 | 1.79 |
|  |  |  |  |  | Lam Chariton | 13 | 1.01 | 0.41 |
|  |  |  |  |  | Lambadardi | 52 | 5.07 | 2.05 |
|  |  |  |  |  | Madanganj | 10 | 0.00 | 0.00 |
|  |  |  |  |  | Madanpur | 92 | 21.34 | 8.63 |
|  |  |  |  |  | Madhabpasa | 185 | 19.05 | 7.71 |
|  |  |  |  |  | Majidbari | 11 | 1.58 | 0.64 |
|  |  |  |  |  | Malikerpara | 35 | 6.88 | 2.79 |
|  |  |  |  |  | Manahar Khaner Bag | 27 | 3.15 | 1.28 |
|  |  |  |  |  | Mangolergaon | 49 | 4.27 | 1.73 |
|  |  |  |  |  | Menikhali | 63 | 4.64 | 1.88 |
|  |  |  |  |  | Mirer Char | 54 | 16.22 | 6.57 |
|  |  |  |  |  | Mirkundi | 52 | 3.93 | 1.59 |
|  |  |  |  |  | Mirkundi Char | 51 | 3.28 | 1.33 |
|  |  |  |  |  | Mugar Char | 31 | 2.66 | 1.08 |
|  |  |  |  |  | Mukfaldi | 40 | 5.14 | 2.08 |
|  |  |  |  |  | Nabiganj | 48 | 2.72 | 1.10 |
|  |  |  |  |  | Nabinagar | 57 | 5.71 | 2.31 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Proposed Road |  |  |  | Nangalban | 188 | 23.35 | 9.45 |
|  |  |  |  |  | Nisang | 96 | 8.04 | 3.25 |
|  |  |  |  |  | Nurpaddi | 25 | 2.43 | 0.98 |
|  |  |  |  |  | Padmalabhdi | 15 | 1.20 | 0.49 |
|  |  |  |  |  | Pirijpur | 31 | 16.03 | 6.49 |
|  |  |  |  |  | Ramgabindag aon | 25 | 1.72 | 0.70 |
|  |  |  |  |  | Ratandi | 52 | 12.27 | 4.97 |
|  |  |  |  |  | River | 1 | 0.94 | 0.38 |
|  |  |  |  |  | Sabdi | 88 | 8.24 | 3.34 |
|  |  |  |  |  | Salsardi | 11 | 1.33 | 0.54 |
|  |  |  |  |  | Sarail | 2 | 0.59 | 0.24 |
|  |  |  |  |  | Satanga | 7 | 0.89 | 0.36 |
|  |  |  |  |  | Sonakhali | 13 | 3.59 | 1.45 |
|  |  |  |  |  | Srirampur | 136 | 13.91 | 5.63 |
|  |  |  |  |  | Subahakardi | 37 | 3.68 | 1.49 |
|  |  |  |  |  | Tamodardi | 66 | 6.36 | 2.57 |
|  |  |  |  |  | Tartaria | 26 | 3.57 | 1.45 |
|  |  |  |  |  | Tatuakandi | 11 | 1.70 | 0.69 |
|  |  |  |  |  | Terakahania | 3 | 0.25 | 0.10 |
|  |  |  |  |  | Tingaon | 32 | 3.24 | 1.31 |
|  |  |  |  |  | Tipurdi | 29 | 4.10 | 1.66 |
|  |  |  |  |  | Uttar Chanpur | 61 | 16.66 | 6.74 |
| 5 | Rural Settlement Zone | 3706.64 | 1500.03 | 20.46 | Alapdi | 60 | 12.98 | 5.26 |
|  |  |  |  |  | Amoir | 86 | 17.96 | 7.27 |
|  |  |  |  |  | Asrafdi | 35 | 3.25 | 1.32 |
|  |  |  |  |  | Babrakpur | 26 | 6.77 | 2.74 |
|  |  |  |  |  | Bagbari | 17 | 4.54 | 1.84 |
|  |  |  |  |  | Baler Chak | 1 | 0.26 | 0.10 |
|  |  |  |  |  | Balia | 96 | 20.70 | 8.38 |
|  |  |  |  |  | Baligaon | 50 | 8.68 | 3.52 |
|  |  |  |  |  | Bandar | 410 | 89.72 | 36.32 |
|  |  |  |  |  | Bandara | 30 | 6.96 | 2.81 |
|  |  |  |  |  | Bara Baraikhali | 38 | 9.19 | 3.72 |
|  |  |  |  |  | Bara Sadipur | 2 | 0.63 | 0.25 |
|  |  |  |  |  | Baranagar | 377 | 78.32 | 31.70 |
|  |  |  |  |  | Barapara | 653 | 128.38 | 51.95 |
|  |  |  |  |  | Barimajlis | 105 | 51.40 | 20.80 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Rural Settlement Zone |  |  |  | Barundi | 97 | 22.39 | 9.06 |
|  |  |  |  |  | Bejergaon | 60 | 14.06 | 5.69 |
|  |  |  |  |  | Bhagalpur | 53 | 11.96 | 4.84 |
|  |  |  |  |  | Bhairabdi | 74 | 19.20 | 7.77 |
|  |  |  |  |  | Bhajandi | 48 | 8.77 | 3.55 |
|  |  |  |  |  | Bhatipara | 26 | 9.12 | 3.69 |
|  |  |  |  |  | Bhiticandi | 27 | 6.32 | 2.56 |
|  |  |  |  |  | Bibijhora | 131 | 24.47 | 9.90 |
|  |  |  |  |  | Binnipara | 18 | 4.35 | 1.76 |
|  |  |  |  |  | Birahmerkandi | 13 | 4.08 | 1.65 |
|  |  |  |  |  | Chairdona | 33 | 7.01 | 2.84 |
|  |  |  |  |  | Chanpatali | 226 | 56.12 | 22.72 |
|  |  |  |  |  | Char Elahinagar | 100 | 39.99 | 16.19 |
|  |  |  |  |  | Char Fatepur | 9 | 1.29 | 0.52 |
|  |  |  |  |  | Char Safikhan | 25 | 6.07 | 2.46 |
|  |  |  |  |  | Chargoaldi | 81 | 14.40 | 5.82 |
|  |  |  |  |  | Chaudhurigaon | 190 | 44.60 | 18.05 |
|  |  |  |  |  | Chelar Char | 65 | 11.21 | 4.54 |
|  |  |  |  |  | Chhota Baraikhali | 13 | 4.72 | 1.91 |
|  |  |  |  |  | Chhota Krishnadi | 20 | 5.42 | 2.19 |
|  |  |  |  |  | Chhota Majher Char | 12 | 2.38 | 0.96 |
|  |  |  |  |  | Chhota Sadipur | 38 | 14.97 | 6.06 |
|  |  |  |  |  | Chhota Silmandi | 20 | 4.00 | 1.62 |
|  |  |  |  |  | Chilarbag | 127 | 63.30 | 25.62 |
|  |  |  |  |  | Chinardi | 120 | 26.84 | 10.85 |
|  |  |  |  |  | Dakshin Chandpur | 38 | 10.28 | 4.16 |
|  |  |  |  |  | Dakshin Kulcharitra | 231 | 53.50 | 21.65 |
|  |  |  |  |  | Darugola | 41 | 9.71 | 3.93 |
|  |  |  |  |  | Dasergaon | 50 | 12.22 | 4.94 |
|  |  |  |  |  | Dashdona | 23 | 3.34 | 1.35 |
|  |  |  |  |  | Deuli | 21 | 8.35 | 3.38 |
|  |  |  |  |  | Dhamgar | 413 | 120.11 | 48.60 |
|  |  |  |  |  | Dighaldi | 25 | 12.07 | 4.89 |
|  |  |  |  |  | Dudhghata | 344 | 78.42 | 31.73 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Rural Settlement Zone |  |  |  | Durgaprasad Kandi | 121 | 20.14 | 8.16 |
|  |  |  |  |  | Ekrampur | 30 | 8.56 | 3.46 |
|  |  |  |  |  | Fardi | 37 | 7.22 | 2.92 |
|  |  |  |  |  | Fular | 39 | 5.84 | 2.36 |
|  |  |  |  |  | Gazariapar | 9 | 2.83 | 1.15 |
|  |  |  |  |  | Gharmora | 243 | 71.87 | 29.09 |
|  |  |  |  |  | Goaldi | 59 | 19.32 | 7.82 |
|  |  |  |  |  | Gobindakul | 141 | 36.01 | 14.57 |
|  |  |  |  |  | Gonpara | 49 | 16.24 | 6.57 |
|  |  |  |  |  | Habibpur | 194 | 39.68 | 16.06 |
|  |  |  |  |  | Hardi | 8 | 3.71 | 1.50 |
|  |  |  |  |  | Hazipur | 92 | 22.18 | 8.98 |
|  |  |  |  |  | Hazradi | 50 | 14.09 | 5.70 |
|  |  |  |  |  | Hosenpur | 84 | 20.99 | 8.50 |
|  |  |  |  |  | Jangal | 373 | 83.76 | 33.89 |
|  |  |  |  |  | Jhau Char | 18 | 4.02 | 1.63 |
|  |  |  |  |  | Jiudhara | 124 | 32.86 | 13.30 |
|  |  |  |  |  | Kabilganj | 116 | 12.63 | 5.11 |
|  |  |  |  |  | Kafurdi | 63 | 16.74 | 6.77 |
|  |  |  |  |  | Kajirgaon | 42 | 12.14 | 4.91 |
|  |  |  |  |  | Kaliganj | 145 | 26.51 | 10.73 |
|  |  |  |  |  | Kalyandi | 234 | 53.24 | 21.55 |
|  |  |  |  |  | Kamargaon | 16 | 9.93 | 4.02 |
|  |  |  |  |  | Kamtal | 164 | 47.49 | 19.21 |
|  |  |  |  |  | Keudhala | 866 | 188.56 | 76.30 |
|  |  |  |  |  | Kuchiamora | 220 | 52.98 | 21.44 |
|  |  |  |  |  | Kulcharitra | 13 | 3.86 | 1.56 |
|  |  |  |  |  | Kuripara | 66 | 20.21 | 8.18 |
|  |  |  |  |  | Kusasan | 4 | 1.97 | 0.80 |
|  |  |  |  |  | Kusiara | 594 | 206.74 | 83.66 |
|  |  |  |  |  | Lakshman Khola | 48 | 14.05 | 5.68 |
|  |  |  |  |  | Lal Alapdi | 47 | 10.19 | 4.13 |
|  |  |  |  |  | Lal Krishnadi | 2 | 0.74 | 0.30 |
|  |  |  |  |  | Lam Chariton | 63 | 12.13 | 4.91 |
|  |  |  |  |  | Lambadardi | 55 | 12.41 | 5.02 |
|  |  |  |  |  | Madanpur | 364 | 73.51 | 29.76 |
|  |  |  |  |  | Madhabpasa | 271 | 69.25 | 28.03 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Rural Settlement Zone |  |  |  | Madhabpur | 97 | 31.84 | 12.89 |
|  |  |  |  |  | Maghbazar | 61 | 11.01 | 4.45 |
|  |  |  |  |  | Majidbari | 75 | 25.78 | 10.43 |
|  |  |  |  |  | Malikerpara | 21 | 4.32 | 1.75 |
|  |  |  |  |  | Manahar Khaner Bag | 250 | 59.99 | 24.29 |
|  |  |  |  |  | Mangolergaon | 231 | 62.98 | 25.49 |
|  |  |  |  |  | Mirbaharkandi | 21 | 3.48 | 1.41 |
|  |  |  |  |  | Mirer Char | 279 | 43.52 | 17.60 |
|  |  |  |  |  | Mirkundi | 99 | 20.90 | 8.46 |
|  |  |  |  |  | Mirkundi Char | 45 | 10.27 | 4.16 |
|  |  |  |  |  | Mithadi | 3 | 1.63 | 0.66 |
|  |  |  |  |  | Mithapur | 14 | 5.73 | 2.32 |
|  |  |  |  |  | Mohanpur | 94 | 16.30 | 6.60 |
|  |  |  |  |  | Mugar Char | 130 | 24.48 | 9.91 |
|  |  |  |  |  | Mukfaldi | 97 | 29.19 | 11.82 |
|  |  |  |  |  | Mukteshpur | 7 | 1.46 | 0.59 |
|  |  |  |  |  | Musapur | 130 | 32.21 | 13.04 |
|  |  |  |  |  | Nabiganj | 44 | 12.20 | 4.94 |
|  |  |  |  |  | Nabinagar | 74 | 11.26 | 4.55 |
|  |  |  |  |  | Nabuchhra | 8 | 1.59 | 0.64 |
|  |  |  |  |  | Nangalban | 391 | 63.29 | 25.61 |
|  |  |  |  |  | Narayandiya | 3 | 0.84 | 0.34 |
|  |  |  |  |  | Nayachargobi ndopur | 229 | 39.70 | 16.08 |
|  |  |  |  |  | Nisang | 112 | 36.53 | 14.78 |
|  |  |  |  |  | Nurpaddi | 63 | 18.13 | 7.33 |
|  |  |  |  |  | Pachcani | 70 | 14.39 | 5.82 |
|  |  |  |  |  | Padmalabhdi | 9 | 3.19 | 1.29 |
|  |  |  |  |  | Panchamighat | 233 | 79.39 | 32.13 |
|  |  |  |  |  | Panchani | 84 | 12.87 | 5.21 |
|  |  |  |  |  | Pirijpur | 298 | 57.77 | 23.38 |
|  |  |  |  |  | Raahdi | 27 | 6.85 | 2.77 |
|  |  |  |  |  | Rahamatpur | 52 | 11.46 | 4.64 |
|  |  |  |  |  | Ramgabindag aon | 61 | 13.72 | 5.56 |
|  |  |  |  |  | Ratandi | 30 | 6.41 | 2.59 |
|  |  |  |  |  | Sabdi | 103 | 20.71 | 8.38 |
|  |  |  |  |  | Sachinapur | 16 | 4.64 | 1.88 |
|  |  |  |  |  | Salsardi | 133 | 52.65 | 21.30 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Rural Settlement Zone |  |  |  | Sambhupura | 133 | 31.05 | 12.57 |
|  |  |  |  |  | Sarail | 22 | 11.79 | 4.77 |
|  |  |  |  |  | Satanga | 28 | 8.95 | 3.62 |
|  |  |  |  |  | Shatarajdhi | 1 | 0.24 | 0.10 |
|  |  |  |  |  | Sonakhali | 75 | 18.28 | 7.40 |
|  |  |  |  |  | Srinibasdi | 1 | 0.18 | 0.07 |
|  |  |  |  |  | Srirampur | 88 | 20.29 | 8.22 |
|  |  |  |  |  | Subahakardi | 161 | 76.48 | 30.94 |
|  |  |  |  |  | Sukardi | 50 | 14.22 | 5.76 |
|  |  |  |  |  | Tahirpur Kazirgaon | 164 | 21.27 | 8.61 |
|  |  |  |  |  | Tamodardi | 45 | 7.81 | 3.16 |
|  |  |  |  |  | Tartaria | 102 | 26.73 | 10.82 |
|  |  |  |  |  | Tatuakandi | 81 | 18.18 | 7.35 |
|  |  |  |  |  | Tingaon | 83 | 27.73 | 11.22 |
|  |  |  |  |  | Tipurdi | 228 | 58.27 | 23.57 |
|  |  |  |  |  | Ulukandi | 29 | 6.08 | 2.46 |
|  |  |  |  |  | Uttamdi | 18 | 4.59 | 1.86 |
|  |  |  |  |  | Uttar Chanpur | 193 | 39.34 | 15.92 |
|  |  |  |  |  | Uttar Kulcharitra | 87 | 23.50 | 9.51 |
|  |  |  |  |  | Uttar Sabdi | 29 | 9.70 | 3.92 |
| 6 | Transport & Communication | 153.44 | 62.10 | 0.85 | Alapdi | 2 | 0.75 | 0.30 |
|  |  |  |  |  | Amoir | 3 | 1.55 | 0.63 |
|  |  |  |  |  | Asrafdi | 2 | 0.51 | 0.21 |
|  |  |  |  |  | Babrakpur | 4 | 0.53 | 0.22 |
|  |  |  |  |  | Bagbari | 1 | 0.05 | 0.02 |
|  |  |  |  |  | Baler Chak | 1 | 0.13 | 0.05 |
|  |  |  |  |  | Balia | 8 | 0.72 | 0.29 |
|  |  |  |  |  | Bandar | 16 | 4.02 | 1.63 |
|  |  |  |  |  | Bandara | 2 | 0.21 | 0.09 |
|  |  |  |  |  | Bara Bibi Khalisa | 1 | 0.19 | 0.08 |
|  |  |  |  |  | Baranagar | 12 | 0.89 | 0.36 |
|  |  |  |  |  | Barapara | 30 | 7.66 | 3.10 |
|  |  |  |  |  | Barimajlis | 4 | 0.43 | 0.17 |
|  |  |  |  |  | Barundi | 1 | 0.77 | 0.31 |
|  |  |  |  |  | Bejergaon | 5 | 1.06 | 0.43 |
|  |  |  |  |  | Bhairabdi | 15 | 2.21 | 0.90 |
|  |  |  |  |  | Bhajandi | 3 | 0.54 | 0.22 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Transport & Communication |  |  |  | Bhatipara | 5 | 0.93 | 0.37 |
|  |  |  |  |  | Bibijhora | 7 | 1.55 | 0.63 |
|  |  |  |  |  | Binnipara | 1 | 0.64 | 0.26 |
|  |  |  |  |  | Birahmerkandi | 1 | 0.29 | 0.12 |
|  |  |  |  |  | Chairdona | 1 | 0.68 | 0.28 |
|  |  |  |  |  | Chak Karamulla | 4 | 0.15 | 0.06 |
|  |  |  |  |  | Chanpatali | 10 | 2.73 | 1.10 |
|  |  |  |  |  | Char Arjundi | 2 | 0.65 | 0.26 |
|  |  |  |  |  | Char Elahinagar | 3 | 2.14 | 0.87 |
|  |  |  |  |  | Char Safikhan | 11 | 0.78 | 0.32 |
|  |  |  |  |  | Chargoaldi | 4 | 0.72 | 0.29 |
|  |  |  |  |  | Chaudhurigaon | 6 | 1.76 | 0.71 |
|  |  |  |  |  | Chelar Char | 2 | 0.55 | 0.22 |
|  |  |  |  |  | Chhota Bandara | 1 | 0.03 | 0.01 |
|  |  |  |  |  | Chhota Baraikhali | 1 | 0.17 | 0.07 |
|  |  |  |  |  | Chhota Majher Char | 3 | 0.35 | 0.14 |
|  |  |  |  |  | Chhota Silmandi | 4 | 1.04 | 0.42 |
|  |  |  |  |  | Chilarbag | 1 | 0.59 | 0.24 |
|  |  |  |  |  | Chinardi | 1 | 0.07 | 0.03 |
|  |  |  |  |  | Dakshin Kulcharitra | 16 | 2.52 | 1.02 |
|  |  |  |  |  | Darugola | 6 | 0.84 | 0.34 |
|  |  |  |  |  | Dasergaon | 6 | 0.86 | 0.35 |
|  |  |  |  |  | Dashdona | 2 | 0.89 | 0.36 |
|  |  |  |  |  | Deuli | 1 | 0.11 | 0.05 |
|  |  |  |  |  | Dhamgar | 10 | 4.37 | 1.77 |
|  |  |  |  |  | Dighaldi | 5 | 1.35 | 0.55 |
|  |  |  |  |  | Dudhghata | 9 | 3.79 | 1.53 |
|  |  |  |  |  | Durgaprasad Kandi | 5 | 1.92 | 0.78 |
|  |  |  |  |  | Ekrampur | 2 | 0.69 | 0.28 |
|  |  |  |  |  | Fular | 5 | 0.11 | 0.04 |
|  |  |  |  |  | Gazariapar | 1 | 0.40 | 0.16 |
|  |  |  |  |  | Gharmora | 6 | 0.46 | 0.18 |
|  |  |  |  |  | Ghosher Chak | 1 | 0.02 | 0.01 |
|  |  |  |  |  | Goaldi | 3 | 1.18 | 0.48 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Transport & Communication |  |  |  | Gobindakul | 11 | 1.51 | 0.61 |
|  |  |  |  |  | Gonpara | 2 | 0.10 | 0.04 |
|  |  |  |  |  | Gudaraghat | 3 | 0.50 | 0.20 |
|  |  |  |  |  | Habibpur | 3 | 1.28 | 0.52 |
|  |  |  |  |  | Hardi | 1 | 0.26 | 0.11 |
|  |  |  |  |  | Hazipur | 1 | 0.10 | 0.04 |
|  |  |  |  |  | Hazradi | 1 | 0.17 | 0.07 |
|  |  |  |  |  | Hosenpur | 3 | 0.89 | 0.36 |
|  |  |  |  |  | Hosenpur Chak | 2 | 1.74 | 0.71 |
|  |  |  |  |  | Itkhola | 6 | 1.04 | 0.42 |
|  |  |  |  |  | Jahgrakhola | 1 | 0.13 | 0.05 |
|  |  |  |  |  | Jhau Char | 2 | 0.71 | 0.29 |
|  |  |  |  |  | Jiudhara | 5 | 0.53 | 0.21 |
|  |  |  |  |  | Kabilganj | 3 | 0.49 | 0.20 |
|  |  |  |  |  | Kafurdi | 10 | 1.16 | 0.47 |
|  |  |  |  |  | Kajirgaon | 8 | 1.70 | 0.69 |
|  |  |  |  |  | Kaliganj | 1 | 1.19 | 0.48 |
|  |  |  |  |  | Kalyandi | 15 | 1.89 | 0.77 |
|  |  |  |  |  | Kamargaon | 6 | 0.81 | 0.33 |
|  |  |  |  |  | Kamtal | 4 | 0.75 | 0.30 |
|  |  |  |  |  | Keudhala | 24 | 7.56 | 3.06 |
|  |  |  |  |  | Kuchiamora | 5 | 1.73 | 0.70 |
|  |  |  |  |  | Kulcharitra | 3 | 0.58 | 0.23 |
|  |  |  |  |  | Kuripara | 1 | 0.00 | 0.00 |
|  |  |  |  |  | Kusasan | 3 | 1.14 | 0.46 |
|  |  |  |  |  | Kusiara | 21 | 3.22 | 1.31 |
|  |  |  |  |  | Lakshman Khola | 4 | 0.66 | 0.27 |
|  |  |  |  |  | Lal Alapdi | 1 | 0.22 | 0.09 |
|  |  |  |  |  | Lal Krishnadi | 1 | 0.09 | 0.04 |
|  |  |  |  |  | Lam Chariton | 5 | 1.61 | 0.65 |
|  |  |  |  |  | Lambadardi | 7 | 1.71 | 0.69 |
|  |  |  |  |  | Madanpur | 8 | 0.89 | 0.36 |
|  |  |  |  |  | Madhabpasa | 9 | 1.59 | 0.64 |
|  |  |  |  |  | Madhabpur | 8 | 0.55 | 0.22 |
|  |  |  |  |  | Majguni | 2 | 0.50 | 0.20 |
|  |  |  |  |  | Majidbari | 2 | 0.87 | 0.35 |
|  |  |  |  |  | Malikerpara | 2 | 0.19 | 0.08 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Transport & Communication |  |  |  | Mangolergaon | 6 | 1.71 | 0.69 |
|  |  |  |  |  | Menikhali | 1 | 0.38 | 0.16 |
|  |  |  |  |  | Mirbaharkandi | 2 | 0.24 | 0.10 |
|  |  |  |  |  | Mirer Char | 1 | 0.30 | 0.12 |
|  |  |  |  |  | Mirkundi | 1 | 0.75 | 0.30 |
|  |  |  |  |  | Mirkundi Char | 1 | 0.31 | 0.13 |
|  |  |  |  |  | Mithadi | 1 | 0.36 | 0.15 |
|  |  |  |  |  | Mithapur | 1 | 0.17 | 0.07 |
|  |  |  |  |  | Mohanpur | 3 | 1.21 | 0.49 |
|  |  |  |  |  | Mugar Char | 8 | 2.86 | 1.16 |
|  |  |  |  |  | Mukfaldi | 7 | 1.12 | 0.45 |
|  |  |  |  |  | Musapur | 13 | 3.34 | 1.35 |
|  |  |  |  |  | Nabiganj | 1 | 0.00 | 0.00 |
|  |  |  |  |  | Nabinagar | 6 | 1.74 | 0.70 |
|  |  |  |  |  | Nabuchhra | 2 | 0.60 | 0.24 |
|  |  |  |  |  | Nangalban | 14 | 2.52 | 1.02 |
|  |  |  |  |  | Narayandiya | 10 | 0.99 | 0.40 |
|  |  |  |  |  | Nayachargobi ndopur | 2 | 1.49 | 0.60 |
|  |  |  |  |  | Nisang | 9 | 1.41 | 0.57 |
|  |  |  |  |  | Nondirgaon | 1 | 0.00 | 0.00 |
|  |  |  |  |  | Nurpaddi | 3 | 0.51 | 0.21 |
|  |  |  |  |  | Pachcani | 2 | 0.88 | 0.36 |
|  |  |  |  |  | Padmalabhdi | 1 | 0.08 | 0.03 |
|  |  |  |  |  | Panchamighat | 20 | 5.49 | 2.23 |
|  |  |  |  |  | Panchani | 2 | 0.69 | 0.28 |
|  |  |  |  |  | Pirijpur | 12 | 4.97 | 2.01 |
|  |  |  |  |  | Raahdi | 3 | 0.25 | 0.10 |
|  |  |  |  |  | Rahamatpur | 3 | 0.63 | 0.26 |
|  |  |  |  |  | Ramgabindag aon | 2 | 0.55 | 0.22 |
|  |  |  |  |  | Ratandi | 1 | 0.34 | 0.14 |
|  |  |  |  |  | Sabdi | 5 | 1.95 | 0.79 |
|  |  |  |  |  | Salsardi | 1 | 0.34 | 0.14 |
|  |  |  |  |  | Sambhupura | 6 | 4.20 | 1.70 |
|  |  |  |  |  | Satanga | 1 | 0.96 | 0.39 |
|  |  |  |  |  | Silmandi | 1 | 0.23 | 0.09 |
|  |  |  |  |  | Sonakhali | 1 | 0.03 | 0.01 |
|  |  |  |  |  | Srirampur | 5 | 1.39 | 0.56 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Transport & Communication |  |  |  | Subahakardi | 4 | 0.90 | 0.36 |
|  |  |  |  |  | Sukardi | 2 | 0.33 | 0.14 |
|  |  |  |  |  | Tahirpur Kazirgaon | 7 | 2.22 | 0.90 |
|  |  |  |  |  | Tamodardi | 6 | 0.84 | 0.34 |
|  |  |  |  |  | Tatuakandi | 6 | 1.40 | 0.57 |
|  |  |  |  |  | Terakahania | 2 | 1.27 | 0.51 |
|  |  |  |  |  | Tingaon | 4 | 0.24 | 0.10 |
|  |  |  |  |  | Tipurdi | 3 | 0.70 | 0.28 |
|  |  |  |  |  | Ulukandi | 5 | 0.81 | 0.33 |
|  |  |  |  |  | Uttamdi | 3 | 0.31 | 0.13 |
|  |  |  |  |  | Uttar Chandibardi | 1 | 0.04 | 0.02 |
|  |  |  |  |  | Uttar Chanpur | 2 | 0.04 | 0.02 |
|  |  |  |  |  | Uttar Kulcharitra | 11 | 1.20 | 0.48 |
|  |  |  |  |  | Uttar Sabdi | 3 | 0.60 | 0.24 |
| 7 | Waterbody | 1308.67 | 529.62 | 7.22 | Alapdi | 3 | 2.34 | 0.95 |
|  |  |  |  |  | Amoir | 11 | 2.95 | 1.19 |
|  |  |  |  |  | Bag Nazirpur | 1 | 0.12 | 0.05 |
|  |  |  |  |  | Baligaon | 8 | 2.60 | 1.05 |
|  |  |  |  |  | Bandar | 42 | 10.00 | 4.05 |
|  |  |  |  |  | Bandara | 6 | 3.62 | 1.46 |
|  |  |  |  |  | Bara Baraikhali | 3 | 2.39 | 0.97 |
|  |  |  |  |  | Bara Bibi Khalisa | 3 | 1.81 | 0.73 |
|  |  |  |  |  | Bara Sadipur | 13 | 2.74 | 1.11 |
|  |  |  |  |  | Baranagar | 35 | 8.06 | 3.26 |
|  |  |  |  |  | Barapara | 29 | 9.04 | 3.66 |
|  |  |  |  |  | Barimajlis | 10 | 3.78 | 1.53 |
|  |  |  |  |  | Barundi | 2 | 0.90 | 0.36 |
|  |  |  |  |  | Bejergaon | 1 | 0.03 | 0.01 |
|  |  |  |  |  | Bhagalpur | 2 | 0.38 | 0.15 |
|  |  |  |  |  | Bhairabdi | 4 | 2.96 | 1.20 |
|  |  |  |  |  | Bhajandi | 3 | 1.86 | 0.75 |
|  |  |  |  |  | Bhatipara | 10 | 3.19 | 1.29 |
|  |  |  |  |  | Bhattacharjya Chak | 50 | 26.26 | 10.63 |
|  |  |  |  |  | Bibijhora | 11 | 1.95 | 0.79 |
|  |  |  |  |  | Bil Boroya | 4 | 2.24 | 0.91 |
|  |  |  |  |  | Binnipara | 1 | 0.22 | 0.09 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Waterbody |  |  |  | Birahmerkandi | 4 | 1.34 | 0.54 |
|  |  |  |  |  | Chairdona | 4 | 41.61 | 16.84 |
|  |  |  |  |  | Chak Baraia | 5 | 2.65 | 1.07 |
|  |  |  |  |  | Chanpatali | 6 | 1.05 | 0.43 |
|  |  |  |  |  | Char Arjundi | 5 | 4.27 | 1.73 |
|  |  |  |  |  | Char Elahinagar | 9 | 55.49 | 22.45 |
|  |  |  |  |  | Char Fatepur | 2 | 1.06 | 0.43 |
|  |  |  |  |  | Char Safikhan | 7 | 1.61 | 0.65 |
|  |  |  |  |  | Char Ulukandi | 102 | 26.78 | 10.83 |
|  |  |  |  |  | Chargoaldi | 12 | 9.92 | 4.01 |
|  |  |  |  |  | Chaudhurigaon | 1 | 0.52 | 0.21 |
|  |  |  |  |  | Chelar Char | 2 | 0.32 | 0.13 |
|  |  |  |  |  | Chhota Baraikhali | 2 | 1.50 | 0.61 |
|  |  |  |  |  | Chhota Majher Char | 4 | 0.29 | 0.12 |
|  |  |  |  |  | Chhota Sadipur | 15 | 2.95 | 1.19 |
|  |  |  |  |  | Chilarbag | 23 | 7.57 | 3.06 |
|  |  |  |  |  | Chinardi | 6 | 3.14 | 1.27 |
|  |  |  |  |  | Dakshin Chandpur | 8 | 1.12 | 0.45 |
|  |  |  |  |  | Dakshin Char | 1 | 3.83 | 1.55 |
|  |  |  |  |  | Dakshin Kulcharitra | 2 | 3.31 | 1.34 |
|  |  |  |  |  | Darugola | 2 | 0.73 | 0.30 |
|  |  |  |  |  | Dashdona | 121 | 60.86 | 24.63 |
|  |  |  |  |  | Decree Char | 3 | 0.69 | 0.28 |
|  |  |  |  |  | Dhamgar | 8 | 4.60 | 1.86 |
|  |  |  |  |  | Dighaldi | 3 | 1.67 | 0.68 |
|  |  |  |  |  | Dudhghata | 85 | 20.21 | 8.18 |
|  |  |  |  |  | Ekrampur | 7 | 44.61 | 18.05 |
|  |  |  |  |  | Fardi | 1 | 0.61 | 0.25 |
|  |  |  |  |  | Fular | 5 | 1.04 | 0.42 |
|  |  |  |  |  | Gazariapar | 196 | 62.11 | 25.14 |
|  |  |  |  |  | Gharmora | 1 | 4.83 | 1.96 |
|  |  |  |  |  | Ghorir Chak | 4 | 1.45 | 0.59 |
|  |  |  |  |  | Giribardi | 1 | 0.47 | 0.19 |
|  |  |  |  |  | Goaldi | 14 | 11.24 | 4.55 |
|  |  |  |  |  | Gobindakul | 2 | 1.45 | 0.59 |
|  |  |  |  |  | Gobindamandali | 1 | 0.00 | 0.00 |
|  |  |  |  |  | Gonpara | 2 | 0.65 | 0.26 |
|  |  |  |  |  | Habibpur | 30 | 15.47 | 6.26 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Waterbody |  |  |  | Hardi | 16 | 1.00 | 0.41 |
|  |  |  |  |  | Hosenpur | 2 | 0.66 | 0.27 |
|  |  |  |  |  | Imamdir Chhat | 3 | 1.05 | 0.42 |
|  |  |  |  |  | Itkhola | 6 | 3.72 | 1.50 |
|  |  |  |  |  | Jagatdi | 3 | 0.33 | 0.13 |
|  |  |  |  |  | Jangal | 16 | 17.87 | 7.23 |
|  |  |  |  |  | Jhau Char | 5 | 23.25 | 9.41 |
|  |  |  |  |  | Jiudhara | 7 | 2.07 | 0.84 |
|  |  |  |  |  | Kabilganj | 15 | 3.22 | 1.30 |
|  |  |  |  |  | Kakair Tek | 21 | 9.04 | 3.66 |
|  |  |  |  |  | Kaliganj | 1 | 0.47 | 0.19 |
|  |  |  |  |  | Kalyandi | 11 | 2.97 | 1.20 |
|  |  |  |  |  | Kamargaon | 1 | 0.49 | 0.20 |
|  |  |  |  |  | Kamtal | 16 | 3.72 | 1.50 |
|  |  |  |  |  | Keudhala | 51 | 30.02 | 12.15 |
|  |  |  |  |  | Kuchiamora | 7 | 5.17 | 2.09 |
|  |  |  |  |  | Kulcharitra | 7 | 3.15 | 1.28 |
|  |  |  |  |  | Kuripara | 8 | 2.97 | 1.20 |
|  |  |  |  |  | Kusasan | 3 | 2.69 | 1.09 |
|  |  |  |  |  | Kusiara | 14 | 9.25 | 3.74 |
|  |  |  |  |  | Lakshman Khola | 6 | 0.82 | 0.33 |
|  |  |  |  |  | Lal Krishnadi | 53 | 13.31 | 5.39 |
|  |  |  |  |  | Lam Chariton | 2 | 3.20 | 1.30 |
|  |  |  |  |  | Lambadardi | 11 | 0.92 | 0.37 |
|  |  |  |  |  | Madanpur | 6 | 2.09 | 0.84 |
|  |  |  |  |  | Madhabpasa | 7 | 2.95 | 1.19 |
|  |  |  |  |  | Madhabpur | 2 | 0.69 | 0.28 |
|  |  |  |  |  | Maghbazar | 2 | 2.00 | 0.81 |
|  |  |  |  |  | Majidbari | 4 | 1.36 | 0.55 |
|  |  |  |  |  | Malikerpara | 2 | 0.65 | 0.27 |
|  |  |  |  |  | Manahar Khaner Bag | 8 | 10.31 | 4.17 |
|  |  |  |  |  | Mangolergaon | 1 | 0.72 | 0.29 |
|  |  |  |  |  | Menikhali | 148 | 33.81 | 13.68 |
|  |  |  |  |  | Mirer Char | 165 | 91.19 | 36.90 |
|  |  |  |  |  | Mirkundi | 7 | 2.60 | 1.05 |
|  |  |  |  |  | Mirkundi Char | 213 | 52.31 | 21.17 |
|  |  |  |  |  | Mithadi | 4 | 0.43 | 0.17 |
|  |  |  |  |  | Mithapur | 9 | 1.10 | 0.45 |
|  |  |  |  |  | Mohanpur | 42 | 15.38 | 6.22 |
|  |  |  |  |  | Mohiskhola | 38 | 19.64 | 7.95 |
|  |  |  |  |  | Mugar Char | 74 | 27.41 | 11.09 |
|  |  |  |  |  | Mukfaldi | 4 | 0.82 | 0.33 |

| Sl. No. | Landuse | Area |  |  | Name of the Mauza with Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Acre | Hectare | % | Mauza Name | Number of Plot | Area (Acre) | Area (Hectare) |
|  | Waterbody |  |  |  | Mukteshpur | 7 | 1.48 | 0.60 |
|  |  |  |  |  | Musapur | 2 | 0.32 | 0.13 |
|  |  |  |  |  | Nabinagar | 2 | 0.90 | 0.36 |
|  |  |  |  |  | Nabuchhra | 2 | 0.33 | 0.13 |
|  |  |  |  |  | Nangalban | 16 | 5.09 | 2.06 |
|  |  |  |  |  | Narayandiya | 8 | 1.14 | 0.46 |
|  |  |  |  |  | Nayachargobi ndopur | 27 | 23.13 | 9.36 |
|  |  |  |  |  | Nisang | 18 | 11.29 | 4.57 |
|  |  |  |  |  | Nurpaddi | 23 | 2.55 | 1.04 |
|  |  |  |  |  | Pachcani | 4 | 1.79 | 0.72 |
|  |  |  |  |  | Padmalabhdi | 2 | 0.44 | 0.18 |
|  |  |  |  |  | Panchamighat | 16 | 66.57 | 26.94 |
|  |  |  |  |  | Panchani | 3 | 2.87 | 1.16 |
|  |  |  |  |  | Pirijpur | 14 | 17.86 | 7.23 |
|  |  |  |  |  | Rahamatpur | 1 | 0.34 | 0.14 |
|  |  |  |  |  | Ramgabindag aon | 3 | 0.82 | 0.33 |
|  |  |  |  |  | Ratandi | 1 | 1.25 | 0.51 |
|  |  |  |  |  | River | 3 | 167.49 | 67.78 |
|  |  |  |  |  | Sabdi | 7 | 3.32 | 1.35 |
|  |  |  |  |  | Sachinapur | 5 | 0.97 | 0.39 |
|  |  |  |  |  | Salsardi | 8 | 2.02 | 0.82 |
|  |  |  |  |  | Sambhupura | 21 | 34.47 | 13.95 |
|  |  |  |  |  | Sarail | 1 | 1.24 | 0.50 |
|  |  |  |  |  | Serpur | 2 | 2.88 | 1.16 |
|  |  |  |  |  | Shatarajdhi | 3 | 0.51 | 0.21 |
|  |  |  |  |  | Silmandi | 4 | 0.06 | 0.02 |
|  |  |  |  |  | Srirampur | 99 | 31.11 | 12.59 |
|  |  |  |  |  | Subahakardi | 5 | 17.34 | 7.02 |
|  |  |  |  |  | Sukardi | 3 | 1.59 | 0.64 |
|  |  |  |  |  | Tahirpur Kazirgaon | 3 | 0.70 | 0.28 |
|  |  |  |  |  | Tartaria | 9 | 1.42 | 0.57 |
|  |  |  |  |  | Tindona | 1 | 0.15 | 0.06 |
|  |  |  |  |  | Tingaon | 2 | 0.28 | 0.11 |
|  |  |  |  |  | Tipurdi | 14 | 4.06 | 1.64 |
|  |  |  |  |  | Uttar Chanpur | 5 | 1.70 | 0.69 |
| Total |  | 18115.50 | 7331.06 | 100 |  | 67159 | 18115.50 | 7331.06 |

The proposed road network of the DPZ 07 is also presented in the following Table 3.19.

##### Table - 3.19: Proposed New Roads of DPZ 07

| Road Name | Width (m.) | Length (Km.) |
| --- | --- | --- |
| Amoir - Gharmora Road | 30 | 10.11 |

| Road Name | Width (m.) | Length (Km.) |
| --- | --- | --- |
| Deuli - Barapara Road | 30 | 3.39 |
| Dhaka-Chittagong Highway | 85 | 9.08 |
| Dudhghata Road | 27 | 1.47 |
| Goladi - Tipordi Road | 30 | 1.37 |
| Kuripara - Malibag Road | 30 | 2.11 |
| Lakshmankhola - Langalband Bazar Road | 30 | 3.58 |
| Lal Krishnadi - Gaziarpar Road | 27 | 2.21 |
| Langalband - Kalagachia Bazar - Farajikanda Road | 30 | 11.77 |
| Madanganj - Kabilganj Road | 27 | 8.21 |
| Madanpur - Madanganj Road | 85 | 1.69 |
| Nabiganj - Kaikertek - Sonargaon Road | 30 | 7.25 |
| Sonakanda - Mirkandi Road | 30 | 3.43 |

Map No. 3.8 shows the area of DPZ 07 as well as proposed roads and landuses.

The Landuses and Road Network as proposed for the Group-B area is shown in the Integrated Planning Map No. 3.9.